3GPP TSG-RAN4 Meeting #88BIS 	R4-1812483

# Chengdu, China, 8 – 12 Oct., 2018

Title:	Reply LS on UE behavior on reception of channels or RS in the same OFDM symbol

Response to:	R1-1809890

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	Jin-Yup Hwang

E-mail Address:	jinyup.hwang@lge.com

1. Overall Description:

RAN4 has discussed RAN1 LS on UE behavior on reception of channels or RS in the same OFDM symbol. RAN4 has not been consensus to define core requirements, so CSI-RS based L3 measurement is precluded in this LS.

RAN4 would like to inform RAN1 the conclusion in RAN4 as follows:

- Answer to Q1

Case 1

  - If the channels/RS in group 1 is QCL-Type D with channels/RS in group 2 and the QCL association is known to UE, UE can receive both channels/RS in group 1 and group 2 in the same OFDM symbol in FR2.

    - However, if L3 measurement resource for neighbor cells overlaps with other channels/RS for serving cells on the same OFDM in SMTC window duration, UE is not expected to receive channels/RS for serving cells on symbols to be measured for L3 measurement.

Case 2 and Case 3

  - In RAN4, introduction of CSI-RS based L3 measurement requirements is FFS in Rel-15. With the exception of this cases, if the TCI of the PDSCH includes at least one of the CSI-RS resource IDs of the CSI-RS resource set, UE may be allocated and receive a PDSCH that is overlapping with one or more symbols configured with CSI-RS resource(s) with repetition set to “OFF”.

Case 4

  - Similar to case 1, UE can receive QCLed RSs transmitted in the same OFDM symbol assuming that the SCS of QCLed RSs is the same.

- Answer to Q2

  - For rate matching issue by scheduling restriction, RAN4 has already agreed in RAN4 NR-AH#4 meeting as follows:

    - Whether or not to define specific behavior for rate matching around scheduling restricted symbols is up to RAN1

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above RAN4 conclusion into consideration for their further work.

3. Date of Next TSG-RAN4 Meetings:

3GPP RAN4 #89 		12 - 16 Nov 2018, 		Spokane, US

3GPP RAN4 #90 		2 Feb - 1 March 2019, 		Athens, Greece