3GPP TSG-RAN WG4 Meeting #88bis	R4-1812489

Chengdu, China, 8 - 12 Oct, 2018

Agenda item:	9.2.2

Source:	Dish Network

Title:	Re-evaluation of NR CA H3 MSD assumptions

Document for:	Discussion

# 1	Introduction

In previous meeting it was agreed to re-evaluate NR CA H3 MSD assumptions [1]. This paper discusses assumptions used in LTE CA and in EN-DC CA in the case where a low band UL 3rd harmonic hits the high band DL. A brief sensitivity analysis is provided to help assessing which changes in assumptions result in smaller MSD and/or lower low band TX loss.

The first pure NR CA combination with a 3rd harmonic relation between lower band UL and higher band DL is CA_n70A-n71A.

# 2	Discussion

The first LTE CA combination that had H3 relation with LB UL and HB DL was CA_4A-17A. The assumptions for that combination are shown in [2] which then became the baseline assumption for all other L-H combinations (at that time H was >1700MHz) even there was no formal agreement to use [2] as a baseline.

Baseline RF architecture and numerical analysis from [2] is shown below for convenience.

The analysis is summarized below assuming high linearity component selection and the implementation of a diplexer at the antenna port.

|  | Primary |  | Diversity |  |
| --- | --- | --- | --- | --- |
| Parameter | Value | H3 level | Value | H3 level |
| B17 Tx | 27.5 |  | 27.5 |  |
| B17 PA H3 | -50 | -22.5 | -50 | -22.5 |
| B17 duplexer | 40 | -62.5 | 40 | -62.5 |
| Harmonic filter | 30.5 | -93.0 | 30.5 | -93.0 |
| LB switch | -96.5 | -91.4 | -96.5 | -91.4 |
| Diplexer | 15 | -106.4 | 15 | -106.4 |
| Antenna isolation |  |  | 10 | -116.4 |
| HB switch attenuation | 0.7 | -107.1 | 0.7 | -117.1 |
| HB switch H3 | -126 | -107.0 | -111.9 | -110.8 |
| B4 duplexer attenuation | 1.6 | -108.6 | 1.6 | -112.4 |
| B4 duplexer H3 | -126 | -108.6 | -111.9 | -109.1 |
| B17 PA to B4 LNA isolation | 80 | -102.5 | 80 | -102.5 |
| Single chip DA to LNA | -100 | -100.0 | -100 | -100.0 |
| Composite |  | -97.7 |  | -97.7 |

The composite additional noise in the table above due to LB UL H3 is defined at HB LNA inputs.

The outcome of the re-evaluation of the assumptions may basically impact three specifications in “H3” CA combination; Additional LB TX loss (TIB), HB MSD, and any combination of these two.

Currently, additional TIB on top of normal L-H combination is 0.3dB to account for H3 trap filter. MSD naturally varies depending on the REFSENS of the band, but as an example the MSD for band whose REFSENS for 5MHz is -100dBm, the MSD is 10dB in LTE specification.

Additional TIB is a sort of a “shared pain” compromise of the assumption that the additional loss for the trap filter is 0.5dB or so. The MSD in LTE specifications then comes pretty much from the analysis above.

Basically, there are three main paths for the H3 noise leaking into HB LNA input in [1]: 1) Coupling via conductive path, 2) PCB coupling from the PA output to LNA input, and 3) PCB/Pin-2-pin coupling from LB RFIC TX output to HB LNA input. They are marked Yellow, Green, and Red, respectively.

One of the biggest architectural changes in the cellular RF industry has probably been LNAs becoming integrated into filter/switch modules instead of being inside RFIC chips. This, among other things might leave room to imrove the assumption of -100 dBm absolute H3 noise level coupled from RFIC TX output to victim LNA input.

Another thing is PAs are nodaways inside PA modules, and that together with LNAs being inside filter/switch modules might leave room to improve the assumption of 80dB isolation between LB PA output and victim LNA input and/or the PA H3 level at the PA output.

Thirdly, even though the conducted path does not play a big role in current analysis, the linearity of the component could be re-evaluated as well, because during 2012 there had been no obvious need to develop highly linear FE components. Now the demand has been there for years so likely the linearity has improved.

We did a brief sensitivity analysis for 5MHz CC BW where the single CC REFSENS is -100dBm by altering the assumptions for the mentioned three coupling paths. The purpose of the sensitivity analysis is to see how changes in each parameters, or combination of them, impact the MSD. The numbers in the table mean improvement in MSD, in decibels. Please note that the improvements in each path, 5, 10, and 15dB are examples. We also note that there also might be other coupling effects to account in current RF designs than the ones used in [1].

The first table assumes only one of the paths is improved at a time.

Table 1 MSD Sensitivity analysis with one variable

| Improvement in coupling path [dB] | Yellow, MSD reduction [dB] | Green, MSD reduction [dB] | Red, MSD reduction [dB] |
| --- | --- | --- | --- |
| 5 | 0.1 | 1.0 | 2.0 |
| 10 | 0.1 | 1.3 | 2.8 |
| 15 | 0.1 | 1.5 | 3.1 |

It can be seen from the Table 1 that the MSD can be improved between 0.1….3.1dB depending on which assumption of of three coupling paths is improved by up to 15dB. For instance, if Red path (RFIC TX output->LNA input PCB/Pin-2-pin coupling) assumption is improved by 10dB, from -100dBm to -110dBm at HB LNA input, the MSD for 5MHz BW is improved by 2.8dB compared with the current LTE specification.

In table below we evaluate the impact of improving both Green and Red paths together. Possible improvements is Yellow path are ignored, because the impacts to  MSD provided by that are small.

Table 2 MSD Sensitivity analysis with two variables

| Improvement in coupling path [dB] |  | Green, MSD reduction |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 |
| Red, MSD reduction | 5 | 3.7 | 4.4 | 4.6 |
|  | 10 | 5.0 | 6.0 | 6.4 |
|  | 15 | 5.6 | 6.7 | 7.2 |

It can be seen from the Table 2 that the MSD can be improved by 3.7…7.2dB depending on what kind of combination of improvements for Red and Green paths (RFIC TX output->LNA input PCB/Pin-2-pin coupling, PA Output->LNA input PCB coupling). For instance, if Red path (RFIC TX output->LNA input) assumption is improved by 10dB, from -100dBm to -110dBm at HB LNA input, and the Green path (PA Output->LNA input PCB coupling) is improved by 15dB, from -102.5dBm to -117.5dBm, the MSD for 5MHz BW is improved by 6.4dB compared with the current LTE specification.

Another angle to look at this is if the additional TX loss, due to H3 trap filter after LB duplexer could be discussed. It is quite difficult not to assume trap filter is used (in MSD analysis) because then good MSD performance would require very rigorously defined modern architectures and so on. Alternatively, one suggestion is to continue assuming trap  filter is used, but not to allow additional TX relaxation. In this case, the TIB for LB would be 0.3dB instead of current 0.6dB. The background behind this is that currently most if not all designs provide 23dBm or more at the antenna connector for <1GHz bands even device supports CA.

We welcome companies to discuss if some of the current assumptions can be updated. We recognize that it is likely easier to suggest not to touch the assumptions, but given the RF architectures and especially FEs have evolved a lot since 2012, we really look forward open discussion on this topic.

# 3	Conclusions

Re-evaluation of H3 assumptions was discussed. We have identified that some improvements in assumptions behind H3 MSD requirements would result in significant improvements in MSD specifications.

# 4	References

[1] R4-1810347 “H3 MSD requirements for Inter-band NR CA”, Dish Network

[2] R4-124627, “Band 4+Band 17 MSD”, Qualcomm Incorporated, AT&T, 3GPP  RAN4#64, Qingdao, China, August 2012