3GPP TSG-RAN4 Meeting #88BIS 	R4-1812491

# Chengdu, China, 8 – 12 Oct., 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.810 | CR |  | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Measurement grid points for beam peak search |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-09-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To reduce OTA test tiem, measurement grid points for beam peak search should be revised. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The number of measurement points for 3D EIRP scan is based on 5deg measurement grid and that for beam peak search is performed at additional measurement points based on 2.5deg measurement grid. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 OTA test time cannot be reduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.3.2, 5.2.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---- < Start of Change > ---

5.2.1.3.2	EIRP Measurement Procedure

The TX beam peak direction is found with a 3D EIRP scan (separately for each orthogonal polarization) with additional a few points based on 2.5deg step size measurement grid around maximum measured EIRP point. The grid points for 3D EIRP scan  is 2664 (5deg step size) using constant step approach or 2592 using constant density approach (using the charged particle implementation). The TX beam peak direction is where the maximum total component of EIRP is found. The spherical coverage measurement grid points is the same as that for 3D EIRP scan, i.e. 2664 (5deg step size) for constant step or 2592 for constant density (using the charged particle implementation). The measurement grids is calculated under assumption of 8x2 patch antenna array, MU of absolute TX power beam peak measurement of 0.5dB.

1)	Connect the SS (System Simulator) with the DUT through the measurement antenna with polarization reference PolMeas to form the TX beam towards the previously determined TX beam peak direction and respective polarization.

2)	Lock the beam toward that direction for the entire duration of the test.

3)	Measure the mean power (Pmeas, θ) of the modulated signal arriving at the power measurement equipment (such as a spectrum analyser, power meter, or gNB emulator).

4)	Calculate EIRPθ by adding the composite loss of the entire transmission path for utilized signal path, LEIRP,θ, and frequency to the measured power Pmeas,θ

5)	Measure the mean power (Pmeas,φ) of the modulated signal arriving at the power measurement equipment.

6)	Calculate EIRPφ by adding the composite losses of the entire transmission path for utilized signal path, LEIRP,φ, and frequency to the measured power Pmeas,φ

7)	Calculate total EIRP = EIRPθ + EIRPφ

5.2.1.3.3	TRP Measurement Procedure

The minimum number of measurement points for TRP measurement grid is calculated based on 8x2 patch antenna array, a maximum standard deviation of 0.25dB.

If constant step approach is used for TRP measurement, several measurement grids are permitted:

1.	264 measurement points (15deg step size) with TX beam peak oriented in any direction; an additional mean error  of 0.34dB need to be considered by using classic TRP equation);

2.	612 measurement points (10deg step size) with the TX beam peak oriented in any direction using classic TRP equation; no additional mean error needs to be considered

If constant density approach (using the charged particle or the golden spiral implementation) is used, the minimum number of measurement points is 140; no additional mean error needs to be considered.

1)	Connect the SS with the DUT through the measurement antenna with desired polarization reference PolMeas to form the TX beam towards the desired TX beam direction and respective polarization.

2)	Lock the beam toward that direction and polarization for the entire duration of the test.

3)	For each measurement point, measure Pmeas,θ and Pmeas,φ. The angle between the measurement antenna and the DUT (θMeas, φMeas) is achieved by rotating the measurement antenna and the DUT (based on system architecture).

4)	Calculate EIRPθ (EIRPφ) by adding the composite loss of the entire transmission path for utilized signal paths, LEIRP,θ, (LEIRP,φ) and frequency to the measured powers Pmeas,θ (Pmeas,φ)

5)	The TRP value for the uniform measurement grid is calculated using

TRP=π2NMi=1N-1j=0M-1EIRPθθi,ϕj+EIRPϕθi,ϕjsinθi

Where N is the number of angular intervals in the nominal theta range from 0 to π and M is the number of angular intervals in the nominal phi range from 0 to 2π.

The TRP values for the constant density grids are calculated using:

TRP=1Ni=0N-1EIRPθθi,ϕi+EIRPϕθi,ϕi

where N is the number of measurement points.

5.2.1.3.4	EIS Measurement Procedure

The RX beam peak direction is found with a 3D RSRP scan (separately for each orthogonal polarization) with a same grid points as Tx beam peak direction search. The RX beam peak direction is where the maximum total component of RSRP is found.

1)	Establish a connection between the DUT and the SS with the downlink signal applied to the θ-polarization of the measurement antenna

2)	Position the UE so that the beam is formed towards the measurement antenna in the RX beam peak direction

3)	Determine EISθ for θ-polarization, i.e., the power level for the θ-polarization at which the throughput exceeds the requirements for the specified reference measurement channel

4)	Switch the downlink to the φ-polarization of the measurement antenna

5)	Determine EISφ for φ-polarization, i.e., the power level for the φ-polarization at which the throughput exceeds the requirements for the specified reference measurement channel

6)	Calculate the resulting EIS for the total component

EIS = [1/EISθ +1/EISφ]-1

---- < End of Change > ---