3GPP TSG-RAN WG4 Meeting #88-BIS                                                      	R4-1812512

# Chengdu, China, 8th – 12th Oct, 2018

Agenda item:            7.12.5.13

Source:	MediaTek Inc.

Title:	NR PDCCH performance for RLM

Document for:          Discussion

# 1	Introduction

In SSB-based RLM test cases, SNR settings highly depend on the PDCCH performance of RMSI CORESET. Based on the simulation assumption [1], we provide NR PDCCH simulation results.

# 2	Discussion

Table 1 summarizes the required SNR to achieve 10% and 2% for OOS and INS, respectively.

Table 1: Required SNR (unit: dB)

|  | Case 1 | Case 2 | Case 3 | Case 4 |
| --- | --- | --- | --- | --- |
| 2x4, OOS | -11.4 | -11.6 | -10.8 | -10.7 |
| 2x2, OOS | -7.7 | -9.2 | -8.1 | -8.0 |
| 2x4, INS | -4.5 | -4.5 | -3.9 | -3.9 |
| 2x2, INS | 0.0 | -0.5 | 0.0 | 0.0 |

In Figure 1-4, the PDCCH curves are also provided.

Fig. 1: PDCCH performance for Case 1               Fig. 2: PDCCH performance for Case 2

Fig. 3: PDCCH performance for Case 3                      Fig. 4: PDCCH performance for Case 4

# 3	Conclusions

In the contribution, we provide our simulation result of PDCCH for RLM.

# 4	Reference

[1] R4-180xxxxxx, “Simulation assumption for RLM”, Mediatek

# 5	Appendix

- Assumption of TRS configuration

  - 2 slots pattern

  - Periodicity 20ms

  - Offset 10ms from SSB

  - OFDM symbols with TRS: #4 and #8

  - Bandwidth: full BWP

  - 0 dB boosting.

Table 1: Simulation assumptions for PDCCH performance, out-of-sync

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  | GHz | FR1, 4 GHz | FR1, 4 GHz | FR2, 30 GHz | FR2, 30 GHz |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Physical layer cell identity, |  |  | 0 | 0 | 0 | 0 |
| NR Channel Bandwidth (BWchannel) |  | MHz | 10 | 40 | 100 | 100 |
| Subcarrier Spacing for SSB |  | KHz | 15 | 30 | 120 | 240 |
| Subcarrier Spacing for PDCCH |  | KHz | 15 | 30 | 120 | 120 |
| Antenna Configuration |  |  | 2x2 Low, 2x4 Low | 2x2 Low, 2x4 Low | 2x2 Low (Note 1) | 2x2 Low (Note 1) |
| Out of sync transmission parameters (based on Table 8.1.2.1-1 in TS 38.133) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | DCI size without CRC |  | 39 bits | 41 bits | 40 bits | 40 bits |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Bandwidth of RMSI CORESET | PRB | 24 | 24 | 24 | 24 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | CORESET start PRB |  | 0 | 0 | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size, L |  | 6 | 6 | 6 | 6 |
|  | Mapping from REG to CCE |  | Interleaved | Interleaved | Interleaved | Interleaved |
|  | nshift |  | 0 | 0 | 0 | 0 |
|  | InterleaverSize. R |  | 2 | 2 | 2 | 2 |
| Propagation channel |  |  | TDL-C 300ns 100Hz | TDL-C 300ns 100Hz | TDL-A 30ns 75Hz | TDL-A 30ns 75Hz |
| Outcome metric |  |  | SNR levels corresponding to 10% PDCCH BLER |  |  |  |
| Note 1: The antenna configuration of 2x2 in FR2 is just for simulation purpose on PDCCH performance, not for OTA test cases Note 2: UE receiver assumptions: Real channel estimation, noise estimation and synchronization Note 3: precoder model for 2TX: random for each REG bundle |  |  |  |  |  |  |

Table 2: Simulation assumptions for PDCCH performance, in-sync

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  | GHz | FR1, 4 GHz | FR1, 4 GHz | FR2, 30 GHz | FR2, 30 GHz |
| Physical layer cell identity, |  |  | 0 | 0 | 0 | 0 |
| CP length |  |  | Normal | Normal | Normal | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | 10 | 40 | 100 | 100 |
| Subcarrier Spacing for SSB |  | KHz | 15 | 30 | 120 | 240 |
| Subcarrier Spacing for PDCCH |  | KHz | 15 | 30 | 120 | 120 |
| Antenna Configuration |  |  | 2x2 Low, 2x4 Low | 2x2 Low, 2x4 Low | 2x2 Low (Note 1) | 2x2 Low (Note 1) |
| In-sync transmission parameters (based on Table 8.1.2.1-2 in TS 38.133) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | DCI size without CRC |  | 39 bits | 41 bits | 40 bits | 40 bits |
|  | Aggregation level | CCE | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | Bandwidth of RMSI CORESET | PRB | 24 | 24 | 24 | 24 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | CORESET start PRB |  | 0 | 0 | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size, L |  | 6 | 6 | 6 | 6 |
|  | Mapping from REG to CCE |  | Interleaved | Interleaved | Interleaved | Interleaved |
|  | nshift |  | 0 | 0 | 0 | 0 |
|  | InterleaverSize. R |  | 2 | 2 | 2 | 2 |
| Propagation channel |  |  | TDL-C 300ns 100Hz | TDL-C 300ns 100Hz | TDL-A 30ns 75Hz | TDL-A 30ns 75Hz |
| Outcome metric |  |  | SNR levels corresponding to 2% PDCCH BLER |  |  |  |
| Note 1: The antenna configuration of 2x2 in FR2 is just for simulation purpose on PDCCH performance, not for OTA test cases Note 2: UE receiver assumptions: Real channel estimation, noise estimation and synchronization Note 3: precoder model for 2TX: random for each REG bundle |  |  |  |  |  |  |