3GPP TSG-RAN4 Meeting #88bis	R4-1812533

# Chengdu, China, 8th Oct – 12th Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for Interruption(section 8.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-9-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The SMTC duration in the interruption requirements has ambiguity. - The SMTC duration time of intra-band interruption should consider both PSCell(PCell) and all activated SCells - UE cannot fulfil current UL carrier RRC reconfiguration requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the SMTC duration definition in SCell addition/release and SCell activation/deactivation. - Combine PCell and any activated SCell description together - UL carrier RRC reconfiguration requirement should follow inter-band SCell addition/release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The SMTC duration definition will be incomplete. - The specification will be not correct - UE cannot fulfil current UL carrier RRC reconfiguration requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1>>

##### 8.2.1.2.3	Interruptions at SCell addition/release

The requirements in this clause shall apply for the UE configured with PSCell.

When one E-UTRA SCell is added or released:

-	an interruption on any activated serving cell in SCG:

-	of up to X1 slot, if the activated serving cell is not in the same band as any of the E-UTRA SCells being added or released, or

-	of up to max{Y1 slot + TSMTC_duration, 5ms} if the activated serving cells are in the same band as any of the E-UTRA SCells being added or released, provided the cell specific reference signals from the activated serving cells and the E-UTRA SCells being added or released are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above activated serving cells in SCG;

When one SCell is added or released:

-	an interruption on any activated serving cell in SCG:

-	of up to X1 slot, if the activated serving cell is not in the same band as any of the SCells being added or released, or

-	of up to Y1 slot + TSMTC_duration if the activated serving cells are in the same band as any of the SCells being added or released, provided the cell specific reference signals from the activated serving cells and the SCells being added or released are available in the same slot, where, TSMTC_duration is

- the longest SMTC duration among all above activated serving cells in SCG and the SCell being added when one SCell is added;

- the longest SMTC duration among all above activated serving cells in SCG when one SCell is released.

Table 8.2.1.2.3-1 Interruption length X1 and Y1 at SCell addition/Release

|  | NR Slot length (ms) | Interruption length X1 slot |  | Interruption length Y1 slotNote 1 |
| --- | --- | --- | --- | --- |
|  |  | Sync | Async |  |
| 0 | 1 | 1 | 2 | 1 |
| 1 | 0.5 | 2 | 3 | 2 |
| 2 | 0.25 | 5 |  | 4 |
| 3 | 0.125 | 9 |  | 8 |

##### 8.2.1.2.4	Interruptions at SCell activation/deactivation

The requirements in this clause shall apply for the UE configured with PSCell and one SCell.

When one E-UTRA SCell is activated or deactivated:

-	an interruption on any activated serving cell in SCG:

-	of up to X2 slot, if the activated serving cell is not in the same band as any of the E-UTRA SCells being activated or deactivated, or

-	of up to max{Y2 slot + TSMTC_duration, 5ms} if the activated serving cells are in the same band as any of the E-UTRA SCells being activated or deactivated, provided the cell specific reference signals from the activated serving cells and the E-UTRA SCells being activated or deactivated are available in the same slot, where TSMTC_duration is the longest SMTC duration among all above activated serving cells in SCG.

When one SCell is activated or deactivated:

-	an interruption on any serving cell in SCG:

-	of up to X2 slot, if the activated serving cell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to Y2 slot + TSMTC_duration if the activated serving cells are in the same band as any of the SCells being activated or deactivated, provided the cell specific reference signals from the activated serving cells and the SCells being activated or deactivated are available in the same slot, where, TSMTC_duration is

- the longest SMTC duration among all above activated serving cells in SCG and the SCell being activated when one SCell is activated;

- the longest SMTC duration among all above activated serving cells in SCG when one SCell is deactivated.

Table 8.2.1.2.4-1 Interruption length X2 and Y2 at SCell activation/deactivation

|  | NR Slot length (ms) | Interruption length X2 slot |  | Interruption length Y2 slot |
| --- | --- | --- | --- | --- |
|  |  | Sync | Async |  |
| 0 | 1 | 1 | 2 | 1 |
| 1 | 0.5 | 1 | 2 | 1 |
| 2 | 0.25 | 3 |  | 2 |
| 3 | 0.125 | 5 |  | 4 |

<< End of Change #1>>

<< Start of Change #2>>

##### 8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration

The requirements in this clause shall apply when a supplementary UL carrier or an UL carrier is configured or de-configured in NR non-standalone operation as defined in [2].

When an UL carrier or supplementary UL carrier is configured or deconfigured, an interruption on E-UTRA PCell, all activated E-UTRA SCells, PSCell and all activated SCells within the same FR as the reconfigured uplink carrier of up to X4 slot, is allowed during the RRC reconfiguration procedure [2]. The interruption is for both uplink and downlink of E-UTRA PCell, all activated E-UTRA SCells, PSCell and all activated SCells within the same FR as the configured or de-configured UL.

Table 8.2.1.2.6-1 Interruption length X4 at UL carrier RRC reconfiguration

|  | NR Slot length (ms) | Interruption length X4 slot |  |
| --- | --- | --- | --- |
|  |  | Sync | Async |
| 0 | 1 | 1 | 2 |
| 1 | 0.5 | 2 | 3 |
| 2 | 0.25 | 5 |  |
| 3 | 0.125 | 9 |  |

<< End of Change #2>>

<< Start of Change #3>>

##### 8.2.2.2.1	Interruptions at SCell addition/release

When any number of SCells between one and TBD is added or released using the same RRCConnectionReconfiguration message as defined in [2], the UE is allowed an interruption on any activated serving cell during the RRC reconfiguration procedure as follows:

-	an interruption on any activated serving cell:

-	of up to the duration shown in table 8.2.2.2.1-1, if the activated serving cell is not in the same band as any of the SCells being added or released, or

-	of up to the duration shown in table 8.2.2.2.1-2, if the activated serving cells are in the same band as any of the SCells being added or released, provided the cell specific reference signals from the activated serving cells and the SCells being added or released are available in the same slot.

Table 8.2.2.2.1-1: Interruption duration for SCell addition/release for inter-band CA

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |

Table 8.2.2.2.1-2: Interruption duration for SCell addition/release for intra-band CA

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration |
| 1 | 0.5 | 2 + TSMTC_duration |
| 2 | 0.25 | 4 + TSMTC_duration |
| 3 | 0.125 | 8 + TSMTC_duration |
| Note: TSMTC_duration is - the longest SMTC duration among all above activated serving cells and the SCell being added when one SCell is added; - the longest SMTC duration among all activated serving cells in the same band when one SCell is released. |  |  |

##### 8.2.2.2.2	Interruptions at SCell activation/deactivation

When an intra-band SCell is activated or deactivated as defined in [17], the UE is allowed

-	an interruption on any activated serving cell:

-	of up to the duration shown in table 8.2.2.2.2-1, if the activated serving cell is not in the same band as any of the SCells being activated or deactivated, or

-	of up to the duration shown in table 8.2.2.2.2-2, if the activated serving cells are in the same band as any of the SCells being activated or deactivated provided the cell specific reference signals from the activated serving cells and the SCells being activated or deactivated are available in the same slot.

Table 8.2.2.2.2-1: Interruption duration for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 2 |
| 3 | 0.125 | 4 |

Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 + TSMTC_duration |
| 1 | 0.5 | 2 + TSMTC_duration |
| 2 | 0.25 | 4 + TSMTC_duration |
| 3 | 0.125 | 8 + TSMTC_duration |
| Note: TSMTC_duration is - the longest SMTC duration among all above activated serving cells and the SCell being activated when one SCell is activated; - the longest SMTC duration among all activated serving cells in the same band when one SCell is deactivated. |  |  |

<< End of Change #3>>

<< Start of Change #4>>

##### 8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration

The requirements in this clause shall apply when a supplementary UL carrier or an UL carrier is configured or de-configured in NR standalone carrier aggregation as defined in [2].

When an UL carrier or supplementary UL carrier is configured or deconfigured, an interruption on PCell and all activated SCells within the same FR as the reconfigured uplink carrier of up to the duration shown in table 8.2.2.2.4-1, is allowed during the RRC reconfiguration procedure [2]. The interruption is for both uplink and downlink of PCell and all the activated SCells within the same FR as the configured or de-configured UL.

Table 8.2.2.2.4-1 Interruption duration for UL carrier RRC reconfiguration

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 2 |
| 2 | 0.25 | 4 |
| 3 | 0.125 | 8 |

<< End of Change #4>>