# 3GPP TSG-RAN WG4 Meeting #88bis                                                            R4-1812540

# Chengdu, China, 8th - 12th Oct, 2018

Source: 	ZTE Corporation

Title: 	Some corrections on TS38.141-1 to align with TS38.104

Agenda Item:	7.9.2.1

Document for:	Approval

1	Introduction

In the last meeting, the applicability of test configuration for single band RIB test should be future discussed.

The RIB is the operating band specific radiated interface, which is used to defined the OTA requirements. As stated in the TS38.141-2 [1], the BS type 1-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 1-O capability of operation in multiple operating bands.

In addition, the manufacturer will declare the parameters of the list of single-band RIB and/or multi-band RIB resulting from the supported operating bands, Therefore, applicability of test configuration for single band RIB test is needed.

In this contribution, we provide a TP to capture the test configuration for single band RIB in multi-band operation.

2	Reference

[1] TS38.141-2, v1.0.0

--------------Start of TP  -------------

4.7.3	Applicability of test configurations for single-band RIB

The applicable test configurations are specified in the tables below for each the supported RF configuration, which shall be declared according to subclause 4.6. The generation and power allocation for each test configuration is defined in subclause 4.8. This subclause contains the test configurations for single-band RIB.

For a NR BS declared to be capable of single carrier operation only, a single carrier (SC) shall be used for testing.

For a single-band RIB declared to support multi-carrier and/or CA operation in contiguous spectrum operation, the test configurations in the second column of table 4.7.3-1 shall be used for testing.

For a single-band RIB declared to support multi-carrier and/or CA operation in contiguous and non-contiguous spectrum and where the parameters in the manufacturer's declaration according to subclause 4.6 are identical for contiguous (C) and non-contiguous (NC) spectrum operation, the test configurations in the third column of table 4.7.3-1 shall be used for testing.

For a single-band RIB declared to support multi-carrier and/or CA in contiguous and non-contiguous spectrum and where the parameters in the manufacture's declaration according to subclause 4.6 are not identical for contiguous and non-contiguous spectrum operation, the test configurations in the fourth column of table 4.7.3-1 shall be used for testing.

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported Channel Bandwidth and the smallest supported sub-carrier spacing.

Table 4.7.3-1: Test configurations for a single-band RIB

| BS test case | Contiguous spectrum capable BS | C and NC capable BS with identical parameters | C and NC capable BS with different parameters |
| --- | --- | --- | --- |
| Radiated transmit power | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA base station maximum output power | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA RE Power control dynamic range | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| OTA total power dynamic range | SC | SC | SC |
| OTA transmit ON/OFF power (only applied for NR TDD BS) | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA frequency error | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude | Tested with Error Vector Magnitude |
| OTA error Vector Magnitude | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA time alignment error | NRTC1 | NRTC1 | NRTC1, NRTC3 |
| OTA Occupied bandwidth | SC, NRTC2 (Note 1) | SC, NRTC2 (Note 1) | SC, NRTC2 (Note 1) |
| OTA ACLR | NRTC1 | NRTC1,NRTC3 | NRTC1, NRTC3 |
| OTA CACLR | - | NRTC3 | NRTC3 |
| OTA operating band unwanted emissions | NRTC1, SC (Note 2) | NRTC1, NRTC3, SC (Note 2) | NRTC1, NRTC3, SC (Note 2) |
| OTA transmitter spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA transmitter intermodulation | NRTC1 | NRTC1, NRTC3 | NRTC1, NRTC3 |
| OTA sensitivity | SC | SC | SC |
| OTA reference sensitivity level | SC | SC | SC |
| OTA dynamic range | SC | SC | SC |
| OTA adjacent channel selectivity | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA In-band blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA out-of-band blocking | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA receiver spurious emissions | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA receiver intermodulation | NRTC1 | NRTC3 | NRTC1, NRTC3 |
| OTA in-channel selectivity | SC | SC | SC |
| Note 1: NRTC2 is only applicable when contiguous CA is supported. Note 2: OBUE SC shall be tested using the widest supported Channel Bandwidth and the highest supported sub-carrier spacing. |  |  |  |

For multi-band operation using single-band RIB, the test configuration in Table 4.7.3-2 shall be used for testing.

NOTE:	The applicability of test configurations in table 4.7.3-2 are not applicable to BS type 2-O.

Table 4.7.3-2: Test configurations for a single-band RIB in multi-band operation

| BS test case | Test configuration |
| --- | --- |
| Radiated transmit power | NRTC1/3 (Note 1), NRTC4 |
| OTA base station maximum output power | NRTC1/3 (Note 1), NRTC4 |
| OTA RE Power control dynamic range | Tested with Error Vector Magnitude |
| OTA total power dynamic range | SC |
| OTA transmit ON/OFF power (only applied for NR TDD BS) | NRTC4 |
| OTA frequency error | Tested with Error Vector Magnitude |
| OTA error Vector Magnitude | NRTC1/3 (Note 1), NRTC4 |
| OTA time alignment error | NRTC1/3 (Note 1), NRTC5 (Note 2) |
| OTA Occupied bandwidth | SC, NRTC2 (Note 3) |
| OTA ACLR | NRTC1/3 (Note 1, 5), NRTC5 (Note 4, 5) |
| OTA CACLR | NRTC3 (Note 1, 5) |
| OTA operating band unwanted emissions | NRTC1/3 (Note 1, 5), NRTC5 (Note 5), SC(Note 7) |
| OTA transmitter spurious emissions | NRTC1/3 (Note 1, 5), NRTC5 (Note 5) |
| OTA transmitter intermodulation | NRTC1/3 (Note 1, 5) |
| OTA sensitivity | SC |
| OTA reference sensitivity level | SC |
| OTA dynamic range | SC |
| OTA adjacent channel selectivity | NRTC1/3 (Note 1), NRTC5 (Note 6) |
| OTA In-band blocking | NRTC1/3 (Note 1), NRTC5 (Note 6) |
| OTA out-of-band blocking | NRTC1/3 (Note 1), NRTC5 (Note 6) |
| OTA receiver spurious emissions | NRTC1/3 (Note 1, 5), NRTC5 (Note 5) |
| OTA receiver intermodulation | NRTC1/3 (Note 1), NRTC5 (Note 6) |
| OTA in-channel selectivity | SC |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. Note 2: NRTC5 is only applicable when inter-band CA is supported. Note 3: NRTC2 is only applicable when contiguous CA is supported. Note 4: NRTC5 may be applied for Inter RF Bandwidth gap only. Note 5: Single-band requirement apply to each single-band RIB for both multi-band operation test and single-band operation test. Note 6: NRTC5 is only applicable for multi-band receiver. Note 7: OBUE SC shall be tested using the widest supported Channel Bandwidth and the highest supported sub-carrier spacing. |  |

--------------End of TP ------------