3GPP TSG-RAN WG4 Meeting #88bis						 R4-1812576

Chengdu, CN, 8th – 12th Oct, 2018

Title: 	Discussion on ACS, IBB, OBB for FR1 CA BW Class B and F

Source: 	OPPO

Agenda item:	7.6.12

Document for:	Approval

Introduction

In this contribution, we will further discuss the definition of ACS, IBB and OBB requirements for intra-band contiguous CA.

Discussion

In last RAN4#88 meeting, NR CA bandwidth Class B and F are approved in [1].

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel_CA ≤ BWChannel,max | 1 |  |
| B | 20 MHz ≤ CBW ≤ 50 MHz | 2 |  |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1 |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |
| F | 50 MHz < BWChannel_CA ≤ 100 MHz | 2 | 2 |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |
| NOTE: BWChannel, max is maximum channel bandwidth supported among all bands in a release |  |  |  |

And an WF [2] on ACS, IBB and OBB for intra-band contiguous CA for FR1 was approved. In order to keep consistency in the TS 38.101-1 for intra-band contiguous for two components, the following options should be considered. Companies are encouraged to clarify the feasibility of both options.

Option 1:

- Remove intra-band contiguous CA configuration combinations set for BW Class B and BW F in Rel-15 and define in Rel-16

- Remove ACS, IBB and OBB requirements for intra-band contiguous CA for BW Class B and F in Rel-15 and define in Rel-16

Option 2:

- Mantain intra-band contiguous CA configuration combinations set for BW Class B and BW F in Rel-15

- Define ACS, IBB and OBB requirements for intra-band contiguous CA for BW Class B and F in Rel-15

- Unless agreement is not reached on ACS, IBB and OBB requirements remove BW Class B and F at the end of RAN4#89

Regarding the progress of specification 38.101-1 and 38.101-3, the related requirements of intra-band or inter-band CA and EN-DC have not been well completed. And in last meeting we have reach an agreement that RAN4 will address to establish receiver requirements other than Refesens for EN-DC [3], which means such requirements for EN-DC with high priority is to be concluded for Rel-15 in 2018 Q3 and Q4. Actually, RAN4 will discuss and agree receiver requirement handling for EN-DC mode in following RAN4#88Bis and #89 meeting, and dedicated agenda item will be opened.

In our view, we cannot see urgent demand from operators for inter-band or intra-band CA configuration combinations set for BW Class B and BW F. Due to the limited time and risk for the first UE products of Rel-15, the documents of related receiver requirements for CA BW class B and F including ACS, IBB and OBB can be treated in the following Rel-16, which may need much workload for evaluation. Thus, we suggest removing the related requirement of CA BW Class B and BW F in Rel-15.

In addition, Basket Rel-16 WIDs including NR Intra and inter-band CA/DC for 2 DL bands with up to 2 UL bands [4][5] have been approved in RAN #80 meeting. Here, the intra-band contiguous CA configuration combinations set for BW Class B and BW F can be properly defined in Rel-16, together with definition of ACS, IBB and OBB requirements.

- Remove intra-band contiguous CA configuration combinations set for BW Class B and BW F in Rel-15 and define in Rel-16

- Remove ACS, IBB and OBB requirements for intra-band contiguous CA for BW Class B and F in Rel-15 and define in Rel-16

Proposal 1:    Remove the related requirement for CA combinations for BW Class B and BW F in Rel-15 and define in Rel-16.

Conclusions

Proposal 1:    Remove the related requirement for CA combinations for BW Class B and BW F in Rel-15 and define in Rel-16.

References

[1] R4-1809784, Draft CR to 38.101-1: Corrections on CA bandwidth classes for FR1, 3GPP TSG-RAN WG4 #88 Meeting, Gothenburg, SE, 20th – 24th August 2018

[2] R4-1811806, WF on ACS, IBB and OBB for intra-band contiguous CA for FR1, 3GPP TSG-RAN WG4#88 Meeting, Gothenburg, SE, 20th – 24th August 2018

[3] R4-1811027, Handling of receiver requirements in EN-DC mode, 3GPP TSG-RAN WG4#88 Meeting, Gothenburg, SE, 20th – 24th August 2018

[4] RP-181426, New WID: Basket Rel-16 WID NR Intra, 3GPP TSG-RAN#80 Meeting

[5] RP-180817, New WID: Basket Rel-16 WID NR inter-band CA/DC for 2 DL bands with up to 2 UL bands, 3GPP TSG-RAN#80 Meeting