3GPP TSG-WG RAN4 Meeting #88bis	R4-1812599

# Chengdu, China, 8th – 12nd October, 2018

Source: 	Nokia, Nokia Shanghai Bell, T-Mobile USA

Title: 					TP for TR 37.716-21-11 Introduction of DC_46-66_n261

Agenda Item: 			9.4.3 DC_R16_2BLTE_1BNR_3DL2UL-Core

Document for:	Approval

# Introduction

This TP introduces the following three-band EN-DC configurations to TR 37.716-21-11.

- DL_46D-66A_n261(2A)_UL_66A_n261A

- DL_46C-66A_n261(2A)_UL_66A_n261A

- DL_46A-66A_n261(2A)_UL_66A_n261A

- DL_46D-66A_n261A _UL_66A_n261A

- DL_46C-66A_n261A_UL_266A_n261A

- DL_46A-66A_n261A_UL_66A_n261A

**************************** Start of changes ********************************************

## 6.X	DC_46-66_n261

### 6.X.1	Operating bands for DC

Table 6.X.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_46-66_n261 | CA_46-66 | n261 | No |

6.X.2	Configurations for DC

Table 6.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_46A-66A_n261A DC_46C-66A_n261A DC_46D-66A_n261A DC_46A-66A_n261(2A) DC_46C-66A_n261(2A) DC_46D-66A_n261(2A) | DC_66A_n261A | DC_46A-66A DC_46C-66A DC_46D-66A DC_46A-66A DC_46C-66A DC_46D-66A | n261A CA_n261(2A) |

Table 6.X.2-2: Supported bandwidths per DC band combination of 2 E-UTRA bands and 1 NR band

| DC operating band / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |  |  |
| DC_46A-66A_n261A | 46 | 15 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |  |  |
| DC_46C-66A_n261A | 46 | 15 | See CA_46C bandwidth combination set 0 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 260 |  |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 460 |  |  |  |
| DC_46D-66A_n261A | 46 | 15 | See CA_46D bandwidth combination set 0 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 280 |  |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 480 |  |  |  |
| DC_46A-66A_n261(2A) | 46 | 15 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  | 440 |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 840 |  |  |  |
| DC_46C-66A_n261(2A) | 46 | 15 | See CA_46C bandwidth combination set 0 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  | 460 |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 860 |  |  |  |
| DC_46D-66A_n261(2A) | 46 | 15 | See CA_46D bandwidth combination set 0in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  | 480 |  |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 880 |  |  |  |

### 6.X.3	Spurious emission band UE co-existence for DC

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 66 and n261 has been already made in clause 6.62.3 of TS 37.863-01-01.

Table 6.X.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.X.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_46-66_n261 | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note) |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.X.4	MSD analysis for DC

No IMD issues are expected for DC configuration between the E-UTRA bands below 6GHz and FR2 NR band due to large frequency separation.

The third order harmonic of band 66 may fall into band 46 as already specified in E-UTRA CA_46A-66A. The exclusion zone specified in TS36.101 Table 7.3.1A-0eC is applicable to have no MSD also for this EN-DC. The MSD analysis available in clause 6.23 of TR36.714-02-01 is also applicable to this EN-DC.

### 6.X.5	∆TIB and ∆RIB values

For DC_46-66-n261, the TIB,c and RIB values are given in the tables below, reusing E-UTRA CA configuration CA_46-66 for band 46 and 66 while no relaxation is introduced to band n261.

Note that RIB for band 46 is defined relative to the REFSENS of E-UTRA CA_46A-66A in TS36.101 as the single carrier REFSENS is not defined for band 46.

Table 6.X.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_46-66_n261 | 66 | 0 |
|  | n261 | 0 |

Table 6.X.5-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_46-66_n261 | 46 | 0 |
|  | 66 | 0 |
|  | n261 | 0 |

**************************** End of changes ********************************************