3GPP TSG-RAN WG4 Meeting #88bis	R4-1812603

Chengdu, China, 8th - 12th Oct, 2018

Agenda item:	7.9.2.3

Source: 	ZTE Corporation .

Title: 	TP to TS 38.141-2: FR2 test model(Section 4.9.3)

Document for:	Approval

Introduction

This contribution is a text proposal to draft TS 38.141-2 [1] to complete FR2 test model.

Text Proposal

--------------Start of text proposal-------------

4.9.3	Test models

4.9.3.1	General

The NR test models described in TS 38.141-1 is also applicable for 1-O. The following sections will describe the NR test models needed for 2-O.

4.9.3.2	NR test models

The set-up of physical channels for transmitter tests shall be according to one of the NR test models (NR-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	[The test models are defined for a single antenna port (using p = 0); 1 code word (q = 0), 1 layer, precoding is not used; unless specified otherwise]

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	Normal CP

-	Virtual resource blocks of localized type

For NR TDD, test models are derived based on the uplink/downlink configuration as showing in the table4.9.3.2-1.

Table4.9.3.2-1: Configurations of TDD gNB test models

| SCS [KHz] | 60 | 120 |
| --- | --- | --- |
| tdd-UL-DL-Configuration, SIB1 |  |  |
| uref [KHz], referenceSubcarrierSpacing | 60 | 120 |
| P [ms],dl-UL-TransmissionPeriodicity | 5 | 5 |
| dslots , nrofDownlinkSlots | 14 | 29 |
| dsym ,nrofDownlinkSymbols | 12 | 10 |
| uslots,nrofUplinkSlots | 4 | 9 |
| usym, nrofUplinkSymbols | 8 | 2 |
| - Note: slot setting is configured by the tdd-UL-DL-Configuration, SIB1. |  |  |

4.9.3.2.1	NR test model 1.1 (NR-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

Table 4.9.3.2-1: Physical channel parameters of NR-TM1.1 for x kHz SCS with y MHz bandwidth

| Parameter | SCS: x kHz, BW: y MHz |
| --- | --- |
| PDCCH |  |
| # of symbols used for control channel | [1] |
| # of CCEs allocated to PDCCH | 1 |
| # of available REGs | 6 |
| Aggregation level(s) | 1 |
| # of RBs not allocated by PDCCH in the first symbol (# of RBs for PDSCH) | NRB – 6 |
| DM-RS configuration and density | Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs |
| PDSCH |  |
| DM-RS configuration and density | Type 1, Comb2 (every other subcarrier) in symbols 3 |
| PT-RS pattern and density | Distributed pattern, every symbol for every second PRB (time density 1 and frequency density ½) |
| # of QPSK PDSCH PRBs which are boosted or deboosted | 0 |

Where NRB is considering the maximum transmission bandwidth configuration seen in table 5.3.2-2.

4.9.3.2.2	NR test model 2 (NR-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Editor’s note: Physical channel parameters for TM2 to be added.

4.9.3.2.3	NR test model 3.1 (NR-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation

Editor’s note: Physical channel parameters for TM3.1 to be added.