TSG-RAN Working Group 4 meeting #88-bis	R4-1812771

Chengdu, China, 8 - 12 October 2018

Source:	Ericsson

Title:	Simulation results for PRACH requirements

Agenda item:	7.13.2.4

Document for:	Discussion

# Introduction

In this contribution, simulation results for initial simulation alignments based on agreed assumptions in [1] are presented herein.

# Simulation

## Simulation setups

The parameters of aforementioned formats for PRACH preamble is specified in [2]. The initial simulation results to be presented in section 2.2 are obtained using the agreed simulation parameters [1]:

| Parameters | Short sequence (Format B4, A3) |  | Long sequence (Format 0) |
| --- | --- | --- | --- |
|  | FR1 | FR2 |  |
| Number of Tx | 1 |  | 1 |
| Number of Rx | 2 |  | 2 |
| Carrier frequency (GHz) | 4 | 30 |  |
| Sub-Carrier Spacing (kHz) | 15, 30 | 60, 120 | 1.25 |
| Propagation Channel | AWGN |  | AWGN |
| Symbols length (in terms of PUSCH symbols) | 14 |  | 14 |
| Cyclic shift Ncs | 46 | 0 |  |
| Preamble start symbol index | 0 |  |  |
| Test metric | False alarm probability ≤ 0.1% |  | False alarm probability ≤ 0.1% |
| Timing offset | 0 |  |  |
| Frequency offset | 0 |  |  |

## Simulation results

In this sub-section, the simulation results are presented. Further discussions are provided in [3].

Figure 1: Simulation results for format 0

Table 1: Simulation results for format 0

|  | SNR [dB] for 99% detection probability |
| --- | --- |
| Format 0 | -16.5 |

Figure 2. Simulation results for format B4

Table 2. Simulation results for format B4

|  | SNR [dB] for 99% detection probability |  |  |  |
| --- | --- | --- | --- | --- |
| SCS (kHz) | 15 | 30 | 60 | 120 |
| Format B4 |  |  |  |  |
|  | -19.3 | -19.3 | -19 | -19 |

eTable 3. Simulation results for A3, Ncs=46

|  | SNR [dB] for 99% detection probability |  |  |  |
| --- | --- | --- | --- | --- |
| SCS (kHz) | 15 | 30 | 60 | 120 |
| Format A3 |  |  |  |  |
|  | -16.4 | -16.4 |  |  |

# Conclusion

Simulation results for initial simulation alignments are presented in Section 2.

# References

R4-1811728, “WF on NR PRACH demodulation requirements”, Ericsson

3GPP TS 38.211 v15.2.0 (2018-07) Release 15

R4-1812770, “PRACH performance requirement discussion v0”, Ericsson