3GPP TSG-RAN WG4 Meeting #88bis	R4-1812833

Chengdu, China, 8 – 12 October 2018

Agenda Item:	6.5.6.2

Source:	Ericsson

Title:	Simulation results of CQI requirements for eFeMTC

Document for:	Discussion

# 1	Introduction

RAN4#88 agreed with the way forward on eFeMTC UE demodulation and CSI reporting requirements [1].

- Way forward for CQI definition test for CQI table 5 for BL/CE UEs - Companies are encouraged to provide the simulation results to set the SNR test points corresponding to 64QAM - Way forward for CQI definition test for CQI table 6 for BL/CE UEs - Companies are encouraged to provide the simulation results to set the SNR test points corresponding to 16QAM

This contribution shows our simulation results for new CQI table and discusses the test point.

# 2	Discussion

Figure 1 shows the median of reported CQI for 4-bit CQI table 5 (CQI table for BL/CE UE supporting 64QAM) and CQI table 6 (CQI table for BL/CE UE supporting wide SNR range).

Figure 1	Median CQI for 4-bit CQI table 5 and 6.

## 2.1	Simulation results for CQI table 5

According to the agreement in the last meeting, we set test points corresponding to CQI index is 11 or larger for CQI table 5. From our simulation results, it is observed that the test point is SNR=11dB or larger. If we consider the coding rate of 0.5, we can select the test points corresponding to CQI index 11 (CR=0.45) or 12 (CR=0.55). If we choose SNR test points corresponding to CQI index 11, some UE may select CQI index 10 which corresponding to 16QAM. Considering the margin, we propose to set the test points corresponding to CQI index 12, that is SNR = 13/14dB.

For reference we should the distribution of reported CQI and BLER with median CQI+1 in Table 1.

Proposal 1: For the CQI definition test with CQI table 5, set test points to SNR=13/14dB. Reuse the same requirements used for the existing CQI definition test.

Table 1	Simulation results of CQI definition test with table 5.

| SNR [dB] | Median CQI | Percentage of reported CQI indexes which is median CQI, median CQI +/- 1 | BLER at median CQI | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- |
| 11 | 11 | 100% | 0.00 | 1.00 |
| 12 | 11 | 100% | 0.00 | 0.59 |
| 13 | 12 | 100% | 0.00 | 1.00 |
| 14 | 12 | 100% | 0.00 | 0.79 |
| 15 | 13 | 100% | 0.00 | 1.00 |
| 16 | 13 | 100% | 0.00 | 0.80 |
| 17 | 14 | 100% | 0.00 | 1.00 |
| 18 | 14 | 100% | 0.00 | 0.97 |

## 2.2	Simulation results for CQI table 6

For CQI table 6, on the other hand, the agreement is to set SNR test points corresponding to 16QAM (CQI index: 9 to 12). From the simulation results, considering the margin, SNR=8/9dB are good candidates for 16QAM. For reference we should the distribution of reported CQI and BLER with median CQI+/-1 in Table 2.

Proposal 2: For the CQI definition test with CQI table 6, set test points SNR=8/9dB. Reuse the same requirements used for the existing CQI definition test.

Table 2	Simulation results of CQI definition test with table 6.

| SNR [dB] | Median CQI | Percentage of reported CQI indexes which is median CQI, median CQI +/- 1 | BLER at median CQI | BLER at Median CQI -1 | BLER at Median CQI +1 |
| --- | --- | --- | --- | --- | --- |
| 5 | 9 | 100 | 0.53 | 0.00 | 1.00 |
| 6 | 9 | 100 | 0.00 | 0.00 | 1.00 |
| 7 | 9 | 100 | 0.00 | 0.00 | 0.65 |
| 8 | 10 | 100 | 0.00 | 0.00 | 1.00 |
| 9 | 10 | 100 | 0.00 | 0.00 | 0.99 |
| 10 | 11 | 100 | 0.07 | 0.00 | 1.00 |
| 11 | 11 | 100 | 0.01 | 0.00 | 0.99 |
| 12 | 12 | 100 | 0.04 | 0.00 | 1.00 |

For CQI-to-MCS table, there are two options in [1]. Table 3 shows the absolute difference between the effective coding rate and target coding rate. Note the effective code rate is given by |R-(A+24)/Nch|, considering the number of code blocks is 1 for 3PRB allocation. From this table it is observed option 1 minimizes the difference between the target coding rate and effective coding rate.

Table 3	Effective coding rate for each CQI2MCS option.

|  |  | Option 1 |  |  | Option 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Allocated PRBs | 3 |  |  | 3 |  |  |
|  | Number of resource elements | 378 |  |  | 378 |  |  |
|  | Target coding rate | MCS | Effective code rate | Abs diff from target CR | MCS |  | Abs diff from target CR |
| 0 | N/A | DTX |  |  | DTX |  |  |
| 1 | 0.05 | 0 | 0.10 | 0.05 | 0 | 0.10 | 0.05 |
| 2 | 0.2 | 2 | 0.22 | 0.02 | 1 | 0.15 | 0.05 |
| 3 | 0.26 | 3 | 0.26 | 0 | 2 | 0.22 | 0.04 |
| 4 | 0.19 | 2 | 0.22 | 0.03 | 1 | 0.15 | 0.04 |
| 5 | 0.14 | 1 | 0.15 | 0.01 | 1 | 0.15 | 0.01 |
| 6 | 0.26 | 3 | 0.26 | 0 | 2 | 0.22 | 0.04 |
| 7 | 0.44 | 7 | 0.47 | 0.03 | 6 | 0.37 | 0.07 |
| 8 | 0.62 | 9 | 0.63 | 0.01 | 9 | 0.63 | 0.01 |
| 9 | 0.41 | 12 | 0.40 | 0.01 | 12 | 0.40 | 0.01 |
| 10 | 0.544 | 15 | 0.571 | 0.027 | 14 | 0.508 | 0.036 |
| 11 | 0.68 | 16 | 0.61 | 0.07 | 16 | 0.61 | 0.07 |
| 12 | 0.83 | 16 | 0.61 | 0.22 | 16 | 0.61 | 0.22 |
| 13 | 0.64 | 22 | 0.62 | 0.02 | 21 | 0.58 | 0.06 |
| 14 | 0.76 | 25 | 0.78 | 0.02 | 24 | 0.72 | 0.04 |
| 15 | 0.87 | 27 | 0.83 | 0.04 | 27 | 0.83 | 0.04 |

Proposal 3: Use the following CQI2MCS table for CQI reporting test with 64QAM table.

| CQI index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS index | DTX | 0 | 2 | 3 | 2 | 1 | 3 | 7 | 9 | 12 | 15 | 16 | 16 | 22 | 25 | 27 |

## 2.3	Applicability

According to RAN1/RAN2 decision, both supporting CQI tables 5 and 6 are optional capabilities. For Cat-M1/Cat-M2 UEs, CQI reporting tests are applicable to Cat-M1/M2 UEs supporting the corresponding CQI tables.

For non-BL UEs capable of coverage enhancement, reporting CQI corresponding to 64QAM has already been verified in the existing CQI reporting test for higher category UEs. We don’t think CQI reporting test with CQI table 5 does not need to be applied for non-BL CE UEs.

On the other hand, CQI reporting test with CQI table 6 is new for non-BL UEs also. Therefore we propose the CQI reporting test with CQI table 6 is applied for non-BL UEs. As same as existing CQI reporting test for non-BL UEs, we propose to decrease 3dB for 2Rx and 6dB for 4Rx from the SNR test points for BL UE case.

Proposal 4: CQI reporting test with CQI table 6 is applicable for non-BL UEs with coverage enhancement. SNR test points are set by decreasing 3dB for 2Rx UE and 6dB for 4Rx UEs.

# 3	Summary

Proposal 1: For the CQI definition test with CQI table 5, set test points to SNR=13/14dB. Reuse the same requirements used for the existing CQI definition test.

Proposal 2: For the CQI definition test with CQI table 6, set test points SNR=8/9dB. Reuse the same requirements used for the existing CQI definition test.

Proposal 3: Use the following CQI2MCS table for CQI reporting test with 64QAM table.

| CQI index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS index | DTX | 0 | 2 | 3 | 2 | 1 | 3 | 7 | 9 | 12 | 15 | 16 | 16 | 22 | 25 | 27 |

Proposal 4: CQI reporting test with CQI table 6 is applicable for non-BL UEs with coverage enhancement. SNR test points are set by decreasing 3dB for 2Rx UE and 6dB for 4Rx UEs.

# 4	References

R4-1811372, “Way forward on eFeMTC UE demodulation and CSI”, Ericsson.

# Appendix

Table 4	4-bit CQI Table 3. (CQI table for BL/CE UE introduced in Rel-13).

| CQI index | modulation | code rate x 1024 x | efficiency x |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 40 | 0.0781 |
| 2 | QPSK | 78 | 0.1523 |
| 3 | QPSK | 120 | 0.2344 |
| 4 | QPSK | 193 | 0.3770 |
| 5 | QPSK | 308 | 0.6016 |
| 6 | QPSK | 449 | 0.8770 |
| 7 | QPSK | 602 | 1.1758 |
| 8 | 16QAM | 378 | 1.4766 |
| 9 | 16QAM | 490 | 1.9141 |
| 10 | 16QAM | 616 | 2.4063 |
| 11 | Reserved | Reserved | Reserved |
| 12 | Reserved | Reserved | Reserved |
| 13 | Reserved | Reserved | Reserved |
| 14 | Reserved | Reserved | Reserved |
| 15 | Reserved | Reserved | Reserved |

Table 5	4-bit CQI Table 5. (CQI table for BL/CE UE supporting 64QAM).

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 40 | 0.0781 |
| 2 | QPSK | 78 | 0.1523 |
| 3 | QPSK | 120 | 0.2344 |
| 4 | QPSK | 193 | 0.3770 |
| 5 | QPSK | 308 | 0.6016 |
| 6 | QPSK | 449 | 0.8770 |
| 7 | QPSK | 602 | 1.1758 |
| 8 | 16QAM | 378 | 1.4766 |
| 9 | 16QAM | 490 | 1.9141 |
| 10 | 16QAM | 616 | 2.4063 |
| 11 | 64QAM | 466 | 2.7305 |
| 12 | 64QAM | 567 | 3.3223 |
| 13 | 64QAM | 666 | 3.9023 |
| 14 | 64QAM | 772 | 4.5234 |
| 15 | 64QAM | 873 | 5.1152 |

Table 6	4-bit CQI Table 6. (CQI table for BL/CE UE supporting wide SNR range).

| CQI index | modulation | code rate x 1024 | repetition |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 56 | 32 |
| 2 | QPSK | 207 | 16 |
| 3 | QPSK | 266 | 4 |
| 4 | QPSK | 195 | 2 |
| 5 | QPSK | 142 | 1 |
| 6 | QPSK | 266 | 1 |
| 7 | QPSK | 453 | 1 |
| 8 | QPSK | 637 | 1 |
| 9 | 16QAM | 423 | 1 |
| 10 | 16QAM | 557 | 1 |
| 11 | 16QAM | 696 | 1 |
| 12 | 16QAM | 845 | 1 |
| 13 | 64QAM | 651 | 1 |
| 14 | 64QAM | 780 | 1 |
| 15 | 64QAM | 888 | 1 |