3GPP TSG-RAN WG4 WG4 #88bis 	R4-1812852

# Chengdu, China, 8 – 12 October 2018

Source:	Ericsson

Title:	TP to TS 38.101-4: 7.4 FR2 PBCH demodulation

Agenda item:	7.13.1.3

Document for:	Approval

## Background

This TP provides the PBCH demodulation requirements FR2.

## Summary

This TP specifies the PBCH demodulation requirements for FR2. This TP also specifies the RMC used for PBCH demodulation requirements.

## Proposal

It is proposed that the attached text proposal is included in TS 38.101-4

## References

- R4-1811730, “Way forward on NR PBCH demodulation requirements”, Huawei, HiSilicon, China Telecom, CMCC, NTT DOCOMO, Vodafone, Orange, Sprint, AT&T, KDDI, SoftBank,  Deutsche Telekom, Nokia, Nokia Shanghai Bell, SKT.

TEXT PROPOSAL:

## 7.4	PBCH demodulation requirements

The receiver characteristics of PBCH are determined by the probability of miss-detection of the PBCH (Pm-bch), which is defined as

Pm-bch=1-AB

Where A is the number of correctly decoded MIB PDUs and B is the number of transmitted MIB PDUs. The Pm-bch is derived with the assumption UE combines the PBCH symbols of the same SS/PBCH block index within the MIB TTI (80ms).

Table 7.4-1 Test parameters for PBCH

| Parameter | Unit | Single antenna port |
| --- | --- | --- |
| Noc at antenna port | dBm/Hz | -98 |
| Cell ID |  | 0 |
| Cyclic prefix |  | Normal |
| Number of SS/PBCH blocks within an SS burst set periodicity |  | 1 |
| SS/PBCH block index Note1 |  | 0 |
| SS/PBCH block periodicity | ms | 20 |
| Reference carrier spacing Note2 | kHz | 120 |
| Slot configuration period Note2 | ms | [0.625] |
| Number of downlink slots Note2 | slots | [3] |
| Number of downlink symbols Note2 | symbols | [10] |
| Number of uplink symbols Note2 | symbols | [2] |
| Number of uplink slots Note2 | slots | [1] |
| Note 1 as specified in 4.1 in TS 38.213 [nn] Note 2 as specified in 11.1 in TS 38.213 [nn] |  |  |

### 7.4.1	1RX requirements (Void)

### 7.4.2	2Rx requirements

#### 7.4.2.1	FDD (Void)

#### 7.4.2.2	TDD

Table 5.4.2.2-2	Minimum performance PBCH

|  |  |  |  |  |  | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test number | Channel bandwidth | SS/PBCH block subcarrier spacing | Reference channel | Propagation condition | Antenna configuration and correlation matrix | Pm-bch (%) | PBCH SNR (dB) |
| 1 | 100 MHz | 120 kHz | R.yy.x | TBD | 1 x 2 Low | 1 | TBD |
| 2 | 100 MHz | 240 kHz | R.yy.y | TBD | 1 x 2 Low | 1 | TBD |

*********************End of change*****************

*********************Next changed section*****************

### A.y.x	Reference measurement channels for PBCH demodulation requirements

Table A.y.x-1: Reference Channel TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channels |  | R.yy.x | R.yy.y |
| Number of transmitter antennas |  | 1 | 1 |
| SS/PBCH block subcarrier spacing | kHz | 120 | 240 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 56/864 | 56/864 |
| Payload (without CRC and timing related PBCH payload bits) | bits | 24 | 24 |

*********************End of change*****************