3GPP TSG-RAN WG4#88bis	R4-1813102

Chengdu, P.R of China, 8 – 12 October 2018

Agenda Item:	6.5.4.1

Source:	Ericsson

Title:	Simulation results for efeMTC WUS receptions

Document for:	Discussion

# Introduction

At last RAN4 meeting, RAN4 discussed and agreed to introduce WUS reception requirements for MTC in [1]. As per the agreements in [1], the WUS reception requirements are specified per DRX cycle, coverage level and number of transmit antennas. The required number of repetitions are currently TBD. In this contribution, the simulation results for WUS reception performance for different DRX cycle lengths and coverage levels are presented with the aim to resolve the TBDs. Our companion paper in [2] contains the proposed changes to the specification based on these results.

# Simulation assumption for WUS for efeMTC

Table 1 shows the agreed simulation assumptions [1].

Table 1: Simulation assumption for efeMTC

| Parameter | Value |
| --- | --- |
| BS TX antenna configuration | 1 Tx, 2 Tx with transmit diversity Note: Tx diversity scheme will depend on RAN1 agreements |
| UE Rx antenna configuration | 1 Rx |
| WUS BW | 2 PRBs |
| Channel model | EPA 1Hz, ETU 1 Hz, EPA 5 Hz, ETU 30 Hz, |
| SNR | -6 dB, -15 dB |
| False alarm rate | 1% |
| Detection probability | 99% |

Based on the simulation study, RAN4 is to resolve the TBD values as agreed in [3]:

Table 2 Conditions for WUS reception for UE normal coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| 0.32 | TBD | TBD |
| 0.64 | TBD | TBD |
| 1.28 | TBD | TBD |
| 2.56 | TBD | TBD |

Table 3 Conditions for WUS reception for UE enhanced coverage level

| DRX cycle length [s] | Required number of repetition of WUS signal with 1 transmit antenna | Required number of repetition of WUS signal with 2 transmit antennas |
| --- | --- | --- |
| 0.32 | TBD | TBD |
| 0.64 | TBD | TBD |
| 1.28 | TBD | TBD |
| 2.56 | TBD | TBD |

# Simulation results

## WUS simulation results for normal coverage/CEModeA

Table 2 WUS reception results for EPA 5 Hz

|  |  |  | 1 Tx |  | 2 Tx |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel | Doppler | DRX | Rmax | SNR | Rmax | SNR |
| EPA | 5 | 0 | 1 | -0,4 | 2 | -12,4 |
| EPA | 5 | 1,28 | 1 | -0,4 | 2 | -12,2 |
| EPA | 5 | 2,56 | 1 | -0,6 | 2 | -12,3 |
| EPA | 5 | 5,12 | 1 | 0,4 | 2 | -11,7 |
| EPA | 5 | 10,24 | 1 | 6,5 | 2 | -9,8 |
| EPA | 5 | 0 | 2 | -2,5 | 4 | -14,9 |
| EPA | 5 | 1,28 | 2 | -2,2 | 4 | -14,9 |
| EPA | 5 | 2,56 | 2 | -2,4 | 4 | -14,7 |
| EPA | 5 | 5,12 | 2 | -1,6 | 4 | -14,2 |
| EPA | 5 | 10,24 | 2 | 1,2 | 4 | -12,4 |
| EPA | 5 | 0 | 4 | -5,8 | 8 | -16,8 |
| EPA | 5 | 1,28 | 4 | -5,8 | 8 | -16,5 |
| EPA | 5 | 2,56 | 4 | -5,3 | 8 | -16,6 |
| EPA | 5 | 5,12 | 4 | -5 | 8 | -15,9 |
| EPA | 5 | 10,24 | 4 | -1,7 | 8 | -14,3 |
| EPA | 5 | 0 | 8 | -8,5 | 16 | -19,3 |
| EPA | 5 | 1,28 | 8 | -8,2 | 16 | -19,1 |
| EPA | 5 | 2,56 | 8 | -8,2 | 16 | -19,1 |
| EPA | 5 | 5,12 | 8 | -7,5 | 16 | -18,4 |
| EPA | 5 | 10,24 | 8 | -5,1 | 16 | -16,8 |
| EPA | 5 | 0 | 16 | -12,8 |  |  |
| EPA | 5 | 1,28 | 16 | -12,4 |  |  |
| EPA | 5 | 2,56 | 16 | -12,4 |  |  |
| EPA | 5 | 5,12 | 16 | -11,9 |  |  |
| EPA | 5 | 10,24 | 16 | -10,1 |  |  |

Table 3 WUS reception results for ETU 30 Hz

|  |  |  | 1 Tx |  | 2 Tx |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel | Doppler | DRX | Repetitions | SNR | Repetitions | SNR |
| ETU | 30 | 0 | 1 | n/a | 2 | -11,9 |
| ETU | 30 | 1,28 | 1 | n/a | 2 | -11,9 |
| ETU | 30 | 2,56 | 1 | n/a | 2 | -11,7 |
| ETU | 30 | 5,12 | 1 | n/a | 2 | -11,3 |
| ETU | 30 | 10,24 | 1 | n/a | 2 | -9,3 |
| ETU | 30 | 0 | 2 | n/a | 4 | -15,1 |
| ETU | 30 | 1,28 | 2 | n/a | 4 | -15,3 |
| ETU | 30 | 2,56 | 2 | n/a | 4 | -14,9 |
| ETU | 30 | 5,12 | 2 | n/a | 4 | -14,2 |
| ETU | 30 | 10,24 | 2 | n/a | 4 | -12,4 |
| ETU | 30 | 0 | 4 | -6 | 8 | -18,5 |
| ETU | 30 | 1,28 | 4 | -6 | 8 | -18,3 |
| ETU | 30 | 2,56 | 4 | n/a | 8 | -18,1 |
| ETU | 30 | 5,12 | 4 | n/a | 8 | -17,7 |
| ETU | 30 | 10,24 | 4 | n/a | 8 | -15,7 |
| ETU | 30 | 0 | 8 | -13,2 | 16 | -22,2 |
| ETU | 30 | 1,28 | 8 | -13,88 | 16 | -22,2 |
| ETU | 30 | 2,56 | 8 | -13,2 | 16 | -21,9 |
| ETU | 30 | 5,12 | 8 | -13 | 16 | -21,4 |
| ETU | 30 | 10,24 | 8 | -10,6 | 16 | -19,6 |
| ETU | 30 | 0 | 16 | -18,7 |  |  |
| ETU | 30 | 1,28 | 16 | -18,7 |  |  |
| ETU | 30 | 2,56 | 16 | -18,6 |  |  |
| ETU | 30 | 5,12 | 16 | -18,1 |  |  |
| ETU | 30 | 10,24 | 16 | -16,1 |  |  |

## WUS simulation results for enhanced coverage/CEModeB

Table 4 WUS reception results for EPA 1 Hz

|  |  |  | 1 Tx |  | 2 Tx |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel | Doppler | DRX | Repetitions | SNR | Repetitions | SNR |
| EPA | 1 | 0 | 2 | -2,2 | 2 | -12,3 |
| EPA | 1 | 1,28 | 2 | -2,2 | 2 | -12,1 |
| EPA | 1 | 2,56 | 2 | -1,6 | 2 | -12,3 |
| EPA | 1 | 5,12 | 2 | -2,2 | 2 | -11,7 |
| EPA | 1 | 10,24 | 2 | 0,9 | 2 | -9,8 |
| EPA | 1 | 0 | 4 | -5,1 | 4 | -14,9 |
| EPA | 1 | 1,28 | 4 | -6 | 4 | -14,7 |
| EPA | 1 | 2,56 | 4 | -4,3 | 4 | -14,7 |
| EPA | 1 | 5,12 | 4 | -4,3 | 4 | -14,2 |
| EPA | 1 | 10,24 | 4 | -2,3 | 4 | -12,3 |
| EPA | 1 | 0 | 8 | -7 | 8 | -16,7 |
| EPA | 1 | 1,28 | 8 | -6,5 | 8 | -16,4 |
| EPA | 1 | 2,56 | 8 | -6,6 | 8 | -16,4 |
| EPA | 1 | 5,12 | 8 | -6,8 | 8 | -15,9 |
| EPA | 1 | 10,24 | 8 | -5,4 | 8 | -14,2 |
| EPA | 1 | 0 | 16 | -10,1 | 16 | -18,8 |
| EPA | 1 | 1,28 | 16 | -10 | 16 | -18,7 |
| EPA | 1 | 2,56 | 16 | -9,6 | 16 | -18,6 |
| EPA | 1 | 5,12 | 16 | -9,2 | 16 | -18,2 |
| EPA | 1 | 10,24 | 16 | -5,9 | 16 | -16,4 |

Table 5 WUS reception results for ETU 1 Hz

|  |  |  | 1 Tx |  | 2 Tx |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel | Doppler | DRX | Repetitions | SNR | Repetitions | SNR |
| ETU | 1 | 0 | 1 | 4,8 | 2 | -12,3 |
| ETU | 1 | 1,28 | 1 | -5,3 | 2 | -11,8 |
| ETU | 1 | 2,56 | 1 | -4,9 | 2 | -11,8 |
| ETU | 1 | 5,12 | 1 | -3,8 | 2 | -11,3 |
| ETU | 1 | 10,24 | 1 | -1,7 | 2 | -9,4 |
| ETU | 1 | 0 | 2 | -7,1 | 4 | -14,8 |
| ETU | 1 | 1,28 | 2 | -7,1 | 4 | -14,6 |
| ETU | 1 | 2,56 | 2 | -6,9 | 4 | -14,5 |
| ETU | 1 | 5,12 | 2 | -6,5 | 4 | -14,2 |
| ETU | 1 | 10,24 | 2 | -4,1 | 4 | -12,1 |
| ETU | 1 | 0 | 4 | -10 | 8 | -16,9 |
| ETU | 1 | 1,28 | 4 | -9,7 | 8 | -16,6 |
| ETU | 1 | 2,56 | 4 | -9,7 | 8 | -16,6 |
| ETU | 1 | 5,12 | 4 | -8,9 | 8 | -15,9 |
| ETU | 1 | 10,24 | 4 | -7,2 | 8 | -14,1 |
| ETU | 1 | 0 | 8 | -11,5 | 16 | -19,2 |
| ETU | 1 | 1,28 | 8 | -11,9 | 16 | -18,8 |
| ETU | 1 | 2,56 | 8 | -11,9 | 16 | -19 |
| ETU | 1 | 5,12 | 8 | -11,4 | 16 | -18,5 |
| ETU | 1 | 10,24 | 8 | -9,3 | 16 | -16,6 |
| ETU | 1 | 0 | 16 | -14,7 | 32 | -20,9 |
| ETU | 1 | 1,28 | 16 | -14,4 | 32 | -20,9 |
| ETU | 1 | 2,56 | 16 | -14,2 | 32 | -20,6 |
| ETU | 1 | 5,12 | 16 | -13,8 | 32 | -20,1 |
| ETU | 1 | 10,24 | 16 | -11,5 | 32 | -18,5 |
| ETU | 1 | 0 | 32 | -16,9 |  |  |
| ETU | 1 | 1,28 | 32 | -16,4 |  |  |
| ETU | 1 | 2,56 | 32 | -16,4 |  |  |
| ETU | 1 | 5,12 | 32 | -15,4 |  |  |
| ETU | 1 | 10,24 | 32 | -14,1 |  |  |

# Summary

The results are summarized in Table 6 based on worst case.

Table 6: Summary of results for receiving WUS in normal and enhanced coverage

| DRX cycle length [s] | Required no. of Rep. for normal coverage with Tx-Div | Required no. of Rep. for normal coverage w/o Tx-Div | Required no. of Rep. for enhanced coverage with Tx-Div | Required no. of Rep. for enhanced coverage w/o Tx-Div |
| --- | --- | --- | --- | --- |
| 0.32 | TBD | TBD | TBD | TBD |
| 0.64 | TBD | TBD | TBD | TBD |
| 1.28 | 2 | 8 | 8 | 32 |
| 2.56 | 2 | 8 | 8 | 32 |

In this contribution, the simulation results for minimum WUS reception performance for release 15 efeMTC are presented based on agreed simulation assumptions. We kindly ask the companies to consider these results to resolve the TBDs in the current tables for minimum reception requirements for WUS as agreed in [1]. Based on the results, following observations are made:

- Proposal: WUS reception requirements are defined based on the required number of repetitions in Table 6.

Note: The simulation results for DRX cycle lengths of 0.32 s and 0.64 s will be presented in a revision once results are available.

# Reference

R4-1811690, “WUS RRM requirements for Rel-15 MTC”, Ericsson, Sony

R4-1813103, “Introduction of WUS requirements for efeMTC”, Ericsson