3GPP TSG-RAN WG4 Meeting #88b	R4-1813166

# Chengdu, China, October 8 – 12, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on updating requirement for BWP switching interruption in TS38.133 (Section 8.2.1 and 8.2.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Interruption requirements due to active BWP switch is not limited to the same frequency range - There is no definition of TBWP_switching_delay_DCI and TBWP_switching_delay_timer defined in section 8.6. - Interruption should be allowed if MIMO-related parameters are changed - Interruption may exceed the BWP switch delay if the victim cell has larger SCS than the cell wherein UE performs BWP switching |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the condition of the same frequency range in the introduction sections - Remove TBWP_switching_delay_DCI and TBWP_switching_delay_timer - Add maxNrofCodeWordsScheduledByDCI and nrofSRS-Ports which will cause interruptions - Clarify that only the start of the interruption needs to be confined within BWP switch delay - Update the requirement structure for better readability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement for BWP switching interruption is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1, 8.2.1.2.7, 8.2.2.1, 8.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS/TR ... CR ...TS38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF 1st  CHANGES----------------------------

#### 8.2.1.1	Introduction

This section contains the requirements related to the interruptions on PSCell, and SCell, when

E-UTRA PCell transitions between active and non-active during DRX, or

E-UTRA PCell transitions from non-DRX to DRX, or

E-UTRA SCell in MCG or SCell in SCG is added or released, or

E-UTRA SCell in MCG or SCell in SCG is activated or deactivated, or

measurements on SCC with deactivated SCell in either E-UTRA MCG or NR SCG

This section also contains the requirements related to the interruptions on other active serving cells wherein the UE is performing BWP switching.

The requirements shall apply for E-UTRA-NR DC with an E-UTRA PCell.

This section contains interruptions where victim cell is PSCell or SCell belonging to SCG. Requirements for interruptions requirements when the victim cell is E-UTRA PCell or E-UTRA SCell belonging to MCG are specified in [15].

For a UE which does not support per-FR measurement gaps, interruptions to the PSCell or active SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on any frequency range. For UE which support per-FR gaps, interruptions to the PSCell or active SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on the same frequency range as the victim cell.

----------------------END OF 1st CHANGES----------------------------

----------------------START OF 2nd CHANGES----------------------------

##### 8.2.1.2.7	Interruption due to Active BWP switching Requirement

When BWP switch involves SCS changes, UE is allowed to cause interruption of up to X slot to other active serving cells.

When BWP switch involves parameters locationAndBandwidth, maxNrofCodeWordsScheduledByDCI and nrofSRS-Ports without SCS, the UE is allowed to cause interruption of up to X slot to other active serving cells if the UE is not capable of per-FR gap, or to other active serving cells in the same frequency range wherein the UE is performing BWP switching if UE is capable of per-FR gap.

Interruption length X is defined in Table 8.2.1.2.7-1. The start of the interruption is only allowed within the BWP switching delay defined in clause 8.6.2 in TS38.133.

Interruptions are not allowed during BWP switch involving only change in other parameter not mentioned above.

Table 8.2.1.2.7-1: interruption length X

|  | NR Slot length (ms) | Interruption length X (slotsnote 1) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: If the BWP switch involves changing of SCS, the interruption due to BWP switch is determined by the larger one between the SCS before BWP swith and the SCS after the BWP switch. |  |  |

----------------------END OF 2nd CHANGES----------------------------

----------------------START OF 3rd CHANGES----------------------------

#### 8.2.2.1	Introduction

This section contains the requirements related to the interruptions on PCell and activated SCell if configured, when up to TBD SCells are configured, deconfigured, activated or deactivated.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [17] for SCell activation/deactivation command. How to specify this is FFS.

This section also contains the requirements related to the interruptions on other active serving cells wherein the UE is performing BWP switching.

----------------------END OF 3rd CHANGES----------------------------

----------------------START OF 4th CHANGES----------------------------

8.2.2.2.5	Interruption due to Active BWP switching Requirement

When BWP switch involves SCS changes, UE is allowed to cause interruption of up to X slot to other active serving cells.

When BWP switch involves parameters locationAndBandwidth, maxNrofCodeWordsScheduledByDCI and nrofSRS-Ports without SCS, the UE is allowed to cause interruption of up to X slot to other active serving cells if the UE is not capable of per-FR gap, or to other active serving cells in the same frequency range wherein the UE is performing BWP switching if UE is capable of per-FR gap.

Interruption length X is defined in Table 8.2.1.2.7-1. The start of the interruption is only allowed within the BWP switching delay defined in clause 8.6.2 in TS38.133.

Interruptions are not allowed during BWP switch involving only change in other parameter not mentioned above.

Table 8.2.2.2.5-1: Interruption length X

|  | NR Slot length (ms) | Interruption length X (slotsnote 1) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: If the BWP switch involves changing of SCS, the interruption due to BWP switch is determined by the larger one between the SCS before BWP swith and the SCS after the BWP switch. |  |  |

----------------------END OF 4th CHANGES----------------------------