3GPP TSG-RAN WG4 Meeting #88bis                 	R4-1813222

Chengdu, China, 8 – 12 October 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PRACH simulation results

Agenda item:	7.13.2.4

Document for:	Discussion

Introduction

In the last meeting, a WF for NR PRACH demodulation was agreed in [1]. In this contribution we present our simulation results based on the simulation assumptions that were agreed earlier.

Simulation results

Simulation assumptions

In the following we will provide simulation results based on the simulation assumptions as agreed earlier for NR PRACH in [2].

Table 2: Simulation assumptions for NR PRACH

| Preamble format | 0 | A2 | B4 | C2 |
| --- | --- | --- | --- | --- |
| Number of Tx | 1 | 1 | 1 | 1 |
| Number of Rx | 2 | 2 | 2 | 2 |
| SCS [KHz] | 1.25 | 15,30,60,120 | 15,30,60,120 | 15,30,60,120 |
| CBW (MHz) | 20, 40, 100 | 15kHz SCS: 10 30kHz SCS: 20, 40, 100 60/120kHz SCS: 100 | 15kHz SCS: 10 30kHz SCS: 20, 40, 100 60/120kHz SCS: 100 | 15kHz SCS: 10 30kHz SCS: 20, 40, 100 60/120kHz SCS: 100 |
| Ncs | 13 | 10 | 15 | 34 |
| Logical sequence index | 22 | 0 | 0 | 0 |
| Carrier Frequency [GHz] | 4 | 4,30 | 4,30 | 4,30 |
| Propagation Channel | AWGN | AWGN | AWGN | AWGN |

Simulation results

Simulation results based on parameters in Table 1 are shown below in Table 2.

Table 3: Initial simulation results for PRACH format 0

| SCS [KHz] | CBW (MHz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- |
| 1.25 | 20 | -16.2 |
| 1.25 | 40 | -16.3 |
| 1.25 | 100 | -16.3 |

Table 4: Initial simulation results for PRACH format A2

| SCS [KHz] | CBW (MHz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- |
| 15 | 10 | -14.7 |
| 30 | 20 | -14.8 |
| 30 | 40 | -15.0 |
| 30 | 100 | -14.8 |
| 60 | 100 | -15.0 |
| 120 | 100 | -14.7 |

Table 5: Initial simulation results for PRACH format B4

| SCS [KHz] | CBW (MHz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- |
| 15 | 10 | -18.6 |
| 30 | 20 | -18.6 |
| 30 | 40 | -18.9 |
| 30 | 100 | -18.7 |
| 60 | 100 | -18.9 |
| 120 | 100 | -18.6 |

Table 6: Initial simulation results for PRACH format C2

| SCS [KHz] | CBW (MHz) | SNR [dB] with Missed detection probability 1 % |
| --- | --- | --- |
| 15 | 10 | -14.5 |
| 30 | 20 | -14.4 |
| 30 | 40 | -14.7 |
| 30 | 100 | -14.5 |
| 60 | 100 | -14.7 |
| 120 | 100 | -14.4 |

Conclusion

In this contribution we have provided simulation results for NR PRACH.

- References

[1] R4-1811728, Way forward on NR PRACH demodulation requirements, Ericsson, 3GPP TSG-RAN WG4 Meeting #88, Gothenburg, Sweden, August 20-August 24, 2018.

[2] R4-1809369, Way forward on NR PRACH demodulation requirements, Ericsson