3GPP TSG-RAN WG4 Meeting #88bis										R4-1813247

Chengdu, China 8 – 12 October 2018

Source: 	Huawei, HiSilicon

Title: 	Discussion on spurious emission band 1 UE co-existence for NB-IoT

Agenda Item:	5.3.1

Document for:		     Approval

# Introduction

The requirements of spurious emission in protected band for NB-IoT are inherited from legacy LTE in Clause 6.6.3.2 of [1]. Compared with legacy LTE, the frequency gap between the NB-IoT edge sub-carrier and the protected band can be much smaller, owing to the much smaller channel bandwidth for NB-IoT (i.e. 200 kHz) compared with LTE. This could make some of the NB-IoT spurious emission requirements much more stringent than the SEM or ACLR requirements for NB-IoT. A particular issue is the 1915-1920 MHz protected band since the high end of this protected band is immediately adjacent to an NB-IoT channel centered on 1920.1 MHz (it should be noted that an NB-IoT channel includes no explicit guard-band, unlike LTE). In this paper, we discuss the details of such a scenario and propose a remedy for the problem.

# Discussion

## The special scenario

Below is the co-existence requirement for band 1, copied from [1].

Table 6.6.3.2-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 65, 67, 68, 69 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27 |

The most demanding case is when the NB-IoT UL channel is deployed at the low range of band 1, i.e. 1920.1 MHz, given that there is a protected frequency range from 1915-1920 MHz (highlighted in yellow above). The frequency arrangement is depicted below.

Figure 1: Protected frequency range immediately below band 1

Protected Frequency Range

1920

1915

1920.1

Freq (MHz)

NB-IoT

In this case, the protected frequency range is right next to the NB-IoT channel and the gap to the first

This is effectively a very tight ACLR requirement. Particularly for single-tone transmission at sub-carrier index 0 or 1, the spurious emission is dominated by the side-lobes of the single-tone NB-IoT spectrum (based on a sinc-shape but with some filtering to meet the SEM while avoiding excess EVM). The effective ACLR requirement is: max TX power – spurious emission limit = 23-1.6 = 21.4 dB.

## Comparison with SEM and ACLR

Consider first the spectrum emission mask (SEM) for NB-IoT, see table below. It can be seen that this takes account of the spectral shape of an NB-IoT signal. The frequency offset ΔfOOB starts from the edges of the NB-IoT channel, which in the worst case of the 1920.1 MHz channel is also the edge of the protected band. It can be seen that the SEM allows very substantially more energy within the protected band than is allowed by the spurious emissions requirement. In other words, the spurious emissions requirement is effectively demanding a much tighter SEM for transmissions on this channel.

Table 6.6.2F.1-1: category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

Another perspective is to consider the NB-IoT ACLR requirement. One of the ALCR requirements for NB-IoT is defined for an adjacent GSM channel that is 300 kHz away from the NB-IoT channel center. Compared with the spurious emissions scenario depicted in Figure 1, there is an extra gap of 100 kHz between the two channels. However, the required ACLR is only 20 dB including the extra 100 kHz gap, which is lower than the effective ACLR requirement (i.e. 21.4 dB) without any gap that is implied by the spurious emissions co-existence requirement for the case of NB-IoT uplink on 1920.1 MHz.

Table 6.6.2F.3-1: category NB1 and NB2 UE ACLR requirements

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | 20 dB | 37 dB |
| Adjacent channel center frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |

It can be seen that a NB-IoT UE which complies with both the SEM and ALCR requirements can substantially fail the co-existence requirement for this particular corner case.

## Potential solution

To address the problem, we propose to follow the existing approach used for legacy LTE which applies some special constraints on the spurious emissions requirements to avoid corner cases. For example, in Note 27 of Table 6.6.3.2-1 of [1], the max transmission bandwidth is reduced to 54 RB; in Note 34, the max transmission bandwidth is reduced to 30 RB with RBstart > 1 and RBstart < 48. Therefore, we propose a similar approach for NB-IoT:

Proposal: Amend Note 27 of Table 6.6.3.2-1 of [1] as below:

NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB; for carriers of category NB1 and NB2 when carrier centre frequency is 1920.1 MHz, the requirement for single-tone uplink transmissions is applicable only for sub-carrier index > 1.

The aim of this proposal is to eliminate the most problematic aspects of the corner case (single-tone UE transmissions on the two lowest frequency sub-carriers when the NB-IoT channel is at 1920.1 MHz and so is immediately adjacent to the 1915-1920 MHz protected band) whilst having no impact on any other NB-IoT scenarios.

# Summary

A corner case has been identified in the spurious emission band 1 UE coexistence requirement for NB-IoT. It occurs when deploying an NB-IoT channel in standalone mode on the lowest NB-IoT channel in band 1, i.e. centered on 1920.1 MHz. Given that this corner case is currently imposing a much more difficult requirement on the UE than the SEM and ACLR requirements, we propose to constrain the requirement as below in order to eliminate the most problematic aspects of the corner case whilst having no impact on any other NB-IoT scenarios:

Proposal: Amend Note 27 of Table 6.6.3.2-1 of [1] as below:

NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB; for carriers of category NB1 and NB2 when carrier centre frequency is 1920.1 MHz, the requirement for single-tone uplink transmissions is applicable only for sub-carrier index > 1.

# References

3GPP TS 36.101 V13.12.0