3GPP TSG RAN WG4 Meeting #88Bis	R4-1813275

Chengdu, China, October 8 – 12, 2018

Agenda Item:	7.5.1

Source:	Huawei, HiSilicon

Title:	SCS restrictions for BWP operations for non-SS block

Document for:	Approval

# Introduction

In RAN4#88, contribution [1] discussed the topic of placing a restriction on SCS based on the configured bandwidth. Several options were captured on-line included

- Restrict the data SCS according to CC BW

- Restrict the BWP location

- Introduce different UE capability

- No further restriction if no UE capability

This contribution evaluates the options listed and proposes no further restriction if no UE capability.

# Discussion

## Background

In RAN4#AH1801, the following was captured [2]

- UE behavior

- The UE meets RF requirements related to its configured BWP according to UE CBW (no additional BWP related RF requirements)

  - FFS whether UE CBW next highest value after BWP bandwidth

- How the UE configures its RF in order to operate the BWP and meet the CBW based RF requirements is implementation dependent

- UE RF requirements

- Single BWP case: single CC RF requirements apply with UE CC BW

- Multiple BWPs case: intra-band CA RF requirements apply with UE CC BW

The WF implies:

- For RF, regardless of the bandwidth of the BWP, the network assumes the UE is operating with at least the CC bandwidth

- For requirements, only the CC bandwidth is considered

## Baseband and single CC

The maximum FFT size of 4096 points limits the CC BW for 15 kHz SCS to 50 MHz (FR1) and for 60 kHz SCS to 200 MHz (FR2).

Based on the listed operating bands, the following bands in Table 1 have their largest CC BW for 30 kHz SCS greater than 50 MHz. For FR2, all bands have CC BW for 120 kHz SCS greater than 200 MHz. In those bands, 60 kHz SCS is supported up to 200 MHz.

Table 1. FR1 TDD bands where 30 kHz SCS > 50 MHz and 15 kHz SCS is supported

| Band | Frequency | BW, MHz | Max BW for 15 kHz SCS | Max BW for 30 kHz SCS |
| --- | --- | --- | --- | --- |
| n40 | 2300 MHz – 2400 MHz | 100 | 50 MHz | 100 MHz |
| n41 | 2496 MHz – 2690 MHz | 194 | 40 MHz | 100 MHz |
| n50 | 1432 MHz – 1517 MHz | 85 | 40 MHz | 80 MHz |
| n77 | 3300 MHz – 4200 MHz | 900 | 40 MHz | 100 MHz |
| n78 | 3300 MHz – 3800 MHz | 500 | 40 MHz | 100 MHz |
| n79 | 4400 MHz – 5000 MHz | 600 | 40 MHz | 100 MHz |

Observation 1. A limited number of FR1 bands have maximum BW for 30 kHz SCS greater than 50 MHz.

Although an argument about more bands supporting larger BWs with 30 kHz in the future is valid argument, many of the bands may not be able to support >50 MHz bandwidths due to regulatory reasons. Based on the study of the bands affected above, a short evaluation of the options available is presented in Table 2.

Table 2. Evaluation

| Option | Evaluation |
| --- | --- |
| Restrict the data SCS according to CC BW | - An unnecessary restriction for network implementation (CA can be used) |
| Restrict the BWP location | - May limit network implementation (CA can be used) |
| Introduce different UE capability | - Network can tailor its operation to UE capabilities - Possible backwards compatibility issue for Rel. 15 UEs - Manufacturers may not want to indicate implementation details |
| No further restriction if no UE capability | - No additional work necessary - Network responsible to ensure its configuration is correct. Options include CA, use of multiple bands |

In general:

- The network can ensure it does not configure 15 kHz SCS when the CC bandwidth is greater than 50 MHz. (no standards specification is needed).

- The network can ensure it does not configure 60 kHz SCS when the CC bandwidth is greater than 200 MHz. (no standards specification is needed)

Proposal 1: No restriction on network behavior is needed for SCS restrictions because network implementation can avoid it.

## Multiple BW signaled for CC

Another topic in [1] was “Companies to investigate the implication of a UE configured with different UE CC BW’s on BWP switching and RF requirements”.

During the configuration of a CC, the network provides the UE the location of the CC with respect to point A and the bandwidth using SCS and number of RBs, as described in [3].

The IE SCS-SpecificCarrier provides parameters determining the location and width of the actual carrier. It is defined specifically for a numerology (subcarrier spacing (SCS)) and in relation (frequency offset) to Point A.

SCS-SpecificCarrier ::=				SEQUENCE {

offsetToCarrier						INTEGER (0..2199),

subcarrierSpacing					SubcarrierSpacing,

carrierBandwidth					INTEGER (1..maxNrofPhysicalResourceBlocks),

...

}

After which, the UE receives a configuration for the BWP within the CC.

BWP ::= 							SEQUENCE {

locationAndBandwidth				INTEGER (0..37949),

subcarrierSpacing					SubcarrierSpacing,

cyclicPrefix						ENUMERATED { extended }													OPTIONAL	-- Need R

}

Since the field locationAndBandwidth is the product of the starting point (0, …, 274) and size, its range of values is NRB(NRB+1)/2. Another note is

The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier (configured in SCS-SpecificCarrier contained within FrequencyInfoDL) corresponding to this subcarrier spacing.

The starting location is referenced to the first subcarrier of the first PRB of the carrier. From the SCS-SpecificCarrier signaling, multiple carriers can overlap in frequency. With overlapping CCs, BWPs that occupy the intersecting resources of the CCs are possible. However, since BWPs are associated with a CC, then the RF requirements for the CC apply to that BWP.

Proposal 2: Since BWPs are associated with the SCS of the CC, no restrictions are needed.

15 kHz numerology

30 kHz numerology

frequency

frequency

Figure 1. Possible CC layouts

One observation is that field locationAndBandwidth is not constrained to the bandwidth of the CC. It is possible that an allocation within the BWP can exceed the CC. This can be error case but is handled by network configuration.

# Conclusion

Observation 1. A limited number of FR1 bands have maximum BW for 30 kHz SCS greater than 50 MHz.

Proposal 1: No restriction on network behavior is needed for SCS restrictions because network implementation can avoid it.

Proposal 2: Since BWPs are associated with the SCS of the CC, no restrictions are needed.

# References

R4-1811543, “WF on data SCS restriction of BWP based on UE CC BW”, Qualcomm, RAN4#88, Aug. 20-24, 2018.

R4-1801006, “WF on wideband operation of NR UE”, RAN4 #AH-1801, Jan. 22 – 26, 2018

38.331