3GPP TSG-RAN WG4 Meeting #88bis	R4-1813493

Chengdu, China, 8th – 12th Oct, 2018

Agenda item:	6.12.2.2

Source: 	Qualcomm Incorporated

Title: 	PDSCH demodulation requirement for 8Rx capable UE

Document for:	Discussion

# Introduction

In the last RAN4 #88 meeting, a way-forward [1] was agreed to capture the progress in the RAN4 discussion for demodulation/SDR/CSI requirement for 8Rx-capable UE. In this contribution, we discuss the PDSCH demodulation requirement for 8Rx capable UE.

# Discussion

RAN4 needs to determine the number of layers to be used for demodulation requirement for 8Rx-UE. In [2], we have discussed the fading demodulation performance of rank-6 and rank-8 transmission for 16QAM ½ rate in the 8x8 EPA5 low correlation channel, as shown in Figure 1 and 2 for completeness. It is shown that 70% of the maximum throughput can be achieved at 13dB and 17.8dB, respectively, and hence both options are feasible from the test point perspective. Considering that 8-Layer demap is more challenging that 6-Layer, our proposal is to define the PDSCH demodulation test based on rank 8.

Observation 1. For rank-6 and rank-8 transmission based on 16QAM ½ rate in 8x8 EPA5 channel with low antenna correlation, 70% of the maximum throughput can be achieved at 13dB and 17.8dB for rank-6 and rank-8 transmission, respectively.

Proposal 1. Introduce a new PDSCH demodulation test for rank > 4 based on 16QAM ½ rate rank-8 in 8x8 EPA5 channel with low antenna correlation.

Figure 3. 16QAM ½ rate Rank-6 in EPA5

Figure 4. 16QAM ½ rate Rank-8 in EPA5

# Conclusions

In this paper, we discussed the open issues in the PDSCH demodulation requirement for 8Rx-capable UE. Observation and proposal made in this paper is summarized as follows.

Observation 1. For rank-6 and rank-8 transmission based on 16QAM ½ rate in 8x8 EPA5 channel with low antenna correlation, 70% of the maximum throughput can be achieved at 13dB and 17.8dB for rank-6 and rank-8 transmission, respectively.

Proposal 1. Introduce a new PDSCH demodulation test for rank > 4 based on 16QAM ½ rate rank-8 in 8x8 EPA5 channel with low antenna correlation.

# References

[1] R4-1811868

[2] R4-1810536