3GPP TSG-RAN4 Meeting #88bis	R4-1813551

# Chengdu, China, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 37.843 | CR | CRNum | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TR 37.843: Annex on near-field TRP measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Guidance on near-field measurement is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new Annex to the TR containing description of near-field measurements including selection of test distance and measurement antenna size. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, there is a risk of inaccurate TRP values caused by erroneous near-field measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------ Beginning of changes ------

# Annex X. Near-field Measurement for TRP

At high frequencies the far-field distance of an emission pattern can be tens or hundreds of meters. As an example, consider a base station antenna of diameter 60 cm and the corresponding emissions at 60 GHz, i.e., a wavelength of approximately 5 mm. The resulting far-field distance is then

2D2/λ≈2⋅6002/5 mm=144 m

This can turn out to be a too large test distance in many test ranges. However, the TRP does not need to be defined via EIRP, which is only defined in the far-field region. Instead, it can equally well be defined via radial power flux density. For time-harmonic field the radial power flux density is

Sr=r⋅ReE×H*

With unit Watt per square meter. This quantity gives the power per unit area that flows out through a spherical surface of a given radius, which in this case is the test distance. The total radiated power is the sum of all these contributions, i.e.

TRP=fsSrdA=fsSrd2sinθdθdϕ

Where fs denotes a full sphere integration. Note that the relation between radial power density flux and EIRP is

EIRP=Sr4πd2

which gives the relation between the TRP expressed as a full sphere integration of EIRP

TRP=14πfsEIRPθ,ϕsinθdθdϕ

By splitting the fields in tangential and radial parts as

E=Et+ErrH=Ht+Hrr

it is found that

Sr=r⋅ReEt×Ht*

In the far-field limit the radial power density flux simplifies to

Sr=Et2/Z0

Here Z0≈377Ω is the free space impedance. From numerical simulations, based on full sphere measurements it has been found that this is a good approximation also in the near-field region provided that the test distance is larger than the radius of the EUT by a few wavelengths (around 2), see Fig 1 and [31]. Hence, if this condition is met the tangential E-field will give the necessary data to calculate the TRP, provided it is accurately measured.

To ensure the latter:

1. The test distance shall be at least the far-field distance of the measurement antenna.

2. The measurement antenna shall be small enough to sample an approximately constant field.

3. The measurement sphere must have a distance of some wavelengths from the EUT.

Figure 1: Relative TRP error in dBs when far-field approximate of power density is used. The red triangles correspond to the test distances used in Figs 2 (a) and (b), respectively. The test distance is r0 and the radius of the EUT is Rsph. Hence the horizontal axis represents the distance between the closest measurement point and the EUT measured in wavelengths.

Figure 2: Angular power density patterns in the near-field of a base station antenna [31]. The patterns are sampled at 0.75 m (a), 2.5 m (b) and 30 m (c) and the far-field distance is 30m. The angular patterns vary with distance but the full-sphere integration for TRP deviates less than 0.065 dB. Note, the lower power density levels in the lower figure are balanced by the larger total area of the measurement sphere.

The first criterion has the effect of minimizing errors related to radial field components and is quantified as

d≥2DMA2/λ

The second criterion implies that the main beam of the measurement antenna illuminates the EUT with a sufficient margin [31] and is quantified as

d≥wλ/2 R+λ

This property is to be considered at any antenna measurement at finite distance.

Furthermore, equation above sets a minimum test distance for a given probe size w at which the maximum SNR of the measurement setup is reached, see [31].

The first and second criteria for test distance have to be considered also for finite test distance mesurement of any far-field parameter such as EIRP.

The third criterion should have very small consequences at millimeter wave frequencies.

------ End of changes ------