3GPP TSG-RAN WG4 Meeting #88-Bis                 	R4-1813656

Chengdu, China, Oct 8th – 12th 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on NR UE Transmit Timing Accuracy Tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-09-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Adding test for NR UE Transmit timing for FR2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding tests for NR UE Transmit timing for FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 transmit timing tests will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section A.7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

------------START of CHANGES-------

A.7.1.2 NR UE Transmit Timing Accuracy tests

A.7.1.2.1 Test Purpose and parameters

The purpose of this test is to verify that the UE can follow frame timing change of the connected

gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment,

minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in

clause 7.1.2.

A.7.1.2.2 Test Applicability

Test 1 and 2 are for FR1 and Test 3 and 4 are for FR2. The SCS of the SSB and UL SCS is chosen to be the largest supported by UE capability in each FR. The DL SCS is chosen to be the same as UL SCS.

A.7.1.2.3 Test environment and description

For this test a single cell is used. In addition, for FR2, a single beam is used. Table A.7.1.2.1-1 defines the parameters to be configured and strength of the transmitted signals.

condition. The transmit timing is verified by the UE transmitting SRS using the configuration defined in Table

A.7.1.2.1-3.

Table A.7.1.2.1-1: General Test parameters for NR Timing Accuracy Tests

| Parameter | Unit | Test1 | Test2 |
| --- | --- | --- | --- |
| RF Channel Number |  | 2 | 2 |
| PDSCH Reference Measurement Channel |  | SR.3.1 TDD | SR.3.1 TDD |
| PDCCH Reference Measurement Channel |  | CR.3.1 TDD | CR.3.1 TDD |
| OCNG Pattern |  | OCNG Pattern 1 | OCNG Pattern 1 |
| DL/UL BW | MHz | 100 MHz | 100 MHz |
| Number of BWP’s |  | 1 | 1 |
| BWP BW |  | 100 MHz | 100 MHz |
| SSB periodicity | ms | 20 | 20 |
| DRX | ms | None | 320 |
| Noc (per RE) | dbm/RE | TBD | TBD |
| Es/Iot | db | TBD | TBD |
| Es/Noc | db | TBD | TBD |
| Propagation Condition |  | AWGN/Static Channel | AWGN/Static Channel |
| SRS Congfig |  | Config1 | Config2 |
| EPRE ratio of PSS to SSS | db | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |

Table A.7.1.2.1-2: SRS Configuration for Timing Accuracy Test

|  |  | Config1 | Config 2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
|  | SRS-ResourceSetId | 0 | 0 |  |
| SRS-Resource | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | sl1 | sl1 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1 | sl640 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | Any 10 bit number |

A.7.1.2.3 Test requirements

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

- For EN-DC mode, set up E-UTRA PCell according to parameters given in Table A.3.7.2.1-1 and setup NR PSCell according to parameters given in Table A.7.1.2.1-1. For NSA mode tests, setup NR Pcell according to parameters given in Table A.7.1.2.1-1.

- After connection set up with the cell, the test equipment will verify that the timing of the NR cell is within (NTA + NTA_offset) ± Te of the first detected path of DL SSB.

  - The NTA offset value (in Tc units) is 25600 for FR1 and 13792 for FR2

  - The Te values depend on the DL and UL SCS for which the test is being run and are given in Table A.7.1.2.1-3

Table A.7.1.2.1-3: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | [12]*64*Tc |
|  |  | 30 | [10]*64*Tc |
|  |  | 60 | [10]*64*Tc |
|  | 30 | 15 | [8]*64*Tc |
|  |  | 30 | [8]*64*Tc |
|  |  | 60 | [7]*64*Tc |
| 2 | 120 | 60 | [3.5]*64*Tc |
|  |  | 120 | [3.5]*64*Tc |
|  | 240 | 60 | [3]*64*Tc |
|  |  | 120 | [3]*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 Editor’s note: The final values of Te for 120KHz SSB SCS are subject to further discussions in further meeting, and may not be outside 3*64*Tc to 3.5*64*Tc. |  |  |  |

- The test system shall adjust the timing of the DL path by values given in Table A.7.1.2.1-4

Table A.7.1.2.1-4 Adjustment Value for DL Timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1/Test3 | Test2/Test4 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |
| 120 | +16*64Tc | +8*64Tc |
| 240 | +8*64Tc | +4*64Tc |

- The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Section 7.1.2 Table 7.1.2-3. This will only be done for Test1 and Test 3.

- The test system shall verify that the UE transmit timing offset stays within (NTA + NTA_offset) ± Te of the first detected path of DL SSB. For Test 2 and Test 4 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment

--------------------END of CHANGES------------