3GPP TSG-RAN WG4 Meeting #88bis	R4-1813686

Chengdu, China, 8 – 12 October 2018

Agenda Item:	6.6.4.1

Source:	Ericsson

Title:	Simulation results of PDSCH demodulation for 1024QAM

Document for:	Discussion

# 1	Introduction

RAN4#87 agreed with the evaluation parameters for PDSCH demodulation with 1024QAM as follow [1].

| No. | TM | PMI feedback (pre-coding) | MIMO layer | MCS | Antenna config. |
| --- | --- | --- | --- | --- | --- |
| 1 | TM4 | Wideband PMI Feedback (PUSCH 3-1, Reporting interval =1) | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 2 | TM4 | Wideband PMI Feedback (PUSCH 3-1, Reporting interval =1) | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |
| 3 | TM9 | Per-PRG Random Precoder | 1-layer | MCS#23 (with 52752 bits) | 4x2 Low |
| 4 | TM9 | Wideband PMI Feedback | 2-layer | MCS#23 (with 52752 bits) | 4x4 Low |

- Test cases for demodulation fading tests for 1024QAM

  - Tx EVM: 2%

- FFS for applicability rule

In this contribution we provide the simulation results.

# 2	Simulation results

## 2.1	TM4

Figure 1 shows the simulation results of PDSCH TM4 (corresponding to tests 1 and 2). Table 1 summarizes the required SNR [dB] to achieve 70% of the maximum throughput.

Figure 1	Simulation results of PDSCH TM4 EPA5 with 1024QAM.

Table 1	Required SNR [dB] to achieve 70% of maximum throughput for TM4.

|  | SNR of 70% of maximum throughput |
| --- | --- |
| Test 1 | 22.4 dB |
| Test 2 | 25.2 dB |

## 2.2	TM9

Figure 2 shows the ideal simulation results of PDSCH TM9 (corresponding to tests 3 and 4). Table 2 summarizes the required SNR [dB] to achieve 70% of the maximum throughput.

Figure 2	Simulation results of PDSCH TM9 EPA5 with 1024QAM.

Table 2	Required SNR [dB] to achieve 70% of maximum throughput for TM9.

|  | SNR of 70% of maximum throughput |
| --- | --- |
| 4x2 rank 1 follow PMI | 23.3 dB |
| 4x2 rank 1 random PMI (Test 3) | 27.4 dB |
| 4x4 rank 2 follow PMI (Test 4) | 26.6 dB |
| 4x4 rank 2 random PMI | 28.9 dB |

# References

- R4-1811378, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon.