3GPP TSG-RAN WG4#88Bis Meeting	R4-1813695

Chengdu, China, 8 – 12 October 2018

Agenda item:	6.12.2.4

Title: 	Discussion on 8Rx UE CSI performance requirement tests

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN Plenary Meeting #78, the Rel.15 WI “UE requirements for LTE DL 8Rx antenna ports” has been approved and its WID is included in [1]. In the scope of the WID [1], there is part related to RAN4 UE demodulation/CSI performance tests as shown below:

- For UE demodulation/CSI requirements

  - Define channel model for downlink 8Rx antennas

    - Specify the antenna configuration and MIMO channel correlation matrices for 8Rx antennas;

    - Specify the static channel model;

  - Define test cases for the rank lower than or equal to 4.

    - Adopt the test cases studied in SI to define requirements.

  - Define test cases for the rank higher than 4 in fading channel.

    - Base on combinations of rank and MCS that can achieve the maximum configured throughput.

  - Define SDR test for 8Rx in WI.

  - Define CSI test for 8Rx.

  - No PDCCH/PCFICH demodulation requirement is expected for 8Rx.

- Considering the test coverage of 8Rx, test applicability rule is needed to define

  - Define applicability rule of existing performance requirements for 8Rx capable UEs.

In RAN4#88 Meeting, a WF [2] was agreed on the demodulation and CSI tests for 8Rx UE. For CQI and RI tests, it has

- CQI reporting definition under AWGN

  - Option 1: TM9 rank 8

  - Option 2: TM9 rank 4

- Do not define RI test for 8Rx UEs

In this contribution, we provide our CQI reporting simulation results and views on CQI tests for 8Rx UEs.

# Simulation results and discussion

As shown in Figure 1, it is observed that, due to the large overhead in TM9 Rank8 transmission, high MCS levels with very large code rate may not be feasible even though the large CQI index is reported.

Figure 1 SDR simulation results for 64QAM, rank=8, MCS=26/27/28

Therefore, we propose that:

Proposal 1: Define CQI reporting definition under AWGN conditions with TM9 rank4.

Table 1 below lists our initial results of Median CQI reporting vs. SNR for a TM9 rank4 scenario.

Table 1 Median CQI vs. SNR

| SNR | Median CQI | Modulation Order Qm | Probability within +/- Median CQI | BLER Median CQI - 1 | BLER Median CQI | BLER Median CQI + 1 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 5 | 2 | 1 |  | 0 | 0.6438 |
| 7 | 9 | 4 | 1 |  | 0 | 1 |
| 13 | 12 | 6 | 1 |  | 0 | 1 |
| 15 | 13 | 6 | 1 | 0 | 0.025 | 1 |

# Conclusion

In this contribution, we provide our CQI reporting simulation results and views on CQI tests for 8Rx UEs. It is observed that, due to the large overhead in TM9 Rank8 transmission, high MCS levels with very large code rate may not be feasible even though the large CQI index is reported. Therefore, we propose that:

Proposal 1: Define CQI reporting definition under AWGN conditions with TM9 rank4.

# Reference

[1] R4-172842, “New WI proposal: UE requirements for LTE DL 8Rx antenna ports”, Huawei, HiSilicon, Dec. 2017

[2] R4-1811868, “Way forward on 8Rx demodulation and CSI tests”, Huawei, August 2018