3GPP TSG-RAN WG4 Meeting #88bis	R4-1813705

Chengdu, China, 8th – 12th Oct, 2018

Agenda item:	6.6.4.1

Source: 	Qualcomm Incorporated

Title: 	Simulation assumption for 1024QAM demodulation requirement

Document for:	Approval

# Introduction

This contribution captures the details of the simulation assumption as agreed in the way-forward from RAN4 #88 meeting [1].

# Simulation Assumptions

## FDD

### CRS-Based

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 | Antenna Ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 4 | 4 |
| PDSCH rank |  |  | 1 | 2 |
| Precoding granularity |  | PRB | 50 | 50 |
| PMI delay |  | ms | 8 | 8 |
| Reporting interval |  | ms | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0x000000000000FFFF | 0x00000000FFFF0000 |
| Note 1: . |  |  |  |  |

| Test number | Bandwidth amd MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 1024QAM | R.1024-1.FDD | OP.1 FDD | EPA5 | 4x2 Low | 70 | TBD | TBD |
| 2 | 10MHz 1024QAM | R.1024-2 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | TBD | TBD |
| For a 4Rx-capable UE, Test 2 will be run and Test 1 will be skipped. Otherwise, Test 2 is not applicable and only Test 1 will be run. |  |  |  |  |  |  |  |  |

### DMRS-Based

| Parameter |  | Unit | Test 3 | Test4 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming | 2 layer precoding based on WB PMI feedback |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | 5 / 2 |
| CSI reference signal configuration |  |  | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Simultaneous transmission |  |  | No | No |
| PDSCH transmission mode |  |  | 9 | 9 |
| PDSCH rank |  |  | 1 | 2 |
| Precoding granularity |  |  | 3 | 50 |
| PMI delay |  |  | 8 | 8 |
| Reporting interval |  |  | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False | False |
| CodeBookSubsetRestriction bitmap |  |  | 0x000000000000FFFF | 0x00000000FFFF0000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 1,2,3,4,6,7,8,9 Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

| Test number | Bandwidth amd MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 3 | 10 MHz 1024QAM | R.1024-3.FDD | OP.1 FDD | EPA5 | 4x2 Low | 70 | TBD | TBD |
| 4 | 10MHz 1024QAM | R.1024-4 FDD | OP.1 FDD | EPA5 | 4x4 Low | 70 | TBD | TBD |
| For a 4Rx-capable UE, Test 4 will be run and Test 3 will be skipped. Otherwise, Test 2 is not applicable and only Test 1 will be run. |  |  |  |  |  |  |  |  |

## TDD

### CRS-Based

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 (Note 1) | -6 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| ULDL config |  |  | 1 | 1 |
| Special subframe |  |  | 4 | 4 |
| Cell-specific reference signals |  |  | Antenna Ports 0,1,2,3 | Antenna Ports 0,1,2,3 |
| PDSCH transmission mode |  |  | 4 | 4 |
| PDSCH rank |  |  | 1 | 2 |
| Precoding granularity |  | PRB | 50 | 50 |
| PMI delay |  | ms | 8 | 8 |
| Reporting interval |  | ms | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| CodeBookSubsetRestriction bitmap |  |  | 0x000000000000FFFF | 0x00000000FFFF0000 |
| Note 1: . |  |  |  |  |

| Test number | Bandwidth amd MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz 1024QAM | R.1024-1.TDD | OP.1 TDD | EPA5 | 4x2 Low | 70 | TBD | TBD |
| 2 | 10MHz 1024QAM | R.1024-2 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | TBD | TBD |
| For a 4Rx-capable UE, Test 2 will be run and Test 1 will be skipped. Otherwise, Test 2 is not applicable and only Test 1 will be run. |  |  |  |  |  |  |  |  |

### DMRS-Based

| Parameter |  | Unit | Test 3 | Test4 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 (Note 1) |
|  |  | dB | -3 | -3 |
| Beamforming model |  |  | Random beamforming | 2-layer precoding based on WB PMI feedback |
| ULDL config |  |  | 1 | 1 |
| Special subframe |  |  | 4 | 4 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 | Antenna ports 15,…,18 |
| Beamforming model |  |  | Annex B.4.1 | Annex B.4.3 |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 4 | 5 / 4 |
| CSI reference signal configuration |  |  | 3 | 3 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 3) | OCNG (Note 3) |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | 50 |
| Simultaneous transmission |  |  | No | No |
| PDSCH transmission mode |  |  | 9 | 9 |
| PDSCH rank |  |  | 1 | 2 |
| Precoding granularity |  |  | 3 | 50 |
| PMI delay |  |  | 8 | 8 |
| Reporting interval |  |  | 1 | 1 |
| Reporting mode |  |  | PUSCH 3-1 | PUSCH 3-1 |
| alternativeCodeBookEnabledFor4TX-r12 |  |  | False | False |
| CodeBookSubsetRestriction bitmap |  |  | 0x000000000000FFFF | 0x00000000FFFF0000 |
| Note 1: . Note 2: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0, 1, 6. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

| Test number | Bandwidth amd MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 3 | 10 MHz 1024QAM | R.1024-3.TDD | OP.1 TDD | EPA5 | 4x2 Low | 70 | TBD | TBD |
| 4 | 10MHz 1024QAM | R.1024-4 TDD | OP.1 TDD | EPA5 | 4x4 Low | 70 | TBD | TBD |
| For a 4Rx-capable UE, Test 4 will be run and Test 3 will be skipped. Otherwise, Test 2 is not applicable and only Test 1 will be run. |  |  |  |  |  |  |  |  |

# FRC Table

## FDD

| Pararameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.1024-1.FDD | R.1024-2.FDD | R.1024-3.FDD | R.1024-4.FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 | 50 | 50 |
| Allocated subframes per Radio Frame |  | 9 | 9 | 9 | 9 |
| Modulation |  | 1024QAM | 1024QAM | 1024QAM | 1024QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Bits | 52752 | 52752 | 52752 | 52752 |
| For Sub-Frames 2,3,7,8 | Bits | 52752 | 52752 | 52752 | 52752 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | N/A | N/A | N/A |
| Number of Code Blocks (Note 3) |  |  |  |  |  |
| For Sub-Frames 1,4,6,9 | Code blocks | 9 | 9 | 9 | 9 |
| For Sub-Frames 2,3,7,8 | Code blocks | 9 | 9 | 9 | 9 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | N/A | N/A | N/A |
| Binary Channel Bits |  |  |  |  |  |
| For Sub-Frames 1,6 | Bits | 68000 | 68000 | 66000 | 66000 |
| For Sub-Frames 4,9 | Bits | 68000 | 68000 | 66000 | 66000 |
| For Sub-Frames 2,7 | Bits | 68000 | 68000 | 64000 | 64000 |
| For Sub-Frames 3,8 | Bits | 68000 | 68000 | 66000 | 66000 |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | N/A | N/A | N/A | N/A |
| Max. Throughput averaged over 1 frame (Note 4) | Mbps | 42.2016 | 42.2016 | 42.2016 | 42.2016 |
| UE Category |  | TBD | TBD | TBD | TBD |
| UE DL Category |  | 20, ≥ 22 | 20, ≥ 22 | 20, ≥ 22 | 20, ≥ 22 |
| Note 1: 1 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 4: Per transport block |  |  |  |  |  |

## TDD

| Pararameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.1024-1.TDD | R.1024-2.TDD | R.1024-3.TDD | R.1024-4.TDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 | 50 (Note 3) | 50 (Note 3) |
| Allocated subframes per Radio Frame (D+S) |  | (3+2) | (3+2) | (3+2) | (3+2) |
| Modulation |  | 1024QAM | 1024QAM | 1024QAM | 1024QAM |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 | 3/4 |
| Information Bit Payload |  |  |  |  |  |
| For Sub-Frames 1,6 | Bits | 39232 | 39232 | 31704 | 31704 |
| For Sub-Frames 4,9 | Bits | 52752 | 52752 | 52752 | 52752 |
| For Sub-Frames 2,3,7,8 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 52752 | 52752 | 43816 | 43816 |
| Number of Code Blocks (Note 4) |  |  |  |  |  |
| For Sub-Frames 1,6 | Code blocks | 7 | 7 | 6 | 6 |
| For Sub-Frames 4,9 | Code blocks | 9 | 9 | 9 | 9 |
| For Sub-Frames 2,3,7,8 | Code blocks | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 9 | 9 | 7 | 7 |
| Binary Channel Bits |  |  |  |  |  |
| For Sub-Frames 1,6 | Bits | 55280 | 55280 | 44280 | 44280 |
| For Sub-Frames 4,9 | Bits | 68000 | 68000 | 64000 | 64000 |
| For Sub-Frames 2,7,3,8 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 5 | Bits | N/A | N/A | N/A | N/A |
| For Sub-Frame 0 | Bits | 65600 | 65600 | 54120 | 54120 |
| Max. Throughput averaged over 1 frame (Note 5) | Mbps | 23.6720 | 23.6720 | 21.2728 | 21.2728 |
| UE Category |  | TBD | TBD | TBD | TBD |
| UE DL Category |  | 20, ≥ 22 | 20, ≥ 22 | 20, ≥ 22 | 20, ≥ 22 |
| Note 1: 1 symbols allocated to PDCCH. Note 2: Reference signal, synchronization signals and PBCH allocated as per TS 36.211 [4]. Note 3: 50 resource blocks are allocated in sub-frames 4, 9 and 41 resource blocks (RB0–RB20 and RB30–RB49) are allocated in sub-frame 0, 1, 6. Note 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 5: Per transport block |  |  |  |  |  |

# Conclusions

In this paper, we summarized the simulation assumption for FDD/TDD 1024QAM demodulation requirement.

# References

[1] R4-1811378