3GPP TSG-RAN WG4 Meeting RAN4#88bis                                                                                             R4-1813728

Chengdu ,China, 8 -12 Oct, 2018

Agenda Item:   7.13.1.1

Source:        Samsung

Title:          Ad-hoc agenda for NR UE demod

Document for:  Approval

Ad-hoc attendees: [Samsung, Intel, Huawei, Qualcomm, MTK,  Ericsson, CATT, ZTE, NTT DoCoMo, CMCC, China Telecom, AT&T, ANRITSU,R&S, Keysight]

# General

List of contributions

| R4-1812083 | Noc level, Band groups and SNR for FR2 demodulation in 38.101-4 | ANRITSU LTD |
| --- | --- | --- |
| R4-1812202 | NR UE Demodulation and CSI reporting requirements scenarios and common parameters | Intel Corporation |
| R4-1812203 | Views on FR2 SNR and Noc setup for NR UE Demodulation and CSI reporting requirements | Intel Corporation |
| R4-1813606 | TDD configuration for UE demodulation requirements | NTT DOCOMO, INC. |

## FR2 SNR and Noc set-up

Proposals from companies

| Companies | Proposals |
| --- | --- |
| Anristu | Proposal 1: Noc values for FR2 demodulation/CSI take both UE power class and band into account Proposal 2: Use the same FR2 Band grouping as in TS 38.133 Proposal 3: Use Noc values based on Refsens: Power Class 3 in n260, Noc = -155dBm/Hz Proposal 4: Express the TS 38.101-4 SNR requirement at the Reference point as [x]dB +∆BB, where ∆BB is captured in the general section and is associated with the chosen Noc level |
| Intel | Proposal #1: Use SNRBB to define the FR2 minimum UE Demodulation performance requirements in TS 38.101-4. Add definition of SNRRP and SNRBB, and mapping between the SNRRP and SNRBB in TS 38.101-4. Proposal #2: Define all FR2 normal demodulation requirements for 50MHz CBW to increase the testable SNR range. Proposal #3: Further discuss how to account the SNR range constraints for FR2 testing in the SDR requirements Option 1: Do not test the SDR requirements unless the TE can achieve sufficiently high baseband SNR level of [X] dB, where X will be captured directly in TS 38.101-4. Option 2: Reduce the MCS tested for FR2 SDR requirements to ensure that operating SNR is low enough. |  |

Previous RAN4 agreement in Test ability SI:

WF R4-1809505

WF R4-1811892

### Issue 1: specification sections for SNR definition and Reference test points

- Option 1: in general section 4 covering definition for conducted requirements and radiated requirements

  - Option 1a Section 4.4 with two sub-clauses for conducted and radiated (Anristu)

  - Option 2a: section 4.4 for conducted and  4.5 for radiated (E///)

R&S: How to capture radiated only for FR2

Intel: can be added into applicable sections

### Issue 2: SNR definition in requirements

- Option 1: Using SNR at reference/test point for performance requirements  with equation : [x]dB +∆BB, ∆BB captured in general section (Anristu)

- Option 2: Using SNRBB (without considering noise floor impact due to Noc limitation) for performance requirements, Add definition of SNRRP and SNRBB, and mapping between the SNRRP and SNRBB in TS 38.101-4. (Intel)

Intel: Usually, for deriving performance requirements, only baseband simulations did. Also considering potential improvement on TE to have different offset values

Ericsson: Prefer option 2, all requirements are decided based on baseband. For mapping between SNR_BB and SNR_RP can be captured in annex.

QC: Prefer option 2

Anrsitu: Where specify SNR_RP and Noc level?

Intel: capture in general section or annex since its maybe depending on UE power class

Agreement: Using SNRBB (without considering noise floor impact due to Noc limitation) for performance requirements, add definition of SNRRP and SNRBB, and mapping between the SNRRP and SNRBB in TS 38.101-4.

### Issue 3: Noc configurations

- Option 1: Only capture the principles / equations for Noc referred to FR2 REFSENS requirements (Intel)

- Option 2: explicitly capture in 38.101-4 with band grouping basis aligned with RRM 38.133? (Anristu)

Can be included in Revised TP from Anristu pending on offline discussion status

### Issue 4: Testable SNR range limitation impact on FR2 performance requirements

- Option 1: No limitation for FR2 performance requirements, meanwhile if the SNR is not testable in current stage then no need to test them (previous agreements?)

- Option 2: Design requirements to ensure SNR testable i.e. reduce CHBW from 100MHz to 50MHz and for SDR choosing proper MCS levels (Intel)

Agreements: Taking option 1 while for some certain test cases if SNR requirements are not testable in Rel-15, we will further consider choosing proper parameters to ensure SNR requirement testable

## Requirements applicable rules

| Companies | Proposals |
| --- | --- |
| Intel | Proposal #1: Use the following framework for SA/NSA requirements - SA/NSA Normal demodulation / CSI reporting performance requirements - Use noise-free LTE link for NSA mode for FR2. - Test case applicability for UEs supporting SA/NSA operation is FFS. - FFS if all SA requirements shall be tested for NSA case. - SA/NSA SDR performance requirements - Introduce both LTE and NR requirements for NSA (EN-DC) case - FFS: Impact of no support of simultaneous Tx/Rx and single UL transmission on the EN-DC requirements and test setups. Proposal #2: Prioritize the following requirements in Rel-15 - NR single carrier normal demodulation performance requirements - NSA EN-DC normal demodulation performance requirements with single NR carrier - SDR requirements for single carrier, CA and EN-DC NR CA/DC normal demodulation requirements are deprioritized in Rel-15 FFS whether to introduce SDR requirements for FR1 + FR2 NR CA and FR1 + FR2 EN-DC scenarios. If requirements are introduced, do not perform conformance tests. Proposal #3: Use default SCS/CBW set to define the base UE performance requirements. Define 1 test per each identified CBW/SCS combination to ensure QPSK 1/3 PDSCH performance under fading environment. Proposal #4: Do not define 4RX PBCH requirements. The 4RX test cases are defined under condition where 4RX provides substantial performance gains over 2RX - Focus on 4x4 low antenna correlation scenarios - Do not consider 4x4 high correlation scenarios UE which passed 4RX tests shall not be required to pass the 2RX tests with similar test purpose. Proposal #5: Use the following option for RF impairments models - Total TX EVM = 6% for QPSK/16QAM/64QAM. - TX EVM is modelled as AWGN - No Tx phase noise is modelled - Phase noise is explicitly modelled for Rx. Rx Phase noise is modelled for requirements definition Proposal #6: For Rel-15 requirements the maximum number of emulated faders for FR1 is [64] and FFS for FR2. FFS if additional constraints on test setup are needed to limit test systems complexity. Proposal #7: Explicitly specify the WI code in the title of the requirements section in the TS 38.101-4. Proposal #8: Introduce dedicated Requirements Applicability sections for as a part of General sections 5 to provide information on the mapping between the set of supported UE features and associated performance requirements. |
| SAMSUNG | Proposal1: In Rel-15, for NSA EN-DC operation firstly focused on NR carrier only requirements (no LTE carrier requirements). Meanwhile in later cases, we can consider introduce specific requirements for EN-DC operation for both LTE carrier and NR carrier. Proposal 2: For NR only requirements, the LTE configuration defined in TS38.133 A3.7.2 can be considered to be reused. Observation 1: Additional test case list can be summarized as below: - CHBW: - FR1 15kHz: 20MHz - FR2 30kHz: 40MHz, FFS for 100MHz - FR2 120kHz: 50MHz, 200MHz - FR2 60kHz: 50MHz - HST: - In last RAN plenary, it’s agreed HST performance requirements will be further discussed in TEI15. - Introducing ZP CSI-RS configurations in part of PDSCH demodulation test case(s) - Introducing additional PDSCH test cases with following PMI under FR2 120kHz with DDDSU DL-UL configurations Proposal 3: Focused to finalize requirements on agreed test case list in R4-1811723 at least before Dec 2018, remaining additional test cases can be introduced under TEI-15. |

### SA/NSA requirements

Previous agreements

- For single carrier SA/NSA Normal demodulation / CSI reporting performance requirements

  - Reuse the test case parameters for NSA/SA requirements

  - Define same minimum performance requirements for NSA/SA modes

  - For NSA requirements define NR requirements only (i.e. no LTE requirements).

  - Use noise-free LTE link for NSA mode (TBC for FR2)

Additional specific EN-DC requirements can be specified after normal requirements finalized.

Issue 1: SA/NSA requirements applicable rules (normal test cases)

- Option 1: only verified NR carrier requirements, additional EN-DC specific requirements can be further discussed after normal requirements finalized.

Issue 2: NSA SDR requirements

- Both LTE carrier and NR carrier need to be verified for EN-DC operation within FR1 for NSA SDR requirements

R&S: only for LTE+FR1 NR carriers? For LTE+FR2, not testable for LTE carrier and NR carrier at the same time

Intel: FR1+FR2 band combinations requirements for CA or EN-DC operation not testable in Rel-15

For interworking operation across FR1+FR2, only FR2 carrier requirements can be testable considering test feasibility under Rel-15.

Further check agreements in RF agreements (LS 1803261)

E///: Do we still need to introduce requirements in Rel-15 for this operation? We want to follow same approach in RF part.

Huawei: We prefer to focuse on other test cases before Dec2018.

Intel: OK to introduce test cases, but not testable.

Issue 3: Test case applicability for UEs supporting SA/NSA operation

Intel: For UE supporting both SA/NSA, how to applied test cases for such type of UEs?

FFS for test applicability of for the same test cases for UE which supporting both SA/NSA operation

Issue 4: LTE carrier configurations for normal NSA test cases without LTE requirements

- Option 1: Reusing the LTE configuration defined in TS38.133 A3.7.2 can be considered to be reused.

- Option 2: From Huawei TP (R4-1813635)

Huawei: LTE configurations are specific design for RRM, maybe not suitable for demodulation

R&S: How many carriers in LTE link needed for this ? We prefer only 1 carrier to provide functionality link ..

LTE carrier configurations for normal NSA test cases without LTE requirements:

1 LTE cell configured enough for this functionality link, using TDD DL-UL configuration 2 align between LTE and NR

Further offline to check LTE carrier configurations between option 1 and option 2, Huawei leading offline discussion

E///: For TDD configuration should be fixed as configuration 2 to align with NR

Intel: need to align with LTE and NR. Some of test cases TDD configurations which not aligned with LTE, may only can be test under SA.

### CA/DC requirements

Issue 1: Handling NR CA/DC requirements

- Option1: Deprioritize NR CA, NR-NR DC requirements in Rel-15

NTT DoCoMO: Prefer introducing NR CA requirements for both FR1 and Fr2 in rel-15

Intel: For normal test cases or SDR test cases? For normal demodulation test cases, we saw the difficulty considering Rel-15 timeline.

QC: Prefer option 1 for normal demodulation test cases.

- Deprioritize NR CA (normal test cases), NR-NR DC requirements before Dec 2018

### Requirements Applicability in specification

Proposal from Intel:

- Explicitly specify the WI code in the title of the requirements section in the TS 38.101-4.

QC: How to manage if sections in specification considering accumated WIs will be introduced in future.

Some of WI may be introduce only limited test cases.

Intel: In LTE, for introducing new requirements can be introduced in new sections. Taking experience in LTE, without that WI code, may hard to track.

Huawei: we have concern on 1, in LTE we have experience, in TEI we also introduce new test cases , how to place in sections?

Ericsson: Support to have WI code, in RAN5 already have similar approach. TEI, we can discuss handling.

QC: In Rel-16, i.e. 256QAM requirements in FR2 then how we can work

Intel: supposed to have separate WI.

Ericsson: Open for sub-clause level

QC: Introduce WI code in lowest levels. Either in sub-clauses or in contents or in titles

WI code shall be specified for each test cases, FFS for how to capture into specification

- Introduce dedicated Requirements Applicability sections for as a part of General sections 5 to provide information on the mapping between the set of supported UE features and associated performance requirements.

# PDSCH

| R4-1812163 | NR PDSCH UE demodulation requirements | Intel Corporation | Proposal #1: Do not define requirements with 70% test point and high rank and high modulation for scenarios with 16 HARQ processes for TDD and 8 HARQ processes for FDD. Proposal #2: Configure 4 PRB bundling for the following tests: - FDD, MCS 4, 1 Layer, Number of PDSCH PRBs 52 - TDD, MCS 4, 1 Layer, Number of PDSCH PRBs 106 Proposal #3: Configure WB bundling for the following tests: - FDD, MCS 4, 1 Layer, Number of PDSCH PRBs 6 - TDD, MCS 4, 1 Layer, Number of PDSCH PRBs 6 Proposal #4: Change MCS index for QPSK FR1 4Rx requirements from MCS4 to MCS8. Proposal #5: Define PDSCH Type B performance requirements under following conditions: - Start symbol (S) 5 - Duration (L) 7 - MCS 4 for 2Rx tests and MCS 8 for 4 Rx tests - 1 additional DMRS Proposal #6: Use the following test configuration for LTE-NR coexistence requirements definition to verify CRS rate matching functionality: - PDCCH configuration: Start symbol #2, 1 symbol duration - PDSCH configuration: Mapping type A, Start symbol #3, 9 symbol duration - DMRS configuration: Type 1, Single symbol, 1 additional DMRS - CRS configuration: 4 ports, vshift 0 Proposal #7: Use 1 additional DMRS for FR2 scenarios with Doppler frequency 300 Hz. Proposal #8: Use the following configuration of CSI-RS resources for FR1 and FR2 PDSCH demodulation requirements RRC configuration; 2 Tx test; 4 Tx test. NZP CSI-RS resource; periodicityAndOffset; slots20, 0; slots20, 0. nrofPorts; 2; 4. frequencyDomainAllocation; Other, ‘000001’; Other, ‘000001’. firstOFDMSymbolInTimeDomain; 12; 12. cdm-Type; 2; 2. density; 1; 1. ZP CSI-RS resource #1; periodicityAndOffset; slots20, 0; slots20, 0. nrofPorts; 2; 4. frequencyDomainAllocation; Other, ‘000100’; Other, ‘001000’. firstOFDMSymbolInTimeDomain; 12; 12. cdm-Type; 2; 2. density; 1; 1. ZP CSI-RS resource #2; periodicityAndOffset; slots20, 0; N/A. nrofPorts; 2. frequencyDomainAllocation; Other, ‘001000’. firstOFDMSymbolInTimeDomain; 12. cdm-Type; 2. density; 1. Proposal #9: Configure CSI-RS for beam management for FR2 PDSCH demodulation requirements. CSI-RS parameters are FFS. |
| --- | --- | --- | --- |
| R4-1812164 | NR PDSCH simulation results | Intel Corporation |  |
| R4-1812165 | NR SDR performance requirements | Intel Corporation | Proposal #1: Adopt methodology described in Section 2.1 to define look up table for deriving of exact MCS parameters for NR SDR requirements. Proposal #2: Use the following test parameters for SDR requirements for NR carriers: - SSB: periodicity 20 ms, slot #0 within period - CSI-RS for tracking: 20 ms, 1 slot, offset 10ms from SSB slot - CSI-RS for CSI acquisition: mapping in slots with SSB in OFDM symbols without SSB - No PDSCH scheduling in SSB slots and TRS slots - No FDM between CORESET and PDSCH - 1 additional DMRS - PTRS configuration for FR2: KPTRS=2, LPTRS=1 - TDD configuration - FR1 15kHz: DDDSU - FR1 30KHz: 7D1S2U - FR2 60 kHz: DDSU - FR2 120 kHz: DDDSU Proposal #3: For EN-DC SDR requirements for LTE CC reuse assumptions from LTE SDR tests with modification of FRC for TDD carriers (i.e. update LTE FRC to enable UL-DL configuration #2). |
| R4-1812166 | NR FR2 UE PDSCH demodulation performance requirements | Intel Corporation | Observations #1: PDSCH performance is rather sensitive to CF value and phase noise impact may lead to performance degradation up to 3.1 dB in scenarios with achievable maximum throughput. Observations #2: Under the worst conditions (CF 52GHz) rather small PDSCH performance degradation (<=1.0 dB) can be observed only for scenarios with Rank 1 and limited set of MCS indexes. Proposal #1: Define FR2 PDSCH 64QAM requirements in band agnostic manner for Rank 1 MCS18 transmission. Proposal #2: FFS definition of 64QAM Rank 2 requirements for FR2. |
| R4-1812167 | Summary of PDSCH simulation results of NR UE demod (FR1 FDD) | Intel Corporation |  |
| R4-1812168 | Summary of PDSCH simulation results of NR UE demod (FR1 TDD) | Intel Corporation |  |
| R4-1812169 | Summary of PDSCH simulation results of NR UE demod (FR2) | Intel Corporation |  |
| R4-1812223 | Remaining issues for NR PDSCH demodulation requirements | China Telecom | Proposal 1: Test LDPC base graph 2 and PDSCH mapping type B in the same test case(s). For the test, use MCS 2; use S=5 and L=7 in the DL slots, and reduce L in the TDD special slot according to the maximal allowable DL symbols. Proposal 2: For FR1 TDD, introduce additional test cases for 30 kHz + 60 MHz, 30 kHz + 80 MHz, 30 kHz + 100 MHz. Proposal 3: For each additional SCS & channel BW in FR1, define additional test cases assuming MCS 19, 2 layer MIMO, 2Tx 2Rx and 2Tx 4Rx. For each additional SCS & channel BW in FR2, define additional test case assuming MCS 13, 2 layer MIMO, 2Tx 2Rx. Proposal 4: For NR CA, the demodulation requirements defined for LTE CA can be considered as a starting point, and other requirements shall also be added when necessary. If it is impossible to finalize the CA requirements in Rel-15 timeline, maybe a phased approach can be discussed. |
| R4-1812396 | Parameters for PDSCH Demodulation Performance Tests | Qualcomm Europe Inc. (Spain) | Proposal 1: PDSCH demodulation performance tests for PDSCH scheduling Type B should be defined with lower priority compared to PDSCH scheduling Type A. Proposal 2: For LTE-NR coexistence scenario, define PDSCH demodulation performance test with PDSCH Type A, Start symbol (S): 3, Duration (L): 9. Proposal 3: For simulation purposes, do not model Rx phase noise. Add phase noise degradation in the implementation margin. Proposal 4: Define band agnostic requirements for FR2 PDSCH demodulation performance based on phase noise degradation at 39GHz for release 15. Proposal 5: Define separate FR2 PDSCH demodulation performance requirements for 52GHz bands in future when those bands are added. Proposal 6: Define FR2 PDSCH demodulation tests for 64QAM Rank2 at least for MCS17. Proposal 7: For High Speed Train (HST) PDSCH demodulation performance tests using HST single tap model: - Define the test with MCS4, Rank 1 - Configure TRS with periodicity of 10ms. - Configure 1 additional DMRS symbol. - Configure 1 symbol PDCCH with REG Bundle Size 6. Proposal 8: Define a table with testable MCS limitation based on number of Rx and Rank for defining SDR requirements. Proposal 9: Use the following for defining SDR requirements: - 1 additional DMRS for both FR1 and FR2. - PTRS configuration: 1 port, per 2PRB in frequency domain, per symbol in time domain. - Number of HARQ processes = 8 for both FR1 and FR2. - Use 85% of peak throughput as test metric. Proposal 10: Define at least one PDSCH demodulation performance test with dynamic TDD configuration. |
| R4-1812460 | Simulation results for NR PDSCH (FR2) | Samsung |  |
| R4-1812461 | Simulation results for NR PDSCH (FR1) | Samsung |  |
| R4-1812462 | Over views on open issues of PDSCH demodulation test cases | Samsung | Proposal1: In Rel-15, for NSA EN-DC operation firstly focused on NR carrier only requirements (no LTE carrier requirements). Meanwhile in later cases, we can consider introduce specific requirements for EN-DC operation for both LTE carrier and NR carrier. Proposal 2: For NR only requirements, the LTE configuration defined in TS38.133 A3.7.2 can be considered to be reused. Observation 1: Additional test case list can be summarized as below: - CHBW: - FR1 15kHz: 20MHz - FR2 30kHz: 40MHz, FFS for 100MHz - FR2 120kHz: 50MHz, 200MHz - FR2 60kHz: 50MHz - HST: - In last RAN plenary, it’s agreed HST performance requirements will be further discussed in TEI15. - Introducing ZP CSI-RS configurations in part of PDSCH demodulation test case(s) - Introducing additional PDSCH test cases with following PMI under FR2 120kHz with DDDSU DL-UL configurations Proposal 3: Focused to finalize requirements on agreed test case list in R4-1811723 at least before Dec 2018, remaining additional test cases can be introduced under TEI-15. |
| R4-1812553 | Updated simulation results on NR PDSCH | CMCC |  |
| R4-1812731 | Views on UE demodulation requirements | NTT DOCOMO, INC. | Proposal 1: ZP CSI-RS for forward compatibility should be tested in normal PDSCH demodulation test. Proposal 2: Use following parameters for ZP CSI-RS configurations for normal PDSCH demodulation test. Parameters; Values. First subcarrier index in the PRB used for CSI-RS (k0); TBD. First OFDM symbol in the PRB used for CSI-RS (l0); TBD. Number of CSI-RS ports (X); TBD. CDM Type; TBD. Density (ρ); TBD. CSI-RS periodicity; TBD. CSI-RS offset; TBD. Proposal 3: One normal PDSCH demodulation test with DDDSU and random precoding should be replaced to follow PMI test. Parameters; Values. Frequency band; FR2. TDD DL/UL Config.; DDDSU. Precoding; Follow PMI. Rank; 1. MCS; 4. Proposal 4: For FR2 UE demodulation test, band-specific requirements is introduced at least for phase noise-limited scenario, i.e., high SNR scenario, details FFS. - Introduce separate requirement at least for n260. Proposal 5: Configurations of NZP CSI-RS should be determined as follows. Parameters; Values. First subcarrier index in the PRB used for CSI-RS (k0); TBD. First OFDM symbol in the PRB used for CSI-RS (l0); TBD. Number of CSI-RS ports (X); TBD. CDM Type; TBD. Density (ρ); TBD. CSI-RS periodicity; TBD. CSI-RS offset; TBD. Proposal 6: Support CBW and SCS in the following Table for PDSCH performance test. CBW (MHz); FR1; FR2. 15 kHz; 30 kHz; 60 kHz; 60 kHz; 120 kHz. 5; N/A; N/A; N/A. 10; N/A; N/A. 15; N/A; N/A. 20; N/A; N/A. 25; N/A; N/A. 30; N/A; N/A. 40; N/A; N/A. 50. 60; N/A; N/A; N/A. 80; N/A; N/A; N/A. 100; N/A. 200; N/A; N/A; N/A. 400; N/A; N/A; N/A; N/A. |
| R4-1812732 | Views on UE demodulation requirements | NTT DOCOMO, INC. |  |
| R4-1812733 | Initial simulation results for SDR test | NTT DOCOMO, INC. |  |
| R4-1812734 | Simulation results for SDR test design | NTT DOCOMO, INC. | Observation: The impact of CBW to the throughput performance is negligible, i.e., up to 0.2 dB. Proposal: SDR test can be designed with CBW-agnostic manner. |
| R4-1812735 | Simulation results for SDR test design | NTT DOCOMO, INC. |  |
| R4-1813084 | NR PDSCH simulation result | MediaTek inc. |  |
| R4-1813265 | NR PDSCH Demodulation Performance Simulation Results | Qualcomm Europe Inc. (Spain) | Observation 1: SNR required for 70% of peak throughput for Test 9 for FR1 TDD and FDD are violating the Testable SNR as defined in [2] for MMSE-IRC receiver. So, this test case should not be defined. It may be testable with low correlation. Observation 2: There are no 64QAM Rank2 test cases defined for FR2. Observation 3: Test 4 for FR2 is defined with ULA Med MIMO correlation which is not realistic for FR2 scenarios. |
| R4-1813438 | NR UE performance test scenarios and open issues | Ericsson | Proposal 1: Add the following additional tests in Rel-15, which could be handled under TEI after the agreed scenarios are specified. - FR1 TDD 100MHz 30kHz - PMI tests with 16 and 32 Tx ports - PDCCH AL as 16 Proposal 2: More thorough study is needed before the phase noise model is concluded for UE performance tests. Similar considerations should be made for BS demodulation tests. |
| R4-1813530 | Draft TP on FR2 PDSCH Demodulation Performance Requirements | Qualcomm Europe Inc. (Spain) |  |
| R4-1813542 | Draft TP on FR2 PDSCH SDR Performance Requirements | Qualcomm Europe Inc. (Spain) |  |
| R4-1813607 | Remaining issues on soft combining verification for PDSCH | NTT DOCOMO, INC. | Proposal 1. The following should be agreed as a package. - Introduce FR1 and FR2 test case for 70% test point targeting for high rank and high modulation order and 16 HARQ processes for TDD and 8 HARQ processes for FDD - MCS #13 and rank 4 (4Rx) and MCS #19 and rank 2 (2Rx) for FDD with 8 HARQ process for FR1 - MCS #13 and rank 4 (4Rx) and MCS #19 and rank 2 (2Rx) for TDD (7D1G2U) with 16 HARQ process for FR1 - MCS #13 and rank 2 (2Rx) for TDD (DDDSU) with 16 HARQ process for FR2 - HARQ process for SDR requirements is [8] for TDD and [4] for FDD. Proposal 2: If LTE and NR share the soft buffer in EN-DC mode, introduce additional EN-DC requirement assuming 30% test point for both LTE and NR with noise. |
| R4-1813632 | Discussion and simulation results on NR PDSCH demodulation performances | Huawei, HiSilicon |  |
| R4-1813635 | TP for introducing demodulation performance requirements for interworking TS 38.101-4 section 9 | Huawei, HiSilicon |  |

## Normal test cases

### Soft combining test

Previous RAN4 agreements:

- Performance metrics

  - 30% of maximum throughput

- Test parameters:

  - FDD and TDD

  - TDD UL-DL configurations: 7D1S2U for FR1 and DDDSU/DDSU for FR2

  - FRC: 16QAM Rank 1 (MCS 13)

  - 16 HARQ processes for TDD and 8 HARQ processes for FDD

  - Channel model:

    - FR1: TDL-C, 300ns and 100 Hz

    - FR2: TDL-A, 30ns and [75 or 300] Hz

- FFS whether to define FR1 and FR2 test case for 70% test point targeting for high rank and high modulation order and 16 HARQ processes for TDD and 8 HARQ processes for FDD

Issue 1: for the additional test cases with 70% test point with high order and high rank

Candidate options:

- Option 1: NO additional Test cases with 70%  (QC, Intel, MTK)

- Option 2: Introduce FR1 and FR2 test case for 70% test point targeting for high rank and high modulation order and 16 HARQ processes for TDD and 8 HARQ processes for FDD meanwhile for SDR test using HARQ process number 8 for TDD and 4 for FDD (NTT DoCoMo, Huawei, Ericsson, China Telecom)

Further offline discussion leading by NTT DoCoMo and Intel

Issue 2: Whether introducing additional test cases for EN-DC with 30% test point to verify soft-buffer sharing between LTE and NR

FFS for further discussion after normal requirements for EN-DC finalized.

### Type B test Cases

Previous RAN4 agreements

- Define at least one test case for PDSCH mapping Type B. Companies are encouraged to bring proposals on the test parameters

  - FR1:

    - Option 1: k0 = 0, S=5, L=7 for MCS 2, DMRS type 1 single symbol with one additional DMRS (Position #5, #9)

    - Other options are note precluded

Issue 1: PDSCH and DMRS configuration

- Option 1: Start symbol (S) 5, Duration (L) 7, 1 additional DMRS

Issue 2: MCS level for Type B

- Option 1: MCS 4 for 2Rx and MCS 8 for 4Rx (Intel, QC)

- Option 2: MCS 2 for 2Rx/4Rx (China Telecom)

Intel: with option 2, SNR is low than -6dB

China Telecom: can verify both graph 2 and type B in one single test case with case 2. For SNR point, with considering margin between ideal and final requirements, -6dB still OK. We already have requirements in LTE around -6dB.

Intel: MCS 4 still server purpose with graph 2. -3dB as limitation for alignment.  -6dB close to part of RRM test conditions which means will verify RRM manner at the same time.

Ericsson: For minimum SNR points, if alignment with -6dB then considering IM margin (2 dB) around, final SNR requirements still in the reasonable range.

Ericsson: SNR can be different under connection set-up phase and test phase, we can further check NR RLM SNR operation requirements in RRM.

FFS for further offline

Issue 3: Whether deprioritize Scheduling B test cases

QC: Type B is mandatory with capability signalling for unicast

Huawei: prefer to deprioritize

### LTE-NR co-existence test case(s)

Candidate options:

Option 1: PDSCH Type A, Start symbol (S): 3, Duration (L): 11

Option 2: PDSCH Type A, Start symbol (S): 3, Duration (L): 9 (Qualcomm, Intel)

Option 3: PDSCH Type B, Start symbol (S): 5, Duration (L): 7

### ZP CSI-RS and NZP CSI-RS configurations

Issue 1: whether need to configured ZP CSI-RS and NZP CSI-RS in demodulation test cases

- Option 1: Configured for all test cases in the same slots with SSB and for PDSCH scheduling will skip these slots

-Intel, Ericsson

- Option 2: configured for part of test cases

Candidate agreements:

- FFS whether need to Configure NZP ,ZP CSI-RS, CSI-IM  in all test cases if introduced Then Default: scheduling in the same slots with SSB and skip SSB slots for PDSCH scheduling

- Modify one or part of existing test case(s) with NZP and ZP CSI-RS scheduling in other slots than SSB slots with PDSCH scheduling

Intel: do we need to configure NZP for beam management in FR2; do we need to configure NZP, ZP,CSI-IM for CSI feedback in demodulation test cases?

Issue 2: detailed configurations

|  | RRC configuration | NTT DoCoMo | Intel |
| --- | --- | --- | --- |
| NZP CSI-RS resource | periodicityAndOffset | 5/0 | slots20, 0 |
|  | nrofPorts | 4 | 2 and 4 |
|  | frequencyDomainAllocation | Row4, ‘000001’ | Other, ‘000001’ |
|  | firstOFDMSymbolInTimeDomain | 13 | 12 |
|  | cdm-Type | Fd-CDM2 | 2 |
|  | density | 1 | 1 |
| ZP CSI-RS resource | periodicityAndOffset | 5/0 5/1 | slots20, 0 |
|  | nrofPorts | 4; 8; | 2 |
|  | frequencyDomainAllocation | Row 4 ‘000010’ Row 6 ,’001111’ | Other, ‘000100’ ; Other, ’001000’ |
|  | firstOFDMSymbolInTimeDomain | 13 | 12 |
|  | cdm-Type | Fd-CDM2 | 2 |
|  | density | 1 | 1 |

### PDSCH requirements applicable rule for FR2

Issue 1: Whether explicitly modelling Rx phase noise?

- Option 1: Explicitly modelling (what’s the modelling assumption)- (Intel, Ericsson)

What's the modelling assumption we can use?

- Option 2: Taking into account for define requirements for IM results submission, up to UE implementation no explicitly modelling   (QC, Samsung, Huawei)

Ericsson: observed performance difference under different frequency ranges and different modelling

Intel: PN impact pending MCS and rank combination

QC: PN impact only impact on high rank and high modulation

Issue 2: How to define requirements in FR2

- Option 1: band agnostic requirements up to certain MCS levels and Rank combinations and frequency bands (Samsung, Intel, QC)

  - For frequency range:

    - up to current defined bands (39GHz) , FFS for new bands to 52GHz

    - Applied to all FR2

  - For Rx Impairment assumption for determine achievable MCS and rank

    - No worse than 8% Rx EVM as total

    - Explicitly PN modelling (modelling assumption?)

- Option 2: band/band grouping specific requirements (Ericsson, NTT DoCoMO)

Candidate options:

Candidate agreement: QPSK and 16QAM requirements can be band agnostic for current agreed test cases at least for up to frequency range 40 GHz.

Ericsson wants to further check and come back this week.

Further offline for 64QAM rank1

### Additional test cases

Previous agreements:

- CHBW &SCS

- Default configurations

  - FR1 FDD: 15 kHz SCS + 10 MHz CBW

  - FR1 TDD: 30 kHz SCS + 40 MHz CBW

  - FR2: 120 kHz SCS + 100 MHz CBW

- Additional test case(s)

  - FR1 FDD: 30 kHz SCS + 20 MHz CBW

  - FR1 TDD:

    - 30 kHz SCS + 20 MHz CBW

    - FFS for 30 kHz SCS + 100 MHz CBW

  - FR2

    - 120 kHz SCS + 50 MHz CBW

    - 120 kHz SCS + 200 MHz CBW

    - 60 kHz SCS + 50 MHz CBW

Other combinations can be considered based on operators’ request

- HST:

  - In last RAN plenary, it’s agreed HST performance requirements will be further discussed in TEI15.

- Introducing ZP CSI-RS configurations in part of PDSCH demodulation test case(s)

- Introducing additional PDSCH test cases with following PMI under FR2 120kHz with DDDSU DL-UL configurations

Issue 1: new test cases for additional CHBW and SCSs

Test configurations for additional test cases

- Option 1: 1 test case for additional combinations (MSC 4 and rank1)-Intel

- Option 2: MCS 13 with rank2 for FR2 and MCS 19 rank2 for FR1-China Telecom

- Option 3: Using different MCS levels (from current agreed values) for different combinations of CHBW and SCS-QC

Agreement: MCS 13 with rank2 for FR2 and MCS 19 rank2 for FR1-China Telecom

Ericsson: What’s timeline for these agreed additional test cases but didn’t capture in current simulation lists?

Huawei: Focused on current simulation lists, after finalized these cases we can further work on other agreed test cases.

QC: Similar concern for timeline, we can pick up one of already agreed MCS and Rank .

Ericsson: Before Nov meeting, focused on current agreed simulation cases, we will further work on other already agreed test cases after current simulation cases finalized.

Intel: Maybe we can depriotize these agreed additional test cases compared to already simulated cases?

Additional combinations proposed by companies

- FR1 15kHz (FDD):

  - 20MHz (NTT DoCoMO)

Intel: For TDD, we need to consider introduce 15 kHz for NR –LTE co-existence

NTT DoCoMO: 20MHz for FDD also important

QC: If we try to add more combinations, these will broke previous discussion and test design purpose. We prefer to stick on what we already agreed; these cases already ensure test coverage.

China Telecomm: In previous meeting, we agreed other combinations not excluded. Meanwhile we also understand the concern on work load and time line.

Intel: What’s the motivation for these new proposals? We already has REFSENS and Maximum input levels test covering all combinations. Timeline is issue.

QC: We can understand operators have different deployment plan. But from performance requirements purpose, all the features already verified with current bandwidths and combinations. Basedband processing performance is similar under different CHBWs.

China Telecom: In RF core requirements, AWGN used, MCS and rank can be different.

- FR1 TDD: 15kHz+20MHz (Intel) for co-existence

- FR1 30KHz TDD:

  - 100MHz (China Telecom, NTT DoCoMo)

FFS for whether above new CHBW and SCS combinations

No new combinations of CHBW and SCS will be introduced for Rel-15 except below combinations:

- FR1 FDD: 15kHz +20MHz

- FR1 TDD: 30kHz +100MHz

We will comeback this week

Issue 2: new proposed additional test cases

- Introduce additional dynamic TDD test cases (QC)

QC: It’s mandatory, we want to ensure these feature be verified. We can take one of existing test cases. With TDD DL-UL configurations change with dynamic pattern.

Huawei: not necessary since we already introduce several TDD DL-UL configurations, in previous we already agreed these will be configured by RRC.

QC: We want to verify uE supporting dynamic configurations rather than SP configurations with using current agreed TDD DL-UL pattern.

MTK: agree with Huawei, no performance difference

QC: Still functionality need to be verified even no performance difference whether UE supporting or not. No additional work required

FFS for further discuss

- Introduce 64QAM rank2 test cases for FR2-MCS 17 (QC)

QC: We want to have test cases in Fr2 covering 64QAM and rank2 combinations.

Introducing test cases for 64QAM and rank2 in FR2 (MCS 17)

### Other issues

Issue 1: MCS for QPSK

- Option 1: MCS4 for 2Rx and 4Rx

- Option 2: MCS 4 for 2Rx and MCS 8 for 4Rx

Issue 2: Test cases with PRB bundling size 4 or WB

Option 1: 4 PRB bundling for the following tests:

- FDD, MCS 4, 1 Layer, Number of PDSCH PRBs 52

- TDD, MCS 4, 1 Layer, Number of PDSCH PRBs 106

Option 1:	Configure WB bundling for the following tests:

- FDD, MCS 4, 1 Layer, Number of PDSCH PRBs 6

- TDD, MCS 4, 1 Layer, Number of PDSCH PRBs 6

QC: we prefer full PRB allocation for both 4RB and WB bundling size

Huawei: We already agree we will cover 4RB and WB bundling size with test cases? Do we need to test them?

QC: larger bundling size will have benefits for performance for channel estimation

Ericsson:  good to have some test cases

Intel: What’s the assumption for CE? Single PRB or bundling size basis

Issue 3: number of additional DMRS for FR2 test cases with 300Hz Doppler

- Option 1: 1 additional

- Option 2: 2 additional

Issue 4:  HST test cases configurations

Proposal from QC:

- Define the test with MCS4, Rank 1

- Configure TRS with periodicity of 10ms.

- Configure 1 additional DMRS symbol.

- Configure 1 symbol PDCCH with REG Bundle Size 6.

Issue 5: work plan

Samsung: Focused to finalize requirements on agreed test case list in R4-1811723 at least before Dec 2018, remaining additional test cases can be introduced under TEI-15.

Issue 6: CSI-RS configurations for FR2 to ensure beam management function during test

## SDR Test

### Issue 1: Test configurations:

Additional DMRS:

- Option 1: 1

PTRS configuration:

- Option 1: 1 port, per 2PRB in frequency domain, per symbol in time domain.

TRS configurations:

- Option 1:  20 ms, 1 slot, offset 10ms from SSB slot

SSB:

- Option 1: periodicity 20 ms, slot #0 within period

PDSCH scheduling:

- No PDSCH in SSB slots and TRS slots

- No FDM between CORESET and PDSCH

TDD configuration

- FR1 15kHz: DDDSU

- FR1 30KHz: 7D1S2U

- FR2 60 kHz: DDSU

- FR2 120 kHz: DDDSU

HARQ Process:

- TDD:8 and FDD :4

### Issue 2: Test metric

Option 1: 85%

### Issue 3: Test methodology

Previous agreements:

- MCS and TBS used for each CC is selected based on test parameters and on indicated UE capabilities including MIMO layers, modulation and scaling factor

  - Exact MCS/TBS for SDR testing are FFS and companies encouraged to bring proposals on the MCS selection

    - Option 1: Define look up table to derive MCS and TBS parameters based on UE capabilities

    - Option 2: Define a procedure to derive MCS and TBS parameters based on UE capabilities

    - Other options not precluded

### Issue 4: CHBWs

- Option 1:SDR test can be designed with CBW-agnostic manner

- Option 2: List CHBW sets

### Issue 5: LTE carrier configurations

- Reusing LTE parameters  of SDR  with update FRC for TDD DL-UL changes

# PDCCH and PBCH

- PDCCH

  - Test case for Aggregation level (AL 16)

  - Test parameters for 1symonl FR1 FDD test case

  - Test cases with 4Rx with interleaved CCE-to-REG mapping

  - Power allocation

- PBCH Test

  - Necessity of 4Rx test

| R4-1812175 | Simulation results for PDCCH | Intel Corporation |  |
| --- | --- | --- | --- |
| R4-1812176 | Discussion on UE performance requirements for PBCH | Intel Corporation | Observation #1: The SNR operating range for PBCH testcases with 4Rx is very low -9 to -7.5 dB Observation #2: In NR RRM requirements based on SS/PBCH are limited to 2Rx Observation #3: Enabling 4Rx for SS/PBCH increases UE power consumption significantly Observation #4: Motivation to introduce 4Rx requirements for PBCH is not clear Observation #5: PBCH performance is not a bottleneck factor for the network coverage and the coverage is typically limited by UL physical channels Observation #6: PBCH performance with 2Rx can be further improved by lowering the SSB periodicity to 5ms or 10ms and enabling more SSBs to be combined within the PBCH TTI. Proposal #1: Do not introduce 4RX PBCH performance requirements Proposal #2: Deprioritize the work on the PBCH performance requirements definition |
| R4-1812224 | Remaining issues for NR PDCCH demodulation requirements | China Telecom | Proposal 1: Cover aggregation level of 16 for PDCCH demodulation requirements. Proposal 2: Assume contiguous frequency domain resources for the CORESET. Proposal 3: Assume EPRE ratio of PDCCH_DMRS to SSS is 0dB, and EPRE ratio of the PDCCH to PDCCH_DMRS is 0dB. |
| R4-1812225 | Antenna configuration for NR PBCH demodulation requirements | China Telecom | Proposal 1: Introduce 1 Tx requirements and/or 2 Tx requirements with transparent precoding. Proposal 2: Introduce 4Rx requirements in addition to 2Rx requirements. |
| R4-1812278 | Simulation results for NR PDCCH | CATT |  |
| R4-1812455 | Discussion on UE demodulation requirements for PDCCH | Intel Corporation | Proposal #1: Introduce 1 symbol PDCCH testcase with 15KHz SCS Proposal #2: Introduce test case with reasonable operating SNR with AL=16 and 2Rx Proposal #3: Define PDCCH test cases with interleaved CCE-to-REG mapping for 4Rx with REG bundle size of 6 Proposal #4: Define the following testcases for PDCCH demodulation requirements with 2Rx: |
| R4-1812476 | Analysis on PBCH with 4Rx | Samsung | Q1: What’s previous RAN agreements for mandating 4Rx Observation 1: RAN plenary only agreed to mandate 4Rx demodulation of control channel (PDCCH) and data channel (PDSCH). Observation 2: Following RAN1 specification, control channel (PDCCH) and broadcast channel (PBCH) are separate physical channel. Q2: What’s the benefit of mandating 4Rx for PBCH demodulation Observation 3: Cell coverage bottleneck is UL not DL. Observation 4: Mandating 4Rx of PBCH demodulation solely without mandating 4Rx for RLM has no benefit for DL cell coverage enhancement since under that SNR points (-14~11 dB), UE already out of sync. Q3: What’s the drawback of mandating 4Rx for PBCH demodulation Observation 5: PBCH decoding related to RRM operation i.e. cell identification, mandating of 4Rx PBCH decoding conflicts with previous RAN agreements for RRM. Observation 6: Mandating 4Rx PBCH decoding has huge impact on UE implementation and power consumption for idle, DRX and even initial cell search stage. |
| R4-1812554 | Further discussion on NR PDCCH demodulation requirements | CMCC | Proposal: It is proposed to specify NR PDCCH demodulation requirements for aggregation level 16. |
| R4-1812555 | Discussion on NR PBCH 4Rx demodulation requirements | CMCC | Proposal 1: It is proposed that RAN4 should specify PBCH demodulation requirements in Rel-15. Proposal 2: It is proposed to define 4Rx PBCH demodulation requirements in Rel-15. |
| R4-1812850 | Discussion on NR PBCH demodulation requirements | Ericsson | Proposal 1: It is sufficient to specify the NR PBCH demodulation requirement with 2Rx. Proposal 2: RAN4 study the PBCH demodulation performance with/without the knowledge of SSB index (or PBCH DMRS sequence). Proposal 3: RAN4 discuss the PBCH demodulation requirements consider SSB index acquisition performance or not. |
| R4-1812851 | TP to TS 38.101-4: 5.4 FR1 PBCH demodulation requirements | Ericsson |  |
| R4-1812852 | TP to TS 38.101-4: 7.4 FR2 PBCH demodulation requirements | Ericsson |  |
| R4-1812892 | Discussion on UE demodulation requirements of NR PBCH | OPPO | Proposal: UE antenna configuration in simulation assumption for test cases for NR PBCH demodulation should just include 1x2, i.e., with 1Tx and 4Rx. Proposal 2: Define 2Rx requirements for NR PBCH demodulation. |
| R4-1813085 | NR PDCCH simulation result | MediaTek inc. |  |
| R4-1813086 | NR PBCH simulation result | MediaTek inc. |  |
| R4-1813356 | NR PDCCH Demodulation Performance Simulation Results | Qualcomm Europe Inc. (Spain) |  |
| R4-1813439 | Simulation results for NR UE PDSCH demodulation tests | Ericsson |  |
| R4-1813608 | Remaining issues on PDCCH demodulation requirements | NTT DOCOMO, INC. | Proposal: Specify the PDCCH demodulation requirements not only for AL = {2, 4, 8} but also AL = {16} in both FR1 and FR2. The following is simulation assumption for AL=16 (green part is modified part from [1]) |
| R4-1813629 | Discussion on NR PBCH demodulation requirements | Huawei, HiSilicon | Proposal: Specify the NR PBCH demodulation performance requirements with 4Rx, and on the NR bands for which the UE shall be equipped with 4Rx ports as a baseline the UE shall comply with the NR 4Rx PBCH demodulation performance requirements. |
| R4-1813630 | Way forward on NR PBCH demodulation requirements | Huawei, HiSilicon |  |
| R4-1813631 | Discussion and simulation results on NR PDCCH demodulation performances | Huawei, HiSilicon |  |
| R4-1813440 | Simulation results for NR UE PDCCH demodulation tests | Ericsson |  |
| R4-1813442 | Summary of alignment and impairment results for NR UE PDCCH demodulation tests | Ericsson |  |
| R4-1813633 | TP for introducing FR1 PDCCH requirements in TS 38.101-4 section 5.3 | Huawei, HiSilicon |  |

## PDCCH

### Issue 1：Aggregation level (AL 16)

Previous agreements: at least {2, 4, 8}, FFS for {16}

Candidate options:

- Option 1: Introduced test case(s) for AL 16 for both 2RX and 4RX: China Telecom, CMCC, NTT DoCoMo, Ericsson

- Option 2: Introduced test case(s) for AL 16 for 2RX only: Intel, QC, MTK

- Option 3: No test case(s) for AL 16 in Rel-15

Intel: with 4Rx, SNR will be -10 dB, for 2Rx, will be -6dB/-4dB.

China Telecom: -10dB is alignment results, for IM results will be -7dB still testable.

Ericsson: we maybe find possible parameters to achieve reasonable SNR levels.

Huawei: want to further check CORESET RBs for TDD FR2 with duration 2

### Issue 2: Test parameters for 1symonl FR1 FDD test case

Previous agreement: candidate options

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Propagation condition | Antenna configuration with 2Rx | Antenna configuration with 4Rx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12 | 15 | 1_0 | 48 | 39 | 1 | [4, 8] | Non-interleaved | 6 | [TDL-A, 30ns, 10Hz] | 1x2 Low | 1x4 Low |
| 13 | 15 | 1_1 | 48 | [50] | 1 | [4, 8] | Non-interleaved | 6 | [TDL-A, 30ns, 10Hz] | 2x2 Low | 2x4 Low |

Option 1: Using parameters of case 12 with AL=4

### Issue 3: Test cases with 4Rx with interleaved CCE-to-REG mapping

Previous agreements:

- Agreement in AH1807: REG bundle size =6 for non-interleaved, 2 for interleaved

- Companies are encouraged to bring evaluation results with both REG bundle size 2 and 6 for 4Rx test cases (case 6, 7 and 8) in RAN4#88bis, and decide which value is set based on performance comparison.

Candidate options:

- Option 1: Test case 6, 7 with REG bundle size =2, CORESET BW = 96; Test case 8, Bundle size =6 with interleave size as 3 (QC, Ericsson, NTT DoCoMo))

- Option 2: For 4Rx, interleaved cases 2, using REG bundle size =6 (Intel)

Intel: For 4Rx, with REG bundle size in both noise limited and interference limited scenarios, with REG bundle size =2, cannot ensure good performance under MMSE-IRC receiver assumption. Even MRC receiver in noise limited scenarios is not good.

QC: REG bundle size =2 still valid configurations in real network, we need ensure test coverage covering both 2 and 6

Intel: if NW deploy with 2, then performance will degraded compared to 4Rx. We should provide some configurations which is useful for NW configurations.

QC: Even UE can report 4Rx, but from NW perspective, both 2Rx and 4Rx possibly exist in real NW and 4Rx UE may not always with 4Rx on.

Intel: REG bundle size 6 also works for 2Rx, and no performance degradation compared to Bundle size 2. 6dB performance difference for 4Rx. Question for NW will deploy 2?

Ericson: support option 1.

Intel: which type receiver assumptions? Performance different will be observed.

### Issue 4: Power allocation

Proposal from China Telecom: Assume EPRE ratio of PDCCH_DMRS to SSS is 0dB, and EPRE ratio of the PDCCH to PDCCH_DMRS is 0dB.

### Additional proposals

Issue 5: Revised time duration for one of test cases among 6, 7 and 8 from 1 to 2 (NTT DoCoMo)

QC: we prefer choosing test case 6 or 7 to ensure SNR testable

Ericsson: want to further check

Issue 6: simulation alignment issue

Intel: for some test cases, hug gap observed, we suggest for further offline aligning simulation assumption

## PBCH

### Issue 1: Necessity of 4Rx PBCH test cases

Candidate options:

- Option 1:Introducing 4Rx PBCH test cases

  - CMCC, China Telecom, Huawei

Question for option1: it’s mandatory or optional? And what’s the applicable condition?

- Option 2: Not introducing 4Rx PBCH test cases

  - Intel, Ericsson, Samsung, Oppo?, LGE,QC

### Issue 2: UE assumption on DMRS SSB index acquisition for PBCH demodulation

Intel: we prefer to include SSB index acquisition process. If we cannot reach consensus, then PBCH will be drop in Rel-15

# CSI

List contributions

| R4-1812179 | Simulation results and discussion on NR CQI reporting under AWGN conditions | Intel Corporation |
| --- | --- | --- |
| R4-1812180 | Simulation results and discussion on NR CQI reporting under fading conditions | Intel Corporation |
| R4-1812181 | Simulation results and discussion on NR PMI reporting | Intel Corporation |
| R4-1812182 | Simulation results and discussion on NR RI reporting | Intel Corporation |
| R4-1812420 | Assumptions for CSI Reporting Tests | Qualcomm Europe Inc. (Spain) |
| R4-1812463 | Over views on CSI test cases | Samsung |
| R4-1812464 | Simulation results for static CQI test cases | Samsung |
| R4-1812465 | Simulation results for fading CQI test cases | Samsung |
| R4-1812466 | Simulation results for PMI test cases | Samsung |
| R4-1812467 | Simulation results for RI test cases | Samsung |
| R4-1812468 | Simulation results summary for NR CSI (FR1 TDD) | Samsung |
| R4-1812469 | Simulation results summary for NR CSI (FR1 FDD) | Samsung |
| R4-1812470 | Simulation results summary for NR CSI (FR2 TDD) | Samsung |
| R4-1812471 | Framework of NR CSI requirements | Samsung |
| R4-1812472 | TP for 38.101-4 section 6.3 FR1 PMI test cases | Samsung |
| R4-1812473 | TP for 38.101-4 section 8.3 FR2 PMI test cases | Samsung |
| R4-1812474 | TP for 38.101-4 section 10 CSI test cases of interworking | Samsung |
| R4-1813087 | NR CSI simulation result | MediaTek inc. |
| R4-1813380 | NR CSI Reporting Simulation Results | Qualcomm Europe Inc. (Spain) |
| R4-1813543 | Draft TP on FR1 Rank Indication Reporting Performance Requirements | Qualcomm Europe Inc. (Spain) |
| R4-1813561 | Draft TP on FR2 Rank Indication Reporting Performance Requirements | Qualcomm Europe Inc. (Spain) |
| R4-1813441 | Simulation results for NR UE CSI tests | Ericsson |
| R4-1813634 | TP for introducing FR1 Reporting of Channel Quality Indicator (CQI) TS 38.101-4 section 6.2 | Huawei, HiSilicon |

## Generic test configurations

### Issue 1:  NZP CSI-RS configurations

Previous RAN4 agreements:

| Parameters | 2 Ports | 4 ports | 8 ports |
| --- | --- | --- | --- |
| nrofPorts | 2 | 4 | 8 |
| firstOFDMSymbolInTimeDomain | 5 | 5 | 5 |
| firstOFDMSymbolInTimeDomain2 | N.A | N.A | N.A |
| cdm-Type | fd-CDM2 | fd-CDM2 | cdm4-FD2-TD2 |
| density | 1 | 1 | 1 |

Frequency location:

- Option1: Other, 001000 (Samsung)

- Option2: Other,000001 (Intel)

### Issue 2: CSI-IM configurations

| Parameters | Samsung | Intel | Agreement |
| --- | --- | --- | --- |
| CSI-IM-RE-pattern | Pattern 0 | Pattern 0 | Pattern 0 for FR1 and 1 for FR2 |
| CSI-IM-ResourceMapping subcarrierLocation-p0/ symbolLocation-p0 | 6/8 |  |  |

QC: Prefer pattern 0 for FR1 and pattern 1 for FR2?

QC: We need to ensure not aligned with other signals i.e. DMRS locations

### Issue 3: ZP CSI-RS configurations

Necessity of introducing ZP CSI-RS for CSI test cases:

- Option 1: Need (Samsung, Intel)

- Option 2: No need (Qualcomm)

| Parameters | Samsung | Intel | Agreement |
| --- | --- | --- | --- |
| Number of ports of ZP CSI-RS | 4 | 4 | 4 |
| ZP CSI-RS CDM Type | FD-CDM2 | FD-CDM2 | FD-CDM2 |
| CSI-RS-Density | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 8 | 5 |  |
| frequencyDomainAllocation | 001000 | 000100 |  |

### Issue 4: CSI report and CSI-RS resource periodicity and offset for periodic CSI

FR1 15 kHz:

- Option 1: 5 slots with 1 slot offset (Qualcomm, Intel, Samsung)

FR1 30 kHz TDD:

- Option 1: 5 slots with 1 slot offset (Qualcomm, Intel, Samsung)

- Option 2: 10 slots with 1 slot offset(Samsung)

FR2 120 kHz TDD:

- Option 1: 5 slots with 1 slot offset  (Qualcomm, Intel)

- Option 2: 20 slots with 1 slot offset (Samsung)

### Issue 5: Scheduling and delay assumption for aperiodic CSI reporting and aperiodic CSI resources

Previous agreements:

- CSI computation delay and gNB emulator scheduling delay assumption

  - Periodic CSI feedback delay can be assumed as below:

    - Minimum CSI computation delay 4ms

    - Minimum gNB emulator scheduling delay 4ms

  - Aperiodic CSI feedback delay followed the definition in TS38.214 5.4, assuming same processing delay for UE computation and gNB emulator processing delay.

    - For simulation purpose, the detailed aperiodic CSI feedback delay for aperiodic CSI reporting tests is FFS.

Candidate options of scheduling pattern and reporting delay

FR1 15 kHz:

- Option 1:Per 5slots with 6 slots delay

- Option 2: 5 slots/0 slot offset with 8 slots delay

FR1 30 kHz TDD:

- Option 1:Per 6 slots with 10 slots delay

- Option 2: 10 slots/1 slot offset with 10 slots delay

FR2 120 kHz TDD:

- Option 1:Per 8 slots with 8 slots delay

- Option 2: 20 slots/1 slot offset with 24 slots delay

Question for discussion, when calculate processing delay which value used table 5.4-1 or 5.4-2 in TS38.214?  Z1 or Z2 used?

### Issue 6: Number of HARQ process

Previous agreements: Reusing PDSCH set-up:FR1 15kHz FDD: 4, FR1 30kHz  : 8, FR2:120kHz: 10

QC: using 8 for TDD

## CQI reporting

Issue 1: Test requirements and test SNR points

| Issues: | Static CQI | Wideband Fading CQI |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Test SNR | Test SNR | Percentage requirements (aifa) | BLER | TP ratio requirements(Gamma) |
| Samsung | FR1 2Rx:[8/9]dB, [14/15]dB FR1 4Rx:[5/6]dB, [11/12]dB FR2 2Rx:[8/9]dB, [14/15]dB | For FR1 2Rx (FDD and TDD): 6/7dB +12/13 dB For FR1 4Rx (FDD and TDD): 3/4dB +9/10 dB For FR2 2Rx (TDD): 6/7dB +12/13 dB | FR1 2Rx: 20% FR1: 4Rx :2% FR2: 2Rx:2% | 0.02 | 1.05 |
| QC |  |  |  |  |  |
| Intel |  |  |  |  |  |
| Agreement |  |  |  |  |  |

Issue 2: Sub-band CQI

Whether introducing sub-band CQI for FR2?

- Option1: FR1 only

- Option2: both FR1 and FR2

MIMO configuration:

- Option 1: 2Tx

- Option 2: 1Tx

## PMI reporting

- Test metric and test requirements

- MIMO correlation for FR2 2Tx PMI test cases

| Issues: | 4Tx PMI |  | 8Tx PMI |  | 2Tx PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test Point | Requirements | Test Point | Requirements | Test Metric | Requirements | MCS and Rank | MIMO correlation |
| Samsung | 70% | 1.4 | 70% | 1.5 | 70% | 1.1 | MCS 13 Rank1 | ULA Medium |
| QC | 90% |  | 90% |  | 90% |  |  |  |
| Intel |  |  |  |  |  |  | MCS 4 Rank1 | ULA Low |
| Agreement |  |  |  |  |  |  |  |  |

## RI Test

MIMO correlation for FR2 RI test and detailed test requirements and test points

# Channel model

| R4-1812177 | Discussion on simplified TDL channel models | Intel Corporation |
| --- | --- | --- |
| R4-1812178 | Discussion on channel models for FR2 | Intel Corporation |
| R4-1812326 | Simplification of FR1 TDL channel models | Huawei Technologies Sweden AB |
| R4-1812327 | Simplification of FR2 TDL channel models | Huawei Technologies Sweden AB |
| R4-1812328 | TP on channel models for TS38.101-4 | Huawei Technologies Sweden AB |
| R4-1812334 | TP for introducing propagation conditions (Annex B) of TS 38.101-4 | Huawei Technologies Sweden AB |
| R4-1812736 | Views on HST scenarios | NTT DOCOMO, INC. |
| R4-1812853 | Simplified propagation channel models for FR1 UE demodulation requirements | Ericsson |
| R4-1812854 | Simplified propagation channel models for FR2 UE demodulation requirements | Ericsson |
| R4-1813572 | Impact of tap reduction on TDL models | Keysight Technologies UK Ltd |

### Issue 1: Simplified method for TDL model

Previous RAN4 agreements:

Way forward on channel models simplification for FR1

- Simplification methods

  - Option 1: Choose strongest paths that contribute to [X%] of total power

  - Option 2: Choose N paths for TDL-A and TDL-C using the frequency correlation method

- Use option 1 and option 2 to generate up to 12 taps after 5 ns quantization for following FR1 models

  - RMS delay spread TDL-A (30 ns)

  - RMS delay spread TDL-B (100 ns)

  - RMS delay spread TDL-C (300 ns)

- Further study the frequency correlation characteristics of channel models for Option 1 and 2

e.g. The most suitable model is selected based on the visual inspection of the quantized FCF. The lower the correlation the better, preferably below

Proposals from companies:

| Intel | Proposal #1: Set delay grid quantization to 5ns for FR1 and FR2 channel models Proposal #2: Use simplification method of choosing strongest paths contributing to 90% of total power for TDL-A, TDL-C and 85% of total power for TDL-B. |
| --- | --- |
| Huawei | Proposal: The simplified models described in Table 1, Table 2, and Table 3 (TDL-A-FR1, TDL-B-FR1, TDL-C-FR1, respectively) are used in SISO NR UE demodulation FR tests. |
| Ericsson | Detailed delay profiles summarized in paper. |
| Key sight | Observation 1: Delay quantization in the simulated cases has negligible impact on delay spread and FCF. Proposal 1: The delay resolution, i.e. increment of the delay grid, should be ≤5ns. Parameter tables can be specified with 5ns delay grid, but a finer delay grid can be used in fading emulation, by rounding the specified delays to the utilized grid. Proposal 2: The tap reduction method should pick 12 strongest taps and remove the rest with all the considered models (TDL-A…D). |

Candidate options for simplified procedures:

- Option 1: : Choose strongest paths that contribute to [X%] of total power Intel

- Option 2: : Choose N paths for TDL-A and TDL-C (Ensure <=12 taps after DS scaling and quantization)

- Ericsson (Ensure <=12 taps after DS scaling and quantization)/Keysight?

Step 1: Choose N strongest paths NLOS paths from the original PDPs shows in TR38.901.

Step 2: Apply the desired delay spread after removing the weak paths.

Step 3: Round the path delay position close to the grid. Set the grid step size to 5 ns.

Step 4: If two or more paths are rounded to the same grid τ, the path power at τ is set as a sum of all the path power values (in linear) rounded to τ.

Step 5: Normalize the power so that the relative power of the strongest path becomes 0dB.

- Option 3: Merge taps after DS scaling and quantization –Huawei

### Issue 2: Channel model for FR2

Agreement: TDL models may not necessarily characterize the spatial FR2 propagation conditions for all scenarios.

Confirm Working assumption: Channel Model Option 1 will be used for FR2.

TDL Model

- TDL-A: 30 ns RMS delay spread

- FFS if additional models are needed

  - Option 1: TDL-C: {60-80} ns RMS delay spread

  - Option 2: TDL-D: [10-30] ns RMS delay spread

  - Other models are not precluded

Doppler spread

- Low speed: 75 Hz for simulation assumptions

- High speed: 300 Hz for simulation assumptions

Channel models of simulation alignment

- Model 1: TDL-A: 30 ns 75 Hz for CSI / PDCCH/ PDSCH/ PBCH

- Model 2: TDL-A: 30 ns 300 Hz for PDCCH/ PDSCH / PBCH

- Note: use channel models without simplifications for simulation alignment in RAN4 88bis

Channel models simplification

- The maximum number of taps: [12]

- 5 ns quantization grid (200MHz sampling frequency)

- Follow same procedure as for FR1 to generate the final channel models

Companies’ proposals:

| Intel | Proposal#1: Define UE demodulation requirements in FR2 with TDL-A-30ns. TDL-C-[60-80]ns and TDL-D-[10-30]ns shall not be used for defining UE demodulation requirements in FR2 Proposal #2: Use simplification method of choosing strongest paths contributing to 90% of total power for TDL-A |
| --- | --- |
| Huawei | Proposal: The simplified models described in Table 1 (TDL-A-FR2) will be used in SISO NR UE demodulation tests in FR2. |
| Ericsson | Proposal 1: RAN4 uses TDLC60 (TDL-C with DS=60ns) for UE demodulation requirements. Proposal 2: If RAN4 will specify the UE demodulation requirements under AWGN-like condition, RAN4 consider to uses TDLD10 (TDL-D with DS=10ns). |

- Option 1: Not introduced test cases with TDL-C and TDL-D (Intel)

- Option 2: Introduced channel mode TDL-C 60ns (Ericsson)

### Issue 3: High speed train scenarios

Previous RAN4 agreements:

Agreement: Use HST model as described on section B3 TS36.101 for FR1 for initial simulation purposes (max Doppler shift specified in table)

  - FFS introduction of new requirements

  - This topic will be discussed in agenda for PDSCH demodulation and other related agendas in the next meeting.

Companies’ proposals:

| NTT DoCoMo | Observation: Rel. 8 LTE high-speed tests are composed of followings. - High Doppler test to verify normal demodulation test - HST test to verify tracking performance of quick variation of Doppler shift Proposal: For FR1, HST test should be introduced with the maximum Doppler shift of 750 kHz. |
| --- | --- |

### Issue 4: the maximum number of emulated faders in Rel-15

FR1: 64

FR2: ?

# Draft TPs for TS 38.101-4

Pre-meeting email review and discussion status

| Items | Related TPs | Discussion Status | Open issues |
| --- | --- | --- | --- |
| Chapter 4 general (editor Ericsson) | TP for chapter 4 (Ericsson) TP for FR2 reference test points, SNR definition, test conditions (Anristu) | Reference test points and SNR definition potentially will be captured in this chapter. (Seems companies reach consensus for capture in this sections) Meanwhile companies still want to further check SNR definitions for the drafting TPs for FR2 | FR2 Test point, SNR definitions Remaining test configurations and test points |
| PDSCH requirements(5.2 and 7.2) | TP for FR1 PDSCH ( Intel) TP for FR2 PDSCH (QC) | Companies still need to align test parameters naming and format for introducing requirements, Detailed skeleton among FR1 and FR2 still to be aligned Intel volunteer to provide template for PDSCH parameters | Applicable rules for 2Rx,4Rx and R-ML receiver in general sub-clause (5.2.1,7.2.1) Test parameter template and sub-clause skeleton alignment Requirements |
| PDCCH requirements (5.3 and 7.3) | TP for FR1 PDCCH ( Huawei) TP for FR2 PDCCH (CATT) | NO discussion till now | Requirements |
| PBCH requirements (5.4, 7.4) | TP for FR1 PBCH ( Ericsson) TP for FR2 PBCH (Ericsson) | NO discussion till now | Necessity of 4Rx Requirements |
| SDR requirements (5.5, 7.5) | TP for FR1 SDR PDSCH ( Intel) TP for FR2 SDR PDSCH (QC) | No discussion through emails Further discussion foreseen i.e. detailed skeleton, procedure | Detailed test procedure, configurations and RMC |
| CQI requirements (6.2,8.2) | TP for FR1 CQI( Huawei) TP for FR2 CQI (Intel) | Template for parameters and subsections need to be aligned ZP CSI-RS configurations FFS CSI-IM configurations FFS | ZP CSI-RS configurations FFS CSI-IM configurations FFS Requirements and test points |
| PMI requirements (6.3,8.3) | TP for FR1 PMI( Samsung) TP for FR2 PMI (Samsung) | Template for parameters and subsections need to be aligned | Requirements Remaining test configurations and test points |
| RI requirements (6.4,8.4) | TP for FR1 RI( Samsung) TP for FR2 RI (Samsung) | Template for parameters and subsections need to be aligned | Requirements Remaining test configurations and test points |
| Interworking requirements (9,10) | TP for Demodulation( Huawei) TP for CSI (Samsung) | Detailed Skeleton alignment needed FFS for LTE carrier configurations Requirements applicable rules | Sub-clause skeleton LTE carrier configurations |
| Annex A FRC | TP for FRC (Intel) | NO much discussion till now Align naming for FRC and TDD DL-UL configurations still required CSI FRC still missing LTE FRC for interworking still missing | CSI FRC LTE FRC |
| Annex B Propagation | TP for Annex B (Huawei) | Discussion on channel model naming and MIMO correlation matrix, revised version shared taking above discussion into account Beam forming model still missing TDL channel PDPs still missing | Beamforming model TDL PDPs |
| Annex C and E | TP for Annex C (CATT) TP for Annex E (CATT) | Comments received and revised version shared with modifications |  |

## TPs for generic sections

| R4-1813443 | TP on performance specification 38.101-4 Chapter 4 general part | Ericsson |
| --- | --- | --- |
| R4-1813444 | TP on performance specification 38.101-4 Chapter 5~8 general part with applicability rules | Ericsson |

SNR definition for FR2

| R4-1812084 | Noc level, Band groups and Reference point for FR2 demod: 38.101-4 Text Proposal | ANRITSU LTD |
| --- | --- | --- |

## TPs for PDSCH

Normal

| R4-1812204 | TP to TS 38.101-4: FR1 PDSCH demodulation requirements (5.2) | Intel Corporation |
| --- | --- | --- |

| R4-1813530 | Draft TP on FR2 PDSCH Demodulation Performance Requirements | Qualcomm Europe Inc. (Spain) |
| --- | --- | --- |

SDR

| R4-1812205 | TP to TS 38.101-4: FR1 SDR requirements (5.5) | Intel Corporation |
| --- | --- | --- |

| R4-1813542 | Draft TP on FR2 PDSCH SDR Performance Requirements | Qualcomm Europe Inc. (Spain) |
| --- | --- | --- |

## TPs for PDCCH

FR1

| R4-1813633 | TP for introducing FR1 PDCCH requirements in TS 38.101-4 section 5.3 | Huawei, HiSilicon |
| --- | --- | --- |

FR2

| R4-1812275 | TP to TS38.101-4 Section 7.3: PDCCH demodulation requirements | CATT |
| --- | --- | --- |

## TPs for PBCH

| R4-1812851 | TP to TS 38.101-4: 5.4 FR1 PBCH demodulation requirements | Ericsson |
| --- | --- | --- |
| R4-1812852 | TP to TS 38.101-4: 7.4 FR2 PBCH demodulation requirements | Ericsson |

## TPs for CQI

FR1

| R4-1813634 | TP for introducing FR1 Reporting of Channel Quality Indicator (CQI) TS 38.101-4 section 6.2 | Huawei, HiSilicon |
| --- | --- | --- |

FR2

| R4-1812206 | TP to TS 38.101-4: FR2 CQI requirements (8.2) | Intel Corporation |
| --- | --- | --- |

## TPs for PMI

| R4-1812472 | TP for 38.101-4 section 6.3 FR1 PMI test cases | Samsung |
| --- | --- | --- |
| R4-1812473 | TP for 38.101-4 section 8.3 FR2 PMI test cases | Samsung |

## TPs for RI

| R4-1813543 | Draft TP on FR1 Rank Indication Reporting Performance Requirements | Qualcomm Europe Inc. (Spain) |
| --- | --- | --- |
| R4-1813561 | Draft TP on FR2 Rank Indication Reporting Performance Requirements | Qualcomm Europe Inc. (Spain) |

## TPs for interworking

Demodulation

| R4-1813635 | TP for introducing demodulation performance requirements for interworking TS 38.101-4 section 9 | Huawei, HiSilicon |
| --- | --- | --- |

CSI

| R4-1812474 | TP for 38.101-4 section 10 CSI test cases of interworking | Samsung |
| --- | --- | --- |

## TPs for annex A FRC

| R4-1812207 | TP to TS 38.101-4: Annex A Measurement channels | Intel Corporation |
| --- | --- | --- |

## TPs for Annex B propagation

| R4-1812328 | TP on channel models for TS38.101-4 | Huawei Technologies Sweden AB |
| --- | --- | --- |
| R4-1812334 | TP for introducing propagation conditions (Annex B) of TS 38.101-4 | Huawei Technologies Sweden AB |

## TPs for Annex C and E

| R4-1812276 | TP to TS38.101-4 Annex C: Downlink physical channels | CATT |
| --- | --- | --- |

| R4-1812277 | TP to TS38.101-4 Annex E: Environmental conditions | CATT |
| --- | --- | --- |