3GPP TSG-RAN WG4 Meeting #88bis	R4-1813775

Chengdu, CN, 8th – 12th Oct 2018

Source: 	Nokia, TMO US

Title: 					TP to TR 37.716-21-11: Introduction of DC_66A-(n)71AA and DC_2A-(n)71AA

Agenda Item: 			9.4.2

Document for:	Approval

# 1	Introduction

This contribution covers these three-band EN-DC configurations to TR 37.716-21-11:

- DC_66A-(n)71AA_UL_66A_n71A

- DC_2A-(n)71AA_UL_2A_n71A

************************** Start of TP ************************************************

### 5.1.x	DC_66A-(n)71AA

### 5.1.x.1	Operating bands for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_66-(n)71 | CA_66-71 | n71 | No1 |
| NOTE 1: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. |  |  |  |

### 5.1.x.2	Configurations for EN-DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_66A-(n)71AA | DC_66A_n71A | CA_66A-71A | n71A |

### 5.1.x.3	∆TIB and ∆RIB values

∆TIB values for DC_66_n71 are already specified in TS 38.101-3 and ∆RIB are zero.

### 5.1.x.4	REFSENS requirements

No REFSENS exceptions needed for DC_66A-(n)71AA. Impact of contiguous intraband EN-DC operation on band 71 on REFSENS is already specified in Table 7.3B.2.1-1 of TS 38.101-3.

************************** Next configuration ************************************************

### 5.1.x	DC_2A-(n)71AA

### 5.1.x.1	Operating bands for EN-DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-(n)71 | CA_2-71 | n71 | No1 |
| NOTE 1: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. |  |  |  |

### 5.1.x.2	Configurations for EN-DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-(n)71AA | DC_2A_n71A | CA_2A-71A | n71A |

### 5.1.x.3	∆TIB and ∆RIB values

∆TIB values for DC_2_n71 are already specified in TS 38.101-3 and ∆RIB are zero.

### 5.1.x.4	REFSENS requirements

REFSENS exceptions needed for DC_2A-(n)71AA due to band 71 uplink harmonic into band 2 is already specified in Table 7.3B.2.3.1-1 of TS 38.101-3. Impact of contiguous intraband EN-DC operation on band 71 on REFSENS is already specified in Table 7.3B.2.1-1 of TS 38.101-3.

************************** End of TP********** ************************************************