3GPP TSG-RAN WG4 Meeting #88bis	R4-1813932

Chengdu, China, 8th – 12th Oct, 2018

Agenda item:    7.13.1.3

Source: 	     Intel Corporation

Title: 	               Simulation results for PDCCH

Document for: Discussion

# Introduction

In RAN4#88 PDCCH demodulation requirements were discussed and the following were agreed in the way forward [1].

- Aggregation levels - AL2, AL4, AL8 - FFS for AL16 - See page 6 for evaluation of PDCCH AL16 scenario. - Applicability of 4Rx PDCCH demodulation requirements - Using similar test condition as LTE, 4Rx NR PDCCH demodulation performance tests only if there are continuous PDSCH grants in every slot. - REG Bundle size for 4Rx PDCCH test cases - Agreement in AH1807: REG bundle size =6 for non-interleaved, 2 for interleaved - Companies are encouraged to bring evaluation results with both REG bundle size 2 and 6 for 4Rx test cases (case 6,7 and 8) in RAN4#88bis, and decide which value is set based on performance comparison. - Test configurations for 1 symbol PDCCH test case(s) - Introduce additional requirements with 1 symbol control channel for FR1 FDD with 10MHz and 15kHz SCS (for LTE-NR coexistence scenario) - See page 5 for the detailed parameters for evaluation - TRS configuration: following same configuration PDSCH demodulation test cases

In RAN4#88 simulation assumptions for initial alignment were also agreed. In this contribution we provide simulation results for PDCCH test cases.

# Simulation Results

## Simulation results with 2Rx

For the testcases agreed in [1], we provide the results for alignment for SNR at 1% Pm-dsg for 2Rx:

Table 1: PDCCH Results with 2Rx

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 6.14 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | -0.91 |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x2 Low | 5.68 |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 3.73 |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | -4.29 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x2 Low | 5.60 |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 1x2 Low | 1.48 |
| 8 | 30 | 1_1 | 90 | 52 | 1 | 8 | Interleaved | 6 | TDL-C, 300ns, 100Hz | 2x2 Low | -4.06 |
| 9 | 120 | 1_0 | 60 | 40 | 1 | 2 | Interleaved | 2 | [TDL-A, 30ns, 75Hz] | 1x2 Low | 4.16 |
| 10 | 120 | 1_1 | 60 | 51 | 1 | 4 | Interleaved(1) | 6 | [TDL-A, 30ns, 300Hz] | 1x2 Low | -0.50 |
| 11 | 120 | 1_1 | 60 | 51 | 1 | 8 | Interleaved | 2 | [TDL-A, 30ns, 75Hz] | 2x2 Low | -2.64 |
| Note1: Interleaver size=2 |  |  |  |  |  |  |  |  |  |  |  |

Results for testcases with 1 symbol PDCCH:

Table 2: Results with 1 symbol PDCCH with 2Rx

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12a | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | 1.98 |
| 12b | 15 | 1_0 | 48 | 39 | 1 | 8 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | -1.77 |
| 13a | 15 | 1_1 | 48 | 50 | 1 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 2x2 Low | 0.45 |
| 13b | 15 | 1_1 | 48 | 50 | 1 | 8 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 2x2 Low | -2.71 |

Results for testcases with AL=16

Table 3: PDCCH Results with AL=16 for 2Rx

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x2 Low | -3.99 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x2 Low | -6.64 |
| 16 | 120 | 1_0 | [60] | 40 | 2 | 16 | Interleaved | 2 | TDL-A, 30ns, 75Hz | 2x2 Low | -5.64 |

## Simulation results with 4Rx

For the testcases agreed in [1], we provide the results for alignment for SNR at 1% Pm-dsg for 4Rx:

Table 4: PDCCH Results with 4Rx

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15 | 1_0 | 24 | 39 | 2 | 2 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low | 0.12 |
| 2 | 15 | 1_0 | 24 | 39 | 2 | 4 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x4 Low | -4.69 |
| 3 | 15 | 1_1 | 24 | 50 | 2 | 2 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x4 Low | 0.44 |
| 4 | 15 | 1_1 | 48 | 50 | 2 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low | -2.04 |
| 5 | 15 | 1_1 | 48 | 50 | 2 | 8 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 2x4 Low | -7.38 |
| 6 | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 2 | TDL-A, 30ns, 10Hz | 1x4 Low | 3.50 |
| 6a | 30 | 1_0 | 102 | 41 | 1 | 2 | Interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low | -1.09 |
| 7 | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 2 | TDL-C, 300ns, 100Hz | 1x4 Low | -0.09 |
| 7a | 30 | 1_1 | 102 | 52 | 1 | 4 | Interleaved | 6 | TDL-C, 300ns, 100Hz | 1x4 Low | -4.20 |
| 8 | 30 | 1_1 | 90 | 52 | 1 | 8 | Interleaved(1) | 6 | TDL-C, 300ns, 100Hz | 2x4 Low | -7.30 |
| Note1: Interleaver size=2 |  |  |  |  |  |  |  |  |  |  |  |

Results for testcases with 1 symbol PDCCH:

Table 5: Results with 1 symbol PDCCH with 4Rx

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12a | 15 | 1_0 | 48 | 39 | 1 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low | -3.46 |
| 12b | 15 | 1_0 | 48 | 39 | 1 | 8 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low | -6.46- |
| 13a | 15 | 1_1 | 48 | 50 | 1 | 4 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 2x4 Low | 4.21 |
| 13b | 15 | 1_1 | 48 | 50 | 1 | 8 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 2x4 Low | -6.55 |

Results for testcases with AL=16:

Table 6: PDCCH Results with AL=16 for 4Rx

| Test No. | SCS(KHz) | Format | CORESET RB | Payload | CORESET time duration | AL | CCE-to-REG Mapping | REG bundle size | Prop. condition | Antenna config | SNR @ 1% Pm-dsg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 15 | 1_0 | 48 | 39 | 2 | 16 | Non-interleaved | 6 | TDL-A, 30ns, 10Hz | 1x4 Low | -8.20 |
| 15 | 30 | 1_0 | 48 | 41 | 2 | 16 | Non-interleaved | 6 | TDL-C, 300ns, 100Hz | 1x4 Low | -9.59 |

# Conclusion

In this contribution we present the simulation results for PDCCH for the alignment testcases.

# Reference

- R4-1811724, “Way forward on NR PDCCH demodulation requirements”, Ericsson