3GPP TSG-RAN WG4 Meeting #88bis                	R4-1814046

Chengdu, China, 08 - 12 October 2018

Source:	China Telecom

Title:	Ad hoc minutes #2 for NR BS demodulation

Agenda Item:	7.13.2.1

Document for:	Discussion

Discussions

Issue 1: Propagation condition

Propagation conditions used in UE demodulation (refer to R4-1811723 agreed in Aug):

- FR1

  - TDL-A with Delay spread RMS=30ns; 10 Hz max Doppler frequency

  - TDL-C with Delay spread RMS=300ns; 100 Hz max Doppler frequency

  - TDL-B with Delay spread RMS=100ns; 400 Hz max Doppler frequency

- FR2

  - TDL-A with Delay spread RMS=30ns; 75 Hz max Doppler frequency

  - TDL-A with Delay spread RMS=30ns; 300 Hz max Doppler frequency

Open issues:

Propagation condition for PUSCH

- Propagation condition for FR1 1Tx PUSCH:

  - MCS 2: TDL-A 30ns, 10Hz, [TDL-C 300ns, 100 Hz], [TDL-B 100ns, 400 Hz]

  - MCS 16: TDL-A 30ns, 10Hz, [TDL-C 300ns, 100 Hz], [TDL-B 100ns, 400 Hz]

  - MCS 20: TDL-A 30ns, 10Hz, [TDL-C 300ns, 100 Hz], [TDL-B 100ns, 400 Hz]

- Propagation condition for FR1 2Tx PUSCH:

  - MCS 2: TDL-A 30ns, 10Hz, [TDL-C 300ns, 100 Hz], [TDL-B 100ns, 400 Hz]

  - MCS 16: TDL-A 30ns, 10Hz, [TDL-C 300ns, 100 Hz], [TDL-B 100ns, 400 Hz]

  - MCS 20: TDL-A 30ns, 10Hz, [TDL-C 300ns, 100 Hz], [TDL-B 100ns, 400 Hz]

- Propagation condition for FR2 1Tx PUSCH:

  - MCS 2: [TDL-A 30ns, 75Hz], [TDL-A 30ns, 300Hz], [...]

  - MCS 16: [TDL-A 30ns, 75Hz], [TDL-A 30ns, 300Hz], [...]

  - MCS 20: [TDL-A 30ns, 75Hz], [TDL-A 30ns, 300Hz], [...]

- Propagation condition for FR2 2Tx PUSCH:

  - MCS 2: [TDL-A 30ns, 75Hz], [TDL-A 30ns, 300Hz], [...]

  - MCS 16: [TDL-A 30ns, 75Hz], [TDL-A 30ns, 300Hz], [...]

  - MCS 20: [TDL-A 30ns, 75Hz], [TDL-A 30ns, 300Hz], [...]

Propagation condition for PUCCH

- Propagation condition for FR1 PUCCH:

  - TDL-A 30ns, 10Hz, [TDL-C 300ns, 100 Hz], [TDL-B 100ns, 400 Hz]

- Propagation condition for FR2 PUCCH:

  - [TDL-A 30ns, 75Hz], [TDL-A 30ns, 300Hz], [...]

Propagation condition for PRACH

- Propagation condition for FR1 PRACH:

  - AWGN, [TDL-A 30ns, 10Hz], [TDL-C 300ns, 100 Hz], [TDL-B 100ns, 400 Hz]

- Propagation condition for FR2 PRACH:

  - AWGN, [TDL-A 30ns, 75Hz], [TDL-A 30ns, 300Hz], [...]

Propagation condition for PUSCH

- Propagation condition for FR1 1Tx PUSCH:

  - Pick one Propagation condition for each MCS (Ericsson, Huawei, ZTE)

Agreements:

- Propagation condition for FR1 1Tx and 2Tx PUSCH:

  - MCS 2: TDL-B 100ns, 400 Hz

  - MCS 16: TDL-C 300ns, 100 Hz

  - MCS 20: TDL-A 30ns, 10Hz

- Propagation condition for FR1 PUCCH:

  - TDL-C 300ns, 100 Hz

  - FFS: TDL-A 30ns, 10Hz and TDL-B 100ns, 400 Hz

- Propagation condition for FR1 PRACH:

  - AWGN, TDL-C 300ns, 100 Hz

Discuss the Propagation condition for FR2 in this week.

Issue 2: Simulation campaign

Open issues:

- For PUSCH CP-OFDM and PUCCH, prioritize the following combinations of channel BW & SCS for simulation towards the Nov meeting:

  - FR1: 10MHz/15kHz; 40MHz/30kHz

  - FR2: 100MHz/60kHz; 100MHz/120kHz

- Both ideal and impairment results to be provided for the Nov meeting

  - Aim to define tentative requirements in [] for some of the agreed cases

Agreements:

- For PUSCH CP-OFDM and PUCCH, towards the Nov meeting, companies are encouraged to provide results for all the agreed combinations of channel BW & SCS, and prioritize the following combinations if time is not enough to run simulation for all the combinations:

  - FR1: 10MHz/15kHz; 40MHz/30kHz

  - FR2: 100MHz/60kHz; 100MHz/120kHz

- Both ideal and impairment results to be provided for the Nov meeting

  - Aim to define tentative requirements in [] for some of the agreed cases

WFs

- WF on PUSCH

  - To list simulation cases

- WF on PUCCH

  - To update propagation channel?

Agreements:

- For PUSCH

- PTRS configuration:

  - Default thresholds for PT-RS patterns for DFT-S-OFDM

    - NRB0=0, NRB1=8, NRB2=NRB3=32, and NRB4=108

- For simulation assumptions, the Rx number for FR1 DFT-S-OFDM

  - 2Rx, 4Rx, 8Rx

- For simulation assumptions, the Tx number for FR2 CP-OFDM

  - 1Tx and 2Tx

Draft CRs and TPs

Issue 1: 38.104 Draft CRs

  - General and skeleton (Ericsson)

  - PUSCH CP-OFDM in FR1 (Nokia)

  - PUCCH format 3 and 4 (Huawei)

  - PRACH (CATT)

Agreements:

- Common:

  - Use the short name for propagation condition (to be shared by Huawei)

  - Terminology for Rx antenna: use “Rx antenna” in this meeting

- PUCCH:

  - Separate sub-clauses under PUCCH format 1 and [2] for ACK miss detection and NAK to ACK

  - Have a sub-cluase in TS 38.104:  8.x.x	DTX to ACK

Issue 2: 38.141 TP

Agreements:

  - AWGN power level for conducted test

    - Use the following table from R4-1812773

Double check the numbers in [] in the next meeting

| Sub-carrier spacing (kHz) | Channel bandwidth [MHz] | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5MHz |
|  | 10 | [-80.3] dBm / 9.36MHz |
|  | 40 | [-74.1] dBm / 38.88MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64MHz |
|  | 20 | [-77.4] dBm / 18.36MHz |
|  | 40 | [-74.2] dBm / 38.16MHz |
|  | 100 | [-70.1] dBm / 98.28MHz |

Issue 3: Status and plan for draft CR/TPs

- Green: provided in this meeting, and can be updated in the next meeting to capture new agreements

- Blue: provided in this meeting, but was not completed

- Red: to be provided in next meeting

- For the next meeting, companies to distribute the draft CRs/TPs one week before the tdoc submission deadline.

|  |  | 38.104 |  | 38.141-1 | 38.141-2 |
| --- | --- | --- | --- | --- | --- |
|  |  | Conducted and radiated, FR1 | Radiated, FR2 | Conducted, FR1 | Radiated, FR1 and FR2 |
| General & skeleton |  | Ericsson | Ericsson | Huawei | Huawei |
| PUSCH | CP-OFDM | Nokia | Ericsson | Nokia | Nokia, Ericsson |
|  | DFT-S-OFDM | China Telecom | China Telecom | China Telecom | China Telecom |
| PUCCH | format 0 | Ericsson | Ericsson | Ericsson | Ericsson |
|  | format 1 | ZTE | ZTE | ZTE | ZTE |
|  | format 2 | Samsung | Samsung | Samsung | Samsung |
|  | format 3 & 4 | Huawei | Huawei | Huawei | Huawei |
| PRACH |  | CATT | CATT | CATT | CATT |
| Annex | FRC | China Telecom | China Telecom | China Telecom | China Telecom |
|  | Propagation conditions | Huawei | Huawei | Huawei | Huawei |
|  | Measurement system set-up and TT | N.A. | N.A. | China Telecom | Huawei |
|  | General rules for statistical testing | N.A. | N.A. | Nokia | Nokia |