3GPP TSG-RAN WG4 Meeting #88bis 	R4-1814085

# Chengdu, Sichuan, China, 8th – 12th October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0056 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-2: adding TRP measurement procedures in Annex F. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ercisson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-10-8 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TRP measurement procedures which agreed at the last RAN4 #88 meeting, were captured in TR 37.843 but not in TS 37.145-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This is a revision of R4-1813541. TRP measurement procedures for 2 cut with pattern multiplcation, 2 or 3 cut with dense sampling, full sphere with sparse sampling, beam-based directions, peak method, equal sector with peak average and pre-scan are included in the annex. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not all TRP measurement procedures are included in the TS 37.145-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | N | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

Annex F (normative): 
TRP measurement grids

# F.1		General

The annex describes various sampling grids and procedures for OTA TRP performance of AAS BS.

# F.2	Spherical equal angle grid

## F.2.1	General

TRPEstimate is defined as

TRPEstimate=π2NMn=0N-1m=0M-1EIRP(θn,ϕm)sinθn

when EIRP measurements is used or as

TRPEstimate=4πd2π2NMn=1N-1m=0M-1PD(θn,ϕm)sinθn

when power density measurements are used, and d is the test distance. N and M are the number of samples in the θ and ϕ angles. Each (θn,ϕm) is a sampling point. The sampling angular intervals for θ and ϕ angles are Δθ=πN and Δϕ=2πM. The sampling intervals Δθ and Δϕ are described in F.2.2.

## F.2.2	Reference angular step criteria

The reference angular steps Δϕ and Δθ in are radians are defined as

Δϕref=min⁡(λDcyl,[π12])

Δθref=⁡(λD,[π12])

Dcyl and D are calculated as

Dcyl=d2+w2

D=d2+w2+h2

The definition of d, w and h is shown in Figure F2.2-1. The radiation source can be EUT antenna array or the whole of EUT.

Figure F2.2-1: Dimensions of a radiation source: depth (d), width (w) and height (h)

In the case of Uniform Linear Array (ULA)  and the EUT is mounted along the yz plane as shown in Figure F2.2-2, the  reference angular step can be determined by

∆θref=180°πarcsin⁡(λ/Dz)

Δϕref=180°πarcsin⁡(λDy)

Where Dy is the length of radiating parts of EUT along y-axis, Dz is the length of radiating parts of EUT along the z-axis and  is wavelength for the measured frequency.

Figure F2.2-2. Spherical coordinate for OTA conformance testing of EUT

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor )of the grid is defined as

Where  and  are the actual angular steps used in the measurement.

# F.3	Spherical equal area grid

TRPEstimate is defined as

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

N is the total number of samples and specified as

N ≥4π∆θ∆ϕ.

The sampling intervals Δθ and Δϕ are described in F.2.2. Each (θn,ϕn) is a sampling point.

# F.4 	Spherical Fibonacci grid

TRPEstimate is defined as

TRPEstimate=1Ii=1IEIRP(θi,ϕi)

I is the total number of samples and specified as

N ≥4π∆θ∆ϕ

The sampling intervals Δθ and Δϕ are described in F.2.2. Each (θi,ϕi) is a sampling point.

# F.5 Orthogonal  cut grid

EIRPav,cut-n=1Pj=1PEIRP(j)

TRPEstimate=1Nn=1NEIRPav,cut-n

Here, at least two cuts (default) shall be used, an optional third cut can be used. The alignment of the cuts must be along the symmetry planes of the antenna array. No alignment is required for spurious emissions.

When alignment is required:

- The first mandatory cut is a horizontal cut passing through the peak direction of the main beam.

- The second mandatory is a vertical cut passing through the peak direction of the main beam. Using the data from these two mandatory cuts, a conditional pattern multiplication can be used.

- The third optional cut is a vertical cut orthogonal to the first and the second cut.

When alignment is not required, the cuts can be aligned arbitrarily.

Once the number and the orientation of the cuts are decided, the total EIRP is measured on the orthogonal cuts and the TRP is then calculated as follows: First the contributions from each cut is calculated as

EIRPav,cut-n=1Pj=1PEIRP(j)

where P is the number of sampling points in the cut. The final contribution for all cuts is calculated as

TRPEstimate=1Nn=1NEIRPav,cut-n

where N is the number of cuts. Note that when orthogonal cuts are measured, the intersection points are measured multiple times and the repeated values can be removed from the samples before averaging.

When two cuts measurements are used, a conditional pattern multiplication can be applied. The following are the conditions for applying pattern multiplication:

    - The vertical cut (and the main beam) is in the xz -plane

    - The frequency of the emission is within the downlink operating band.

    - The bandwidth of the emission is the same as the bandwidth of the in-band modulated signal

    - The emission appears/disappears when the Tx power is turned on/off.

    - The antenna arrays of the EUT

      - Have rectangular grids of antenna element positions

      - Have symmetry planes that are vertical and horizontal.

      - Have parallel antenna planes

The antenna array is here assumed to be placed in the yz-plane. The pattern multiplication is performed in uv-coordinates and the data in the two cuts are denoted EIRPcut1ϕ at θ=θH and a vertical cut with data EIRPcut2θ at ϕ=0. The data is split in two parts corresponding to the forward and backward hemispheres. The uv-coordinates are the projections of the angular directions onto the antenna plane, here the yz-plane. Using the spherical coordinates as depicted in Fig. F.1.2.-1 the u and v coordinates are defined as

u=sinθsinϕv=cosθ

Note that only the data on the cuts are measured.

Calculate power density/EIRP values outside the two cardinal cuts as

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

The pattern multiplication is applied separately for the forward (fwd) and backward (bwd) hemisphere. The TRP is then calculated as

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

Note: the numerical singularity at u2+v2=1 must be treated with care, e.g. by change of variables.

# F.6 Wave vector space grid

If EUT is mounted along the yz plane as shown in Figure F2.2-1, ,the reference step in wave vector space can be determined by

Δugrid=λDy

Δvgrid=λDzwhere Dy is the length of radiating  parts of EUT along  y-axis, Dz is the length of radiating parts of EUT along the z-axis.

According to the relationship between the normalized wave vector and spherical coordinate, the wave vector can be  represented as following:

u=sinθsinϕ, v=cos⁡(θ)

The total radiated power (TRP) in the wave vector space is determined by

TRP=ΔugridΔvgrid4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

Where due to practical reasons such as time constraints or turn-table precision, measurement with the reference steps is not practical, sparser grids can be used. Use of sparse grids can lead to errors in TRP assessment. In order to characterize these errors, the SF (sparsity factor)of the grid is defined as

Where  and  are the actual steps used in the wave vector space in the measurement.

# F.7 Orthogonal 2 cuts with pattern multiplication

This method can be used when the antenna symmetries are compatible with pattern multiplication, see Subclause F.1.2.4. The procedure is as follows:

- Calculate the reference angular steps as described in Subclause F.1.2.

- Align the EUT to allow for proper pattern multiplication. See Subclause F.5. Measure EIRP on two orthogonal cuts with steps smaller or equal to the reference steps according to step 1.

- Apply pattern multiplication according to Subclause F.5 to extrapolate the two cuts data to full-sphere.

- Apply numerical integration to obtain the TRP estimate as described in Subclause F.5.

# F.8 Orthogonal 2 or 3 cut with dense sampling

# The procedure is as follows:

- Follow steps described in F.5 and calculate the TRP estimate.

- Add the appropriate correction factor ΔTRP according to Table F.8-1 to ensure overestimation with 95% confidence.

- Compare the (TRP estimate + ΔTRP) to the limit.

- If the (TRP estimate + ΔTRP) is above the limit, perform the measurement on an additional third cut and repeat steps 1 to 3.

|  | Three cuts | Two cuts |
| --- | --- | --- |
| Correction factor ΔTRP (dB) | [2.0] | [2.5] |

Table F.8-1: The correction factor for two or three cuts dense sampling.

# F.9 Full sphere with sparse sampling

The procedure is as follows:

- Set the angular grid:

  - Non-harmonic frequencies: choose the angular steps Δϕ and Δθ smaller than or equal to [15] degrees. Calculate the sparsity factor (SF) as

SF=maxΔϕΔϕref,ΔθΔθref

and the correction factor as:

ΔTRP=SF-1SFmax-1⋅1.0 dB,

where SFmax corresponds to 15 degrees angular step. If the sparsity factor is smaller than 1, the correction factor ΔTRP is 0 dB.

  - Harmonic frequencies with fixed beam test signal: choose the angular steps smaller than or equal to the reference angular steps Δϕref and Δθref. Correction factor ΔTRP is 0 dB.

  - [Harmonic frequencies with beam sweeping test signal: set the angular steps to [15] degrees. Correction factor is ΔTRP 0 dB].

- Apply a suitable numerical integration to calculate the TRP estimate.

- Add the appropriate correction factor ΔTRP according to step 1 to ensure an overestimation with 95% confidence.

- Compare the (TRP estimate + ΔTRP) with the limit. If the (TRP estimate + ΔTRP) is above the limit, choose a smaller angular step and repeat steps 2-4. If the sparsity factor is less than one, no significant improvement of accuracy is expected.

# F.10 Beam-based directions

Beam-based direction can be used if directivity of the EUT antenna is known for the base station operating band. TRPEstimate is defined as

TRPEstimate=EIRPpeakDEUT, where EIRPpeak is the maximum EIRP in the beam peak direction within a particular beam direction pair and DEUT is directivity of the EUT antenna.

# F.11 Peak method

# The peak method can be used when frequencies with unwanted peak emissions are identified during pre-scan. The method does not provide an estimate of TRP.

# For each peak emission frequency identified during pre-scan, measure peak EIRP or power density as follows:

# Move EUT and test antenna to the same position where the peak emission is recorded during the pre-scan.

# Move the EUT around the position and test antenna orientation to find the final peak EIRP or power density.

# The measured peak power density or EIRP shall be used to demonstrate conformance.

NOTE:	Peak EIRP is the linear sum of two orthogonal polarised components.

# F.12 Equal sector with peak average

# Equal sector with peak average can be performed on frequencies with unwanted peak emission, which are considered by the peak method for further measurements.

# The spherical angle ϕ is divided into K equal sectors. If the largest dimension of EUT is less than 60 cm, then each sector is a half quadrant of 45°.

For each peak emission frequency, measure peak EIRP of beams belonging to different sectors of the sphere as follows:

# Move EUT and test antenna to the same position where the emission peak is recorded during the pre-scan.

# Move EUT around the position and test antenna orientation to find the final peak EIRP.

# Repeat Steps 1 to 2 until all sectors are covered.

# Calculate TRPEstimate as

# TRPEstimate=1Kk=1KEIRPk, where EIRPk is the peak EIRP in the kth sector.

# NOTE:	Peak EIRP is the linear sum of two orthogonal polarised components.

# F.13 Pre-scan

# Pre-scan is used to identify frequencies with unwanted emission power levels above a certain threshold. The pre-scan does not provide an estimate of TRP.

# The procedure for pre-scan is as follows:

# Scan the entire surface around EUT.

# Rotate test antenna to cover all possible polarisations of emissions to detect maximum emissions.

# Record the list of frequencies and corresponding unwanted emission power levels, EUT spatial positions, and test antenna polarization for which the maximum emission levels occur.

# Emissions which are [20 dB] or more below the specified limit shall not require further measurements.

<End of modified section>