3GPP TSG-RAN Working Group 4 Meeting #88-Bis	R4-1814101

Chengdu, China, 8th October – 12th October 2018	(revision of R4-1813455)

Source:	Nokia, Globalstar

Title:	TP to TR 36.791 (E-UTRA 2.4 GHz TDD Band for US): In-Device Coexistence Considerations for Unlicensed Operations in the 2.4 GHz ISM Band

Document for:	Approval

Agenda Item:	          6.2.1

## 1	Introduction

The FCC has recently adopted rules for terrestrial use of the 2.4 GHz MSS band.  This band is authorized for TDD

operation and spans 11.5 MHz from 2483.5 MHz to 2495 MHz.  A new work item was created in RAN#79, to create

specifications for a corresponding new E-UTRA operating band [1].

The WI has a study phase, with the following core objectives:

- Define impact on 2.4 GHz Bluetooth operations

- Define impact on 2.4 GHz 802.11 operations

- Address potential Bluetooth and 802.11 coexistence issues

A skeleton TR 36.791 (E-UTRA 2.4 GHz TDD Band for US) was approved in RAN4#88 [2]. Way Forward for the completion of the study phase of the WI was agreed in RAN4#88, with the following item on In-Device Coexistence: [3]

- GlobalStar shall evaluate IDC impact when a LTE TDD UE operating in the 2483.5-2495 MHz is collocated with a BT/BLE/WiFi device.

The present contribution provides a TP to TR 36.791 on in-device co-existence considerations for unlicensed operation in the 2.4 GHz ISM Band, according to the agreed WF [3].

## 2	References

[1]	  RP-180583, New WI Proposal for E-UTRA 2.4 GHz TDD Band, Globalstar, RAN#79

[2]	  3GPP TR 36.791 “E-UTRA 2.4 GHz TDD Band for US”

[3]	Way Forward for impact analysis on Bluetooth and IEEE 802.11 operation for the new TDD E-UTRA band proposed by GlobalStar, RAN4#88, CableLabs, Broadcom

<<< Start of TP >>>

# 2	References

[1]	  3GPP TR 21.905 “Vocabulary for 3GPP Specifications.”

[2]	Terrestrial Use of the 2473[2483.5]-2495 MHz Band for Low-Power Mobile Broadband Networks; Amendments to Rules for the Ancillary Terrestrial Component of Mobile Satellite Service Systems, IB Docket No. 13-213, Report and Order, FCC 16-181

[X1]	3GPP TR 36.816, Study on signaling and procedure for interference avoidance for in-device coexistence
(Release 11)

[X2]	Federal Communications Commission Report TR 15-1002 (IB Docket No. 13-213), Electromagnetic Emissions Characterization of Samples Used at TLPS Demonstration, May 7, 2015

[X3]	R2-114331, Solutions for IDC interference in LTE + BT voice scenario, Ericsson, ST-Ericsson

[X4]	R2-115088, DRX for avoiding IDC interference with TDD configuration 6, Ericsson, ST-Ericsson

<<< Unmodified section skipped >>>

## 6.5.6	Analysis of In-Device Coexistence Impacts of 10 MHz LTE Emission Upon ISM Band Technologies

#### 6.5.6.1	General

This clause presents frequency and time domain analysis of In-Device Coexistence (IDC) within a UE incorporating 2.4 GHz TDD LTE radio and Wi-Fi / Bluetooth 2.4 GHz ISM band radio(s).

3GPP TR 36.816 [X1] presents the study results on IDC interference, and potential solutions to address the impact. The usage scenarios identified in [X1] are:

1a) LTE + BT earphone (VoIP service)

1b) LTE + BT earphone (Multimedia service)

2) LTE + WisFi portable router

3) LTE + WiFi offload

4) LTE + GNSS Receiver

No issues with GNSS Receiver are expected due to the large frequency separation to the satellite receiver frequencies (<1600 MHz). Sufficient coexistence has been demonstrated with LTE bands significantly closer than the 2.4 GHz E-UTRA TDD band, hence no study has been done.

#### 6.5.6.2	Frequency Domain Analysis

##### 6.5.6.2.1	General

Similar as in [X1], for the IDC interference with Bluetooth and Wi-Fi, the assumptions are made:

Table 6.5.6.2-1: IDC assumptions for the RF analysis

| Parameter | Value |
| --- | --- |
| LTE Band | 2483.5-2495 MHz |
| LTE center frequency | 2490 MHz |
| ISM technology considered | BT, WLAN |
| Interference directions considered | LTE to BT/WLAN; BT/WLAN to LTE |
| Interference mechanisms considered | Spurious emission and blocking |
| Filter | FBAR |
| Antenna Isolation | 12 dB |
| LTE Tx power | 23 dBm |
| WLAN Tx power | 20 dBm |
| BT Tx power | 20 dBm |
| LTE RSSI (as victim) | -94 dBm |
| WLAN RSSI | -79 dBm |
| BT RSSI | -90 dBm |
| LTE Bandwidth | 10 MHz |
| WLAN Bandwidth | 22 MHz |
| BT Bandwidth | 1 MHz |
| Performance measure | Desensitization (in dB) |

The interference mechanisms considered are both spurious emissions (TX leakage) and receiver blocking.

The LTE band filter is according to the filter topology 1 as outlined in clause 6.5.1. Based on initial studies, this is the worst of the two topologies overall for coexistence purposes. The ISM band filter does not provide selectivity on the considered LTE frequency band, i.e. has a flat frequency response in this context. 4 dB insertion loss across the passband has been used for filters of all radio technologies.

The victim receiver is assumed to be in the max gain mode, and 9 dB Noise Figure has been assumed. The effect of TX leakage is evaluated against this noise level. For receiver blocking, a 3-stage cascaded amplifier noise model has been assumed (the stages being RF filter, LNA, and the rest of the RX chain). At max gain, the model has 9 dB NF, and the LNA gain is assumed to be lowered in the presence of a strong blocking signal in order to prevent RF front-end overload. The actual blocking performance depends on receiver implementation and gain control algorithms.

##### 6.5.6.2.2	LTE to ISM Interference

The LTE TX leakage is based on RAN4 minimum RF performance simulation as shown in clause 6.5.1.

Table 6.5.6.2-2: IDC impact from LTE TX leakage to ISM receiver noise level

| LTE->ISM | Wi-Fi |  |  | Bluetooth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ch#11 | Ch#6 | Ch#1 | 2480 | 2460 | 2440 | ≤2430 |
| TX leakage power | -27 dBm/ 22 MHz | -52 dBm/ 22 MHz | -75 dBm/ 22 MHz | -16 dBm/ MHz | -60 dBm/ MHz | -77 dBm/ MHz | -88 dBm/ MHz |
| Antenna isolation | 12 dB | 12 dB | 12 dB | 12 dB | 12 dB | 12 dB | 12 dB |
| TX leakage at ISM input | -39 dBm/ 22 MHz | -64 dBm/ 22 MHz | -89 dBm/ 22 MHz | -28 dBm/ MHz | -72 dBm/ MHz | -89 dBm/ MHz | -100 dBm/ MHz |
| ISM input-referred noise | -91 dBm/ 22 MHz | -91 dBm/ 22 MHz | -91 dBm/ 22 MHz | -105 dBm/ MHz | -105 dBm/ MHz | -105 dBm/ MHz | -105 dBm/ MHz |
| ISM noise level increase | 52 dB | 27 dB | 4.1 dB | 77 dB | 33 dB | 16 dB | 6.2 dB |

Regarding receiver blocking, the ISM receiver desense can be estimated as follows:

Table 6.5.6.2-3: IDC impact of ISM receiver blocking

| LTE->ISM | Wi-Fi/BT Any |
| --- | --- |
| LTE TX power | 23 dBm |
| Antenna isolation | 12 dB |
| LTE blocking power at ISM input | 11 dBm |
| LTE blocking power at LNA input | 7 dBm |
| LNA gain to prevent mixer from saturating (assuming 0 dBm peak output power and 10 dB peak-to-average power ratio) | -17 dB |
| Effect to RX NF due to AGC | >28 dB |

The NF effect is based on the assumption, that the LNA-mixer saturation is largely independent on frequency offset, as the 2.4 GHz E-UTRA band is practically inside the RF input passband. The LNA gain is lowered in order to prevent saturation in the following receiver stages, and this increases the front-end Noise Figure.

As a conclusion, the IDC interference from LTE10 uplink transmissions may cause significant desense to the whole 2.4 GHz ISM band (in the excess of >28 dB to as much as 77 dB, depending on the victim receiver type and operating frequency).

##### 6.5.6.2.3	ISM to LTE Interference

The TX leakage of Wi-Fi is based on measured Wi-Fi unwanted emissions [X2], Annex A (Conducted emissions test data). The measured Wi-Fi equipment are access points, and it is estimated that similar or worse unwanted emissions spectrum would be applicable for practical Wi-Fi mobile chipsets. The far-away spurious emissions noise floor is assumed at -135 dBm/Hz (at around fc+50 MHz), and the emission spectrum is decaying from the shown level (at fc+25 MHz) linearly in the dB domain.

The TX spectral regrowth of Bluetooth is assumed to be narrowband, and the TX leakage is based on wideband noise only. The level is estimated at -122 dBm/Hz. The IDC analysis is based on TX leakage and LTE receiver blocking.

-

Table 6.5.6.2-4: IDC impact of IDC TX leakage to LTE receiver noise level

| Wi-Fi->LTE | Wi-Fi |  |  | Bluetooth |
| --- | --- | --- | --- | --- |
|  | Ch#11 | Ch#6 | Ch#1 | Any channel |
| TX leakage power | -15 dBm/9 MHz | -60 dBm/9 MHz | -65 dBm/9 MHz | -52 dBm/9 MHz |
| Antenna isolation | 12 dB | 12 dB | 12 dB | 12 dB |
| TX leakage at LTE input | -27 dBm/9 MHz | -72 dBm/9 MHz | -77 dBm/9 MHz | -40 dBm/9 MHz |
| LTE input-referred noise | -95.5 dBm/9 MHz | -95.5 dBm/9 MHz | -95.5 dBm/9 MHz | -95.5 dBm/9 MHz |
| LTE noise level increase | 69 dB | 24 dB | 19 dB | 55.5 dB |

Table 6.5.6.2-5: IDC impact of LTE receiver blocking

| ISM->LTE | Wi-Fi |  |  | Bluetooth |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ch#11 | Ch#6 | Ch#1 | 2480 | 2470 | ≤2460 |
| ISM TX power | 20 dBm | 20 dBm | 20 dBm | 20 dBm | 20 dBm | 20 dBm |
| Antenna isolation | 12 dB | 12 dB | 12 dB | 12 dB | 12 dB | 12 dB |
| ISM blocking power at LTE input | 8 dBm | 8 dBm | 8 dBm | 8 dBm | 8 dBm | 8 dBm |
| LTE filter suppression | 11 dB | 16 dB | 16 dB | 4 dB | 11 dB | 16 dB |
| ISM blocking power at LNA input | -3 dBm | -8 dBm | -8 dBm | 4 dBm | -3 dBm | -8 dBm |
| LNA gain to prevent mixer from saturating (assuming 0 dBm peak output power and 10 dB peak-to-average power ratio) | -7 dB | -2 dB | -2 dB | -4 dB | 3 dB | 8 dB |
| Effect to RX NF due to AGC | 18 dB | 13 dB | 13 dB | 15 dB | 8 dB | 4 dB |

As a conclusion, the IDC interference from IDC transmitter to LTE receiver may be dominated by the TX unwanted emissions, the noise floor increase being from 19 dB up to 69 dB. LTE receiver blocking is also not insignificant and may be an important contributor.

#### 6.5.6.3	Time Domain Analysis

##### 6.5.6.3.1	General

Table 5.3-1 of TR 36.816 [X1] lists the applicability of different identified TDM solutions for each usage scenario. It may be noted that no 3GPP specification for HARQ process reservation-based solution was developed during the IDC WI; however, some additions to the DRX parameter values were done during Rel-11, to make the DRX based solution more applicable to the usage scenarios.

The DRX based solution is applicable to all usage scenarios listed in TR 36.816. As a complementary solution, some LTE uplink subframes may be blocked by the UE autonomously, if the transmission would jeopardize critical ISM signaling.

The basics of the DRX based solution is shown below (TR 36.816 Figure 5.2.1.2.1-2):

Figure 6.5.6.3-1: Example of DRX configured by eNB to enable TDM [X1]

The Rel-11 improvements to DRX to better work as a TDM solution for IDC allow relatively sharp transition from the LTE active duration to the off-period. This is achieved by using short DRX inactivity and retransmission timer values, low maximum retransmission count, and by restricting the scheduling towards the end of the active time. The penalty is ~1% data loss at HARQ level (due to typical 10% BLER target), but may be compensated by more robust MCS selection at the scheduler. In the following analysis, 8 ms of the LTE on-duration is left unscheduled and still unavailable for ISM operations.

The theoretical data rate for TD-LTE 10 MHz channel is around 15-58 Mbps in DL (20-80% of subframes), and 7-21 Mbps in UL (20-60% of subframes). This assumes 64-QAM modulation (256-QAM is an optional capability), 2 MIMO streams in DL and a single stream in UL.

All of the TDM analysis are based on bi-directional interference scenario, i.e. LTE TX causes interference to ISM RX, and ISM TX to LTE RX. In practical cases, one of the interference directions may be missing, e.g. if the TX power levels are sufficiently low.

##### 6.5.6.3.2	LTE + BT Earphone (VoIP Service)

Annex B of TR 36.816 [X1] contains analysis for this scenario, when no specific LTE solution is applied. The TD-LTE subframes and Bluetooth TX and RX slots align so that TX and RX are not concurrent. For EV3 Bluetooth voice connection, the following results have been obtained (TR 36.816 Table B.2.1A-1). Note that only the interference scenario which corrupts all available Bluetooth channels is shown in the below table.

Table 6.5.6.3.2-1: BT EV3 and LTE collision ratio [X1]

| LTE TDD UL/DL Configuration | BT Collision ratio | LTE Collision ratio |
| --- | --- | --- |
| 0 | 0% | 0% |
| 1 | 0% | 0% |
| 2 | 0% | 8.3% |
| 3 | 0% | 14.3% |
| 4 | 0% | 16.7% |
| 5 | 12.5% | 18.5% |
| 6 | 25% | 0% |

For some TDD configurations, the amount of collisions may be significant. Note that in the VoIP service scenario, LTE will typically only require some DL and UL subframes during each VoIP period, so that some of the potential collisions may be recovered by re-transmissions.

For TDD configurations 1 and 2, which are in use on E-UTRA Band 41, [X1] shows the time-domain analysis without any LTE IDC solution. The analysis assumes that the Bluetooth link timing can be adjusted relative to the LTE frame timing, which should be possible at least if the IDC Bluetooth is the master.

Figure 6.5.6.3.2-1: TDD Configuration 1 and BT EV3 (Offset of BT relative to LTE frame: 3.375 ms) [X1]

Figure 6.5.6.3.2-2: TDD Configuration 2 and BT EV3 (Offset of BT relative to LTE frame: 2.375 ms) [X1]

The UE requires a TDM solution for the Bluetooth earphone scenario. In this case, some HARQ processes may be left unscheduled by the eNB. The UE may provide bitmap assistance data in the IDC indication, and the eNB may configure a suitable DRX cycle to allow sufficient time slots for BT operation in all master and slave modes in all TDD configurations [X3, X4].

Furthermore, knowledge of a fixed LTE frame structure and LTE TDD frame configuration are basic elements which allow a TDM solution.

##### 6.5.6.3.3	LTE + BT Earphone (Multimedia Service)

Based on the analysis done for VoIP scenario, it can be seen that the collisions with Bluetooth may be mostly avoided in TD-LTE by aligning the frame structures properly [X1]. Further, since the multimedia streaming application is somewhat delay tolerant, a reasonable DRX cycle may be applied in addition to the synchronization mechanism between the UE’s radio modules, in case the Bluetooth frame structure cannot be aligned (e.g. when operating in slave mode).

Based on [X1], the LTE on-duration of the DRX cycle should be less than [60] ms, and the opportunity for ISM transmissions around [15-60] ms. The LTE throughput available using this solution would be roughly 48-77%. However, as the performance in many TD-LTE configurations may already be manageable, the performance using DRX should be sufficient.

Same as in previous use case, knowledge of a fixed LTE frame structure and LTE TDD frame configuration are basic elements which allow a TDM solution.

##### 6.5.6.3.4	LTE + Wi-Fi Portable Router

Based on [X1], DRX is a potential solution for this usage scenario. The LTE on-duration and the opportunity for ISM transmissions should be typically not more than [20-60] ms. The LTE throughput available using this solution would be roughly 15-65%, depending on the choice of DRX cycle parameters.

Since 54 Mbps PHY rate is available in Wi-Fi, the router usage scenario may be satisfied by roughly equal time share, or by allocating more time for LTE than for Wi-Fi.

##### 6.5.6.3.5	LTE + Wi-Fi Offload

For the offload usage scenario, DRX based solution is also applicable [X1]. The difference to the portable router scenario is that the time shares should be roughly proportional to the amount of traffic over both radio interfaces, and the Wi-Fi beacon transmissions should align with the LTE off-time of the DRX cycle. Some overhead for each DRX cycle (e.g. 8 ms as assumed in earlier analysis) should be taken into account in the time share estimations.

In the IDC indication assistant information, the UE may indicate suitable DRX cycle parameter values for on-duration, cycle length, and offset, to accommodate best the traffic on both radios and the Wi-Fi beacon periods.

#### 6.5.6.4	IDC conclusions

The RF interference analysis shows that the IDC interference may be severe in both directions and affect the whole 2.4 GHz ISM band. The situation is similar as for the highest 20-25 MHz of E-UTRA band 40, according to analysis done in [X1]. However, the LTE receiver may be made somewhat immune to at least Bluetooth transmissions on the lower part of the ISM band, by using a TDD band filter that provides sufficient suppression.

In case the RF interference is strong enough to be an issue, the TDD configuration may in some scenarios provide sufficient time slots for ISM operations, especially if the amount of traffic through LTE is small.

In case the UE is not able to solve the IDC issues by itself, the IDC solution specified in Rel-11 allows to indicate to the network a suitable TDM pattern (DRX parameters or HARQ bitmap), and the network may then configure DRX with suitable parameters to facilitate coexistence between the UE’s radios. There are DRX based solutions to cover all usage scenarios considered in this clause. Any DRX based solution will impact maximum LTE throughput, and the sharp transition from on-duration to off-time will typically incur in small probability of data loss at HARQ level.

The TDM solutions for LTE BT IDC requires the collocated BT device to have knowledge of a fixed LTE frame structure and of the LTE TDD frame structure.

<<< End of TP>>>