3GPP TSG-RAN WG4 Meeting #88bis	R4- 1812173

Chengdu, China, 8th – 12th Oct, 2018

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.133 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on Beam Failure Detection (Section 8.5) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarification on Beam Failure Detection requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on beam failure detection requirements: - Delete SSB based BFD requirements - Threshold for Beam failure detection - Evaluation period in FR2 - CSI-RS configuration BFD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for beam failure detection are not clearly defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1 >>

## 8.5	Link Recovery Procedures

### 8.5.1	Introduction

The UE shall assess the downlink link quality of a serving cell based on the reference signal in the set  as specified in TS 38.213 [3] in order to detect beam failure instance. The RS resources in the set  are periodic CSI-RS resources. UE is not required to perform beam failure detection outside the active DL BWP.

On each RS resource in the set , the UE shall estimate the radio link quality and compare it to the threshold Qout_LR for the purpose of accessing downlink radio link quality of the serving cell.

The threshold Qout_LR is defined as the level at which the downlink radio level link cannot be reliably received and shall correspond to the BLERout of 10%block error rate of a hypothetical PDCCH transmission. For CSI-RS based beam failure detection, Qout_LR_CSI-RS is derived based on the hypothetical PDCCH transmission parameters listed in Table 8.5.2.1-1.

The UE shall perform L1-RSRP measurements based on the reference signal in the set as specified in TS 38.213 [3] in order to detect candidate beam. The RS resources in the set  can be periodic CSI-RS resources and/or SSBs. UE is not required to perform candidate beam detection outside the active DL BWP.

On each RS resource in the set , the UE shall perform L1-RSRP measurements and compare it to the threshold Qin_LR for the purpose of selecting new beam(s) for beam failure recovery.

The threshold Qin_LR corresponds to the value of higher layer parameter candidateBeamThreshold.

### 8.5.2	Requirements for CSI-RS based beam failure detection

#### 8.5.2.1	Introduction

The requirements in this section apply for each CSI-RS resource in the set  configured for a serving cell, provided that the CSI-RS resource configured for beam failure detection are actually transmitted within the UE active DL BWP during the entire evaluation period specified in section 8.5.2.2.

Table 8.5.2.1-1: PDCCH transmission parameters for beam failure instance

| Attribute | Value for BLER |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | Same as the number of symbols of CORESET QCLed with respective CSI-RS for BFD |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | 0dB |
| Bandwidth (MHz) | Same as the number of PRBs of CORESET QCLed with respective CSI-RS for BFD |
| Sub-carrier spacing (kHz) | Same as the SCS of CORESET QCLed with respective CSI-RS for BFD |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Same as the CP length of CORESET QCLed with respective CSI-RS for BFD |
| Mapping from REG to CCE | Distributed |

#### 8.5.2.2	Minimum requirement

Editor’s note: The requirements below have been derived without considering gap sharing when CSI-RS resource for BFD are partially overlapping with measurement gaps.

UE shall be able to evaluate whether the downlink radio link quality on the configured CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS [ms] period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS [ms] period.

The value of TEvaluate_BFD_CSI-RS is defined in Table 8.5.2.2-1 for FR1.

The value of TEvaluate_BFD_CSI-RS is defined in Table 8.5.2.2-2 for FR2 with

-	N=1,

if UE is not provided higher layer parameter RadioLinkMonitoringRS and UE is provided by higher layer parameter TCI-state for PDCCH CSI-RS that has QCL-TypeD, or

if the CSI-RS configured for BFD is QCL-Type D with DM-RS for PDCCH and the QCL association is known to UE, or

-	N=MaxNumberRxBeam, otherwise.

For FR1,

-	P=1/(1 – TCSI-RS/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when BFD-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=1/(1 – TCSI-RS/MGRP) , when BFD-RS is partially overlapped with measurement gap and BFD-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=1/(1 – TCSI-RS /TSMTCperiod), when BFD-RS is not overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is Psharing factor, when BFD-RS is not overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P is 1/(1- TCSI-RS /MGRP - TCSI-RS /TSMTCperiod), when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TCSI-RS = 0.5*TSMTCperiod

-	P is 1/{1- TCSI-RS /min (TSMTCperiod ,MGRP)}, when BFD-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor is 3.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Editor’s Note: FFS definition of overlap between CSI-RS for BFD-RS and SMTC

Longer evaluation period would be expected if the combination of BFD-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

The values of MBFD used in Table 8.5.2.2-1 and Table 8.5.2.2-2 are defined as

- MBFD = 10, if the CSI-RS resource configured for BFD is transmitted with Density = 3.

Table 8.5.2.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| non-DRX | max([50], [MBFD *P] * TCSI-RS) |
| DRX cycle ≤ 320ms | max([50], [1.5×MBFD *P]*max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD*P] * TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

Table 8.5.2.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| non-DRX | max([50], [MBFD *P*N] * TCSI-RS) |
| DRX cycle ≤ 320ms | max([50], [1.5×MBFD*P*N]*max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD *P*N] * TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

<< End of Change #1 >>