3GPP TSG-RAN WG4 Meeting #88-Bis	R4-1812503

Chengdu, China, 8th- 12th October, 2018

Agenda item:		7.11.3.4

Source:	MediaTek Inc.

Title:	Discussion on gap sharing scheme

Document for:		Discussion

# 1	Introduction

In RAN4 #AH1807 meeting, the option 1 of gap sharing schemes had been agreed [1]:

- Applicability of measurement gap sharing factor: - Option 1: only gap occasions that both intra-frequency and inter-frequency/inter-RAT MOs exist

The value of gap sharing factor is specified in Table 9.1.2-5 of TS 38.133. However, the detail principle and formula on how to apply gap sharing factors on the corresponding measurement requirements are still unclear.

- the performance of intra-frequency measurements with no measurement gaps as specified in section 9.2.5, when SMTC configured for intra-frequency measurement are fully overlapping with measurement gaps, shall consider the factor Kintra = 1 / X * 100, - the performance of intra-frequency measurements with measurement gaps as specified in section 9.2.6 shall consider the factor Kintra = 1 / X * 100, - the performance of inter-frequency measurement as specified in section 9.3 and the performance of inter-RAT measurement as specified in section 9.4 shall consider the factor Kinter = 1 / (100 – X) * 100,

Table 9.1.2-5: Value of parameter X

| Network signaling ParameterName (to be determined by RAN2) | Value of X (%) |
| --- | --- |
| ‘00’ | Equal splitting |
| ‘01’ | 25 |
| ‘10’ | 50 |
| ‘11’ | 75 |

In this paper, we discuss how to apply the gap sharing factors on the corresponding measurement requirements and propose the gap occasion pattern based gap sharing scheme.

# 2	Discussion

First of all, in current spec, there are four values of X agreed in Table 9.1.2-5; however, RAN4 did not clarify which value should be used if Network does not configure measGapSharingScheme to UE. We suggest to use “Equal splitting” as a default setting and propose

Proposal 1: RAN4 to clarify “Equal splitting” is the default setting, if Network does not configure “measGapSharingScheme” to UE.

As we know, in NR system, the percentages that UE can conduct intra-freq. measurement and inter-freq. measurement in all gap occasions depends on not only the impact of gap sharing factors, but also the impact of SMTC configurations. Without a precise principle of gap sharing scheme, it will be very hard to evaluate impact of gap sharing factors on the corresponding reporting delay. Therefore, we would like to propose a gap sharing principle with a clear equation in order to make sure that both UE and network have a common understanding on the length of reporting delay. Here, gap sharing factors apply to the individual gap occasion pattern.

An example is used to clarify the definition of gap occasion pattern. Assume that SMTC configurations (periodicity, offset) of 3 measurement objects (MOs) are f1 intra (20, 0), f2 intra (80, 0), and f3 inter (40, 0) with unit of ms. MGRP is 20ms with offset 0ms. Just as shown in below figure, we can see that there are 3 gap occasion patterns listed as follows:

- Pattern 1: f1, f2, and f3 can be observed.

- Pattern 2: only f1 can be observed.

- Pattern 3: f1 and f3 can be observed.

| Gap occasion pattern | P1 | P2 | P3 | P2 | P1 | P2 | P3 | P2 | P1 | P2 | P3 | P2 | P1 | P2 | P3 | P2 | P1 | P2 | P3 | P2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| f1 intra | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 | f1 |
| f2 intra | f2 |  |  |  | f2 |  |  |  | f2 |  |  |  | f2 |  |  |  | f2 |  |  |  |
| f3 inter | f3 |  | f3 |  | f3 |  | f3 |  | f3 |  | f3 |  | f3 |  | f3 |  | f3 |  | f3 |  |

Assume that gap sharing factors are Kintra=2 and Kinter=2. This means

- If both intra-freq. and inter-freq. measurements are possible in a certain gap occasion pattern, they share this gap occasion pattern with equal probability.

- Otherwise, this gap occasion pattern is only taken by either intra-freq. or inter-freq. measurement.

In the example, gap sharing is only needed on pattern 1 and pattern 3.

- For pattern 1:

  - 50% gap occasions are used to measure the intra-freq. layer. Since number of competitive intra-freq. layers is θintra,j=2, each intra-freq. layer (f1, f2) actually shares 25% of the gap occasions

  - 50% gap occasions are used to measure the inter-freq. layer. Since number of competitive inter-freq. layers is θinter,j=1 , this inter-frequency layer (f3) shares 50% of the gap occasions.

- For pattern 2:

  - Gap occasions are not shared. Since number of competitive intra-freq. layers is θj=1, the intra-freq. layer (f1) actually has 100% measurement opportunity of the gap occasions

- For pattern 3:

  - 50% gap occasions are used to measure the intra-freq. layer (f1). The number of competitive intra-freq. layers is θintra,j=1.

  - 50% gap occasions are used to measure the inter-freq. layer (f3). The number of competitive inter-freq. layers is θinter,j=1.

As shown in Appendix A, we simply take the gap occasion patterns with lowest sharing ratio in the final requirement in order to allow the flexibility in UE measurement behavior. For an example, frequency layer f1 takes 25% in P1 and 50% in P3. Then we simply assume f1 takes 25% in all gaps that needs to perform gap sharing.

The final CSFwithin_gap#i can be calculated, according to equation in Appendix A, as below

CSFwithin_gap#i=ceilnumber of gap occasions in which we can observe layer i in an gap occasion cycleall gap occasions in a gap occasion cyclethe percentage that layer i can share in this gap occasion

Therefore, CSF for each frequency layer is (derivation can be found in the Appendix):

- CSFwithin_MGf1=ceil425%+100%+25%+100%=ceil1.33=2

- CSFwithin_MGf2=ceil125%=4

- CSFwithin_MGf3=ceil250%+50%=2

In the last step, the final measurement requirement can be calculated based below equation

T#i=ceil1.5×8×CSFwithin_MG#i×maxTMGRP,TSMTC#i, TDRX

Thus the reporting delay for each frequency layer is (assume no DRX):

- Tf1=5×CSFwithin_gapf1×maxTMGRP,TSMTC#i=5×2×20=200ms

- Tf2=5×CSFwithin_gapf2×maxTMGRP,TSMTC#i=5×4×80=1600ms

- Tf3=8×CSFwithin_gapf3×maxTMGRP,TSMTC#i=8×2×40=640ms

That provides sufficient margin for UE to conduct both the intra-freq. and inter-freq. measurements. A more detailed values of all parameters are listed in Table 1.

Table 1: An example of gap occasion pattern based gap sharing scheme.

To avoid the ambiguity of reporting time between UE and NE sides, a clear gap sharing rule is needed. Therefore, we propose

Proposal 2: An gap occasion pattern based gap sharing scheme is used to specify the corresponding measurement requirements. For each gap occasion pattern, measurement opportunity for intra-freq. is 1/Kintra and measurement opportunity for inter-freq. is 1/Kinter.

# 3	Summary

In this contribution, we propose

Proposal 1: RAN4 to clarify “Equal splitting” is the default setting, if Network does not configure “measGapSharingScheme” to UE.

Proposal 2: An gap occasion pattern based gap sharing scheme is used to specify the corresponding measurement requirements. For each gap occasion pattern, measurement opportunity for intra-freq. is 1/Kintra and measurement opportunity for inter-freq. is 1/Kinter.

# 4	Reference

[1] R4-1809353, Discussion summary for UE measurement gap AI 5.2.3, Intel, Jul. 2018.

# 5	Appendix A

For UE configured with the E-UTRA-NR dual connectivity operation, the effective total number of frequencies being monitored is N’freq, NSA, which is defined as:

N’freq, NSA = Nfreq, NSA + Nfreq, NSA, gap_based_intra-freq,

where

Nfreq, NSA is defined in Section 9.1.3.1.

Nfreq, NSA, gap_based_intra-freq is the number of NR intra-frequency carriers being monitored within the measurement gap

The scaling value CSFwithin_gap_i below have been derived without considering the BSIC verification of GSM inter-RAT carriers or the phase for BSIC initial search.

For UE which supports and is configured with per FR gap, CSFwithin_gap_i is defined on a per frequency range basis, considering the gap pattern and carriers configured in the relevant frequency range. For UE which is configured with per UE gap, CSFwithin_gap_i is defined considering the per UE gap pattern and all configured carriers.

Considering that there are N'freq,NSA carriers to be measured, denote each carrier with index i where i={0…N’freq,NSA-1}

Determine the gap utilization reputation period G

G=max(MGRP, SMTC period0, … SMTC periodi , PRS period1,…))

Let J= G/MGRP, and denote each gap in the period G with index j where j={0,…, J -1}

For each carrier i, and each gap j, determine if the carrier i is a candidate to be measured in the measurement gap j, considering its SMTC configuration.

An NR carrier is a candidate to be measured if its SMTC occasion at least partially coincides in time with the considered measurement gap excluding RF switching time.

The result of this determination is

λi,j=1 if carrier i is a candidate to be measured in gap j

λi,j=0 otherwise

For each measurement gap, count the number of competing measurement objects according to

θj=i=0Nfreq,NSA-1λi,ji=0Nfreq,NSA-1λi,jθj=i=0Nfreq,NSA'-1λi,j

If measGapSharingScheme is not “Equal splitting”, for each measurement gap, count the number of competing intra-frequency measurement objects according to

θintra,j=i=0Nfreq,NSA'-1λi,jXi, where Xi=1,  if carrier i is intra-frequencyXi=0,  otherwise

θinter,j=i=0Nfreq,NSA'-1λi,jYi ,where Yi=1,  if carrier i is inter-frequencyYi=0,  otherwise

For each measurement gap j and each carrier i, compute the assumed measurement scaling factor according to the following

- If λi,j=1 and carrier i involves an LTE RSTD measurement [or GSM BSIC verification measurement] with periodicity Tprs larger than largest configured NR SMTC periodicity then

εi,j=1 and εk,j=0 for all k≠i, and

do not evaluate rules (2), (3), (4) and (5), for gap j.

- If λi,j=1, measGapSharingScheme is not “Equal splitting” and θintra,j>0 and θinter,j>0, and carrier i is an intra-frequency measurement object then  εi,j=K1Nintra,j

εi,j=1Kintra×θintra,j εi,j=1Kintra×θintra, where θintra=max 0≤j'≤J-1 θintra,j'.=max j θintra,jεi,j=1Kintra×Nintra,j

- If λi,j=1, measGapSharingScheme is not “Equal splitting” and θintra,j>0 and θinter,j>0, and carrier i is an inter-frequency/inter-RAT measurement object then

εi,j=1Kinter×Ninter,jεi,j=1Kinter×θinter, where θinter=max0≤j'≤J-1 θinter,j'.εi,j=1Kintra×Nintra,j

- If λi,j=1 and measGapSharingScheme is “Equal splitting” then

εi,j=1θ, where  θ=max0≤j'≤J-1 θj'.

- If  λi,j=0 then

εi,j=0.

For each carrier i, compute the total number of measurement opportunities according to

Δi=j=0J-1λi,j

The scaling factor for carrier i is then determined according to

CSFwithin_gap_i=ceilΔij=0J-1εi,j