3GPP TSG-RAN WG4 Meeting #88-Bis                                                          R4-1812514

# Chengdu, China, 8th – 12th Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for SSB based RLM in EN-DC and SA (section A.4.5.1, A.5.5.1, A.6.5.1, and A.7.5.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  |  | Date: |  | 2018-09-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no test cases for RLM in EN-DC and SA |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RLM test cases for EN-DC and SA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases for RLM in EN-DC and SA will be missed in TS 38.133 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new A.4.5.1, new A.5.5.1, new A.6.5.1, new A.7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR ... CR ...TS38.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGE 1>

### A.4.5.1	Radio Link Monitoring

In the following section, any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

-	UE output power higher than Transmit OFF power [-50] dBm (as defined in TS 38.101 [TBD]) means uplink signal

-	UE output power equal to or less than Transmit OFF power [-50] dBm (as defined in TS 38.101 [TBD]) means no uplink signal.

### A.4.5.1.1			Radio Link Monitoring Out-of-sync Test for FR1 FDD PSCell configured with SSB-based RLM RS in non-DRX mode

#### A.4.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR1 FDD PSCell radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.4.5.1.1.1-1, A.4.5.1.1.1-2, and A.4.5.1.1.1-3 below. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.4.5.1.1.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.4.5.1.1.1-1: General test parameters for FR1 FDD PSCell out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX |  |  | OFF | OFF | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 1 | 1 | 1 | 1 |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T3 |  | s | [0.6] | [0.6] | [0.6] | [0.6] |
| D1 |  | s | [0.24] | [0.24] | [0.44] | [0.44] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.4.5.1.1.1-2: Cell specific test parameters for FR1 FDD (cell # 2) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in non-DRX mode

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_bete | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.1.1.1-3: Measurement gap configuration for FR1 FDD out-of-sync tests in non-DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [0] | [0] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap) |  |  |

Figure A.4.5.1.1.1-1: SNR variation for out-of-sync testing

#### A.4.5.1.1.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.4.5.1.2			Radio Link Monitoring In-sync Test for FR1 FDD PSCell configured with SSB-based RLM RS in non-DRX mode

#### A.4.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR1 FDD PSCell radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.4.5.1.2.1-1, A.4.5.1.2.1-2, A.4.5.1.2.1-3, and A.4.5.1.2.1-4 below. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.4.5.1.2.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.4.5.1.2.1-1: General test parameters for FR1 FDD in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX |  |  | OFF | OFF | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 0.5 | 0.5 | 0.5 | 0.5 |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T3 |  | s | [1.46] | [1.46] | [1.36] | [1.36] |
| T4 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T5 |  | s | [1] | [1] | [1.1] | [1.1] |
| D1 |  | s | [0.42] | [0.42] | [0.72] | [0.72] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

A.4.5.1.2.1-2: Cell specific test parameters for FR1 FDD (cell # 2) for in-sync radio link monitoring tests #1-A, and #1-B in non-DRX mode

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

A.4.5.1.2.1-3: Cell specific test parameters for FDD (cell # 2) for in-sync radio link monitoring tests #2-A and #2-B in non-DRX mode

| Parameter | Unit | Test 2-A |  |  |  |  | Test 2-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

A.4.5.1.2.1-4: Measurement gap configuration for FDD in-sync tests in non-DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [TBD] | [TBD] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |  |

Figure A.4.5.1.2.1-1: SNR variation for in-sync testing

#### A.4.5.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.4.5.1.3			Radio Link Monitoring Out-of-sync Test for FR1 TDD PSCell configured with SSB-based RLM RS in non-DRX mode

#### A.4.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR1 TDD PSCell radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.4.5.1.3.1-1, A.4.5.1.3.1-2, and A. 4.5.1.3.1-3 below.  There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.4.5.1.3.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.4.5.1.3.1-1: General test parameters for FR1 TDD out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX |  |  | OFF | OFF | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 1 | 1 | 1 | 1 |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T3 |  | s | [0.6] | [0.6] | [0.6] | [0.6] |
| D1 |  | s | [0.24] | [0.24] | [0.44] | [0.44] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.4.5.1.3.1-2: Cell specific test parameters for FR1 TDD (cell # 2) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in non-DRX mode

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.1.3.1-3: Measurement gap configuration for TDD out-of-sync tests in non-DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [0] | [0] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |  |

Figure A.4.5.1.3.1-1: SNR variation for out-of-sync testing

#### A.4.5.1.3.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.4.5.1.4			Radio Link Monitoring In-sync Test for FR1 TDD PSCell configured with SSB-based RLM RS in non-DRX mode

#### A.4.5.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR1 TDD PSCell radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.4.5.1.4.1-1, A.4.5.1.4.1-2, A.4.5.1.4.1-3, and A.4.5.1.4.1-4 below. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.4.5.1.4.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.4.5.1.4.1-1: General test parameters for FR1 TDD in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX |  |  | OFF | OFF | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 0.5 | 0.5 | 0.5 | 0.5 |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T3 |  | s | [1.46] | [1.46] | [1.36] | [1.36] |
| T4 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T5 |  | s | [1] | [1] | [1.1] | [1.1] |
| D1 |  | s | [0.42] | [0.42] | [0.72] | [0.72] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.4.5.1.4.1-2: Cell specific test parameters for FR1 TDD (cell # 2) for in-sync radio link monitoring tests #1-A, and #1-B in non-DRX mode

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.1.4.1-3: Cell specific test parameters for TDD (cell # 2) for in-sync radio link monitoring tests #2-A and #2-B in non-DRX mode

| Parameter | Unit | Test 2-A |  |  |  |  | Test 2-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.1.4.1-4: Measurement gap configuration for TDD in-sync tests in non-DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [TBD] | [TBD] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |  |

Figure A.4.5.1.4.1-1: SNR variation for in-sync testing

#### A.4.5.1.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.4.5.1.5			Radio Link Monitoring Out-of-sync Test for FR1 FDD PSCell configured with SSB-based RLM RS in DRX mode

#### A.4.5.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the FR1 FDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.4.5.1.5.1-1, A.4.5.1.5.1-2, A.4.5.1.5.1-3, A.4.5.1.5.1-4 and A.4.5.1.5.1-5. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.4.5.1.5.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.4.5.1.5.1-1: General test parameters for FR1 FDD out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX cycle |  | ms | 1280 | 1280 | 40 | 40 |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 32 | 32 | 4 | 4 |
| T2 |  | s | 12.8 | 12.8 | 1.6 | 1.6 |
| T3 |  | s | 13 | 13 | 1.8 | 1.8 |
| D1 |  | s | [12.9] | [12.9] | [1.3] | [1.3] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.4.5.1.5.1-2: Cell specific test parameters for FR1 FDD (cell # 2) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in DRX mode.

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.1.5.1-3: Measurement gap configuration for FDD out-of-sync tests in DRX mode.

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [0] | [0] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |  |

Table A.4.5.1.5.1-4: DRX-Configuration for FDD out-of-sync tests.

| Field | Test 1-A | Test 1-B | Test 2-A | Test 1-B |
| --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] | [sl1] | [sl1] |
| longDRX-CycleStartOffset | [ms1280] | [ms1280] | [ms40] | [ms40] |
| shortDRX | disable | disable | disable | disable |

Table A.4.5.1.5.1-5: DRX-Configuration for FDD out-of-sync tests.

| Field | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |
| TimeAlignmentTimer | [Infinity] | infinity | infinity | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] | [sl5] | [sl10] |

Figure A.4.5.1.5.1-1: SNR variation for out-of-sync testing.

#### A.4.5.1.5.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.4.5.1.6			Radio Link Monitoring In-sync Test for FR1 FDD PSCell configured with SSB-based RLM RS in DRX mode

#### A.4.5.1.6.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the FR1 FDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables Table A.4.5.1.6.1-1, Table A.4.5.1.6.1-2, Table A.4.5.1.6.1-3 and Table A.4.5.1.6.1-4. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure Table A.4.5.1.6.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.4.5.1.6.1-1: General test parameters for FR1 FDD in-sync testing in DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX cycle |  | ms | 40 | 40 |
| Gap pattern ID |  |  | [N.A.] | [N.A.] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] |
| T1 |  | s | 4 | 4 |
| T2 |  | s | 1.6 | 1.6 |
| T3 |  | s | [1.36] | [1.36] |
| T4 |  | s | 0.4 | 0.4 |
| T5 |  | s | 4 | 4 |
| D1 |  | s | [1] | [1] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.4.5.1.6.1-2: Cell specific test parameters for FR1 FDD (cell # 2) for in-sync radio link monitoring tests #1-A, and #1-B in DRX mode

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 8: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.1.6.1-3: DRX-Configuration for FDD in-sync tests

| Field | Test 1-A | Test 1-B |
| --- | --- | --- |
|  | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |
| longDRX-CycleStartOffset | [ms40] | [ms40] |
| shortDRX | disable | disable |

Table A.4.5.1.6.1-4: TimeAlignmentTimer -Configuration for FDD in-sync testing

| Field | Test 1-A | Test 1-B |
| --- | --- | --- |
|  | Value | Value |
| TimeAlignmentTimer | infinity | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] |

Figure A.4.5.1.6.1-1: SNR variation for in-sync testing.

#### A.4.5.1.6.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.4.5.1.7			Radio Link Monitoring Out-of-sync Test for FR1 TDD PSCell configured with SSB-based RLM RS in DRX mode

#### A.4.5.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the FR1 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.4.5.1.7.1-1, A.4.5.1.7.1-2, A.4.5.1.7.1-3, A.4.5.1.7.1-4 and A.4.5.1.7.1-5. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.4.5.1.7.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.4.5.1.7.1-1: General test parameters for FR1 TDD out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX cycle |  | ms | 1280 | 1280 | 40 | 40 |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 32 | 32 | 4 | 4 |
| T2 |  | s | 12.8 | 12.8 | 1.6 | 1.6 |
| T3 |  | s | 13 | 13 | 1.8 | 1.8 |
| D1 |  | s | [12.9] | [12.9] | [1.3] | [1.3] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.4.5.1.7.1-2: Cell specific test parameters for FR1 TDD (cell # 2) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in DRX mode

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.1.7.1-3: Measurement gap configuration for TDD out-of-sync tests in DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [0] | [0] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |  |

Table A.4.5.1.7.1-4: DRX-Configuration for TDD out-of-sync tests

| Field | Test 1-A | Test 1-B | Test 2-A | Test 1-B |
| --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] | [sl1] | [sl1] |
| longDRX-CycleStartOffset | [ms1280] | [ms1280] | [ms40] | [ms40] |
| shortDRX | disable | disable | disable | disable |

Table A.4.5.1.7.1-5: TimeAlignmentTimer -Configuration for TDD out-of-sync testing

| Field | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |
| TimeAlignmentTimer | [Infinity] | infinity | infinity | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] | [sl5] | [sl10] |

Figure A.4.5.1.7.1-1: SNR variation for out-of-sync testing

#### A.4.5.1.7.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.4.5.1.8			Radio Link Monitoring In-sync Test for FR1 TDD PSCell configured with SSB-based RLM RS in DRX mode

#### A.4.5.1.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the FR1 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables Table A.4.5.1.8.1-1, Table A.4.5.1.8.1-2, Table A.4.5.1.8.1-3 and Table A.4.5.1.8.1-4. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure Table A.4.5.1.8.1-1 shows the variation of the downlink SNR in the active Cell 2 to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.4.5.1.8.1-1: General test parameters for FR1 TDD in-sync testing in DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.1-1 | Table A.3.7.2.1-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX cycle |  | ms | 40 | 40 |
| Gap pattern ID |  |  | [N.A.] | [N.A.] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] |
| T1 |  | s | 4 | 4 |
| T2 |  | s | 1.6 | 1.6 |
| T3 |  | s | [1.36] | [1.36] |
| T4 |  | s | 0.4 | 0.4 |
| T5 |  | s | 4 | 4 |
| D1 |  | s | [1] | [1] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.4.5.1.8.1-2: Cell specific test parameters for FR1 TDD (cell # 2) for in-sync radio link monitoring tests #1-A, and #1-B in DRX mode

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.4.5.1.8.1-3: DRX-Configuration for TDD in-sync tests

| Field | Test 1-A | Test 1-B |
| --- | --- | --- |
|  | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |
| longDRX-CycleStartOffset | [ms40] | [ms40] |
| shortDRX | disable | disable |

Table A.4.5.1.8.1-4: TimeAlignmentTimer -Configuration for TDD in-sync testing

| Field | Test 1-A | Test 1-B |
| --- | --- | --- |
|  | Value | Value |
| TimeAlignmentTimer | infinity | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] |

Figure A.4.5.1.8.1-1: SNR variation for in-sync testing

#### A.4.5.1.8.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

<END OF CHANGE 1>

<START OF CHANGE 2>

A.5.5.1	Radio Link Monitoring

In the following section, any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

-	UE output power higher than Transmit OFF power [-50] dBm (as defined in TS 38.101 [TBD]) means uplink signal

-	UE output power equal to or less than Transmit OFF power [-50] dBm (as defined in TS 38.101 [TBD]) means no uplink signal.

A.5.5.1.1			Radio Link Monitoring Out-of-sync Test for FR2 TDD PSCell configured with SSB-based RLM RS in non-DRX mode

A.5.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR2 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.5.5.1.1.1-1, A.5.5.1.1.1-2, and A. 5.5.1.1.1-3 below. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.5.5.1.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Table A.5.5.1.1.1-1: General test parameters for FR2 TDD out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.2-1 | Table A.3.7.2.2-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] | [100] |
| SSB Subcarrier Spacing |  | KHz | [120] | [120] |
| [Data] Subcarrier Spacing |  | KHz | [120] | [120] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX |  |  | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] | [40] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [40] | [40] |
| T1 |  | s | 1 | 1 |
| T2 |  | s | 0.4 | 0.4 |
| T3 |  | s | [0.6] | [0.6] |
| D1 |  | s | [0.24] | [0.44] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |

Table A.5.5.1.1.1-2: Cell specific test parameters for FR2 TDD (cell # 2) for out-of-sync radio link monitoring tests #1 and #2 in non-DRX mode

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | TBD |  |  | TBD |  |  |
| Propagation condition |  | [TDL-A 30ns 75Hz] |  |  | [TDL-A 30ns 75Hz] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |

Table A.5.5.1.1.1-3: Measurement gap configuration for TDD out-of-sync tests in non-DRX mode

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [0] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |  |

Figure A.5.5.1.1.1-1: SNR variation for out-of-sync testing

A.5.5.1.1.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.5.5.1.2			Radio Link Monitoring In-sync Test for FR2 TDD PSCell configured with SSB-based RLM RS in non-DRX mode

A.5.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell. This test will partly verify the FR2 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.5.5.1.2.1-1, A.5.5.1.2.1-2, and A.5.5.1.2.1-3 below. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.5.5.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Table A.5.5.1.2.1-1: General test parameters for FR2 TDD in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.2-1 | Table A.3.7.2.2-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] | [100] |
| SSB Subcarrier Spacing |  | KHz | [120] | [120] |
| [Data] Subcarrier Spacing |  | KHz | [120] | [120] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX |  |  | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] | [40] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [40] | [40] |
| T1 |  | s | 0.5 | 0.5 |
| T2 |  | s | 0.4 | 0.4 |
| T3 |  | s | [1.46] | [1.36] |
| T4 |  | s | 0.4 | 0.4 |
| T5 |  | s | [1] | [1.1] |
| D1 |  | s | [0.42] | [0.72] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |

Table A.5.5.1.2.1-2: Cell specific test parameters for FR2 TDD (cell # 2) for in-sync radio link monitoring tests #1 and test #2 in non-DRX mode

| Parameter | Unit | Test 1 |  |  |  |  | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | TBD |  |  |  |  | TBD |  |  |  |  |
| Propagation condition |  | [TDL-A 30ns 75Hz] |  |  |  |  | [TDL-A 30ns 75Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.5.5.1.2.1-3: Measurement gap configuration for TDD in-sync tests in non-DRX mode.

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [TBD] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |

Figure A.5.5.1.2.1-1: SNR variation for in-sync testing

A.5.5.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

A.5.5.1.3			Radio Link Monitoring Out-of-sync Test for FR2 TDD PSCell configured with SSB-based RLM RS in DRX mode

A.5.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the FR2 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.5.5.1.3.1-1, A.5.5.1.3.1-2, A.5.5.1.3.1-3, A.5.5.1.3.1-4 and A.5.5.1.3.1-5. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test.The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.5.5.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Table A.5.5.1.3.1-1: General test parameters for FR2 TDD out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| Active E-UTRA PCell |  |  | Cell 1 | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.2-1 | Table A.3.7.2.2-1 |
| E-UTRA RF Channel Number |  |  | 1 | 1 |
| Active PSCell |  |  | Cell 2 | Cell 2 |
| RF Channel Number |  |  | 2 | 2 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] | [100] |
| SSB Subcarrier Spacing |  | KHz | [120] | [120] |
| [Data] Subcarrier Spacing |  | KHz | [120] | [120] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX cycle |  |  | 1280 | 40 |
| Gap pattern ID |  |  | [N.A.] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] | [40] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [40] | [40] |
| T1 |  | s | 32 | 4 |
| T2 |  | s | 12.8 | 1.6 |
| T3 |  | s | 13 | 1.8 |
| D1 |  | s | [12.9] | [1.3] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.5.5.1.3.1-2: Cell specific test parameters for FR2 TDD (cell # 2) for out-of-sync radio link monitoring tests #1, and #2 in DRX mode

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | TBD |  |  | TBD |  |  |
| Propagation condition |  | [TDL-A 30ns 75Hz] |  |  | [TDL-A 30ns 75Hz] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |

Table A.5.5.1.3.1-3: Measurement gap configuration for TDD out-of-sync tests in DRX mode

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [0] |
| Note 1: E-UTRAN PCell and PSCell are SFN-synchronous and frame boundary aligned. (Ensure that RLM RS is partially overlapped with measurement gap). |  |  |

Table A.5.5.1.3.1-4: DRX-Configuration for TDD out-of-sync tests

| Field | Test 1 | Test 2 |
| --- | --- | --- |
|  | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |
| longDRX-CycleStartOffset | [ms1280] | [ms40] |
| shortDRX | disable | Disable |

Table A.5.5.1.3.1-5: TimeAlignmentTimer -Configuration for TDD out-of-sync testing

| Field | Test1 | Test2 |
| --- | --- | --- |
|  | Value | Value |
| TimeAlignmentTimer | [Infinity] | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl40] | [sl40] |

Figure A.5.5.1.3.1-1: SNR variation for out-of-sync testing

A.5.5.1.3.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.5.5.1.4			Radio Link Monitoring In-sync Test for FR2 TDD PSCell configured with SSB-based RLM RS in DRX mode

A.5.5.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PSCell when DRX is used. This test will partly verify the FR2 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables Table A.5.5.1.4.1-1, Table A.5.5.1.4.1, Table A.5.5.1.4.1-3 and Table A.5.5.1.4.1-4. There are two cells, Cell 1 is the E-UTRAN PCell, and Cell 2 is the PSCell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure Table A.5.5.1.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.5.5.1.4.1-1: General test parameters for FR2 TDD in-sync testing in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active E-UTRA PCell |  |  | Cell 1 |
| E-UTRA parameters |  |  | Table A.3.7.2.2-1 |
| E-UTRA RF Channel Number |  |  | 1 |
| Active PSCell |  |  | Cell 2 |
| RF Channel Number |  |  | 2 |
| PDCCH parameters |  |  | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] |
| OCNG parameters |  |  | TBD |
| CP length |  |  | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] |
| SSB Subcarrier Spacing |  | KHz | [120] |
| [Data] Subcarrier Spacing |  | KHz | [120] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX cycle |  | ms | 40 |
| Gap pattern ID |  |  | [N.A.] |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 2000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] |
| Periodic CSI reporting |  |  | PUCCH |
| CSI reporting periodicity |  | slot | [40] |
| Propagation channel |  |  | [TDL-A 30ns 75Hz] |
| T1 |  | s | 4 |
| T2 |  | s | 1.6 |
| T3 |  | s | [1.36] |
| T4 |  | s | 0.4 |
| T5 |  | s | 4 |
| D1 |  | s | [1] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.5.5.1.4.1-2: Cell specific test parameters for FR2 TDD (cell # 2) for in-sync radio link monitoring tests #1 in DRX mode

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | TBD |  |  |  |  |
| Propagation condition |  | [TDL-A 30ns 75Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 2 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |

Table A.5.5.1.4.1-3: DRX-Configuration for TDD in-sync tests

| Field | Test 1 |
| --- | --- |
|  | Value |
| drx-onDurationTimer | [ms6] |
| drx-InactivityTimer | [ms1] |
| drx-RetransmissionTimerDL | [sl1] |
| drx-RetransmissionTimerUL | [sl1] |
| longDRX-CycleStartOffset | [ms40] |
| shortDRX | disable |

Table A.5.5.1.4.1-4: TimeAlignmentTimer -Configuration for TDD in-sync testing

| Field | Test 1 |
| --- | --- |
|  | Value |
| TimeAlignmentTimer | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl40] |

Figure A.5.5.1.4.1-1: SNR variation for in-sync testing.

A.5.5.1.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

<END OF CHANGE 2>

<START OF CHANGE 3>

<END OF CHANGE 3>

A.6.5.1	Radio Link Monitoring

In the following section, any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

-	UE output power higher than Transmit OFF power [-50] dBm (as defined in TS 38.101 [TBD]) means uplink signal

-	UE output power equal to or less than Transmit OFF power [-50] dBm (as defined in TS 38.101 [TBD]) means no uplink signal.

A.6.5.1.1			Radio Link Monitoring Out-of-sync Test for FR1 FDD PCell configured with SSB-based RLM RS in non-DRX mode

A.6.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 FDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.1.1-1, A.6.5.1.1.1-2, and A.6.5.1.1.1-3 below. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.6.5.1.1.1-1: General test parameters for FR1 FDD out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active PSCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX |  |  | OFF | OFF | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 1 | 1 | 1 | 1 |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T3 |  | s | [0.6] | [0.6] | [0.6] | [0.6] |
| D1 |  | s | [0.24] | [0.24] | [0.44] | [0.44] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.6.5.1.1.1-2: Cell specific test parameters for FR1 FDD (cell # 1) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in non-DRX mode

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.1.1.1-3: Measurement gap configuration for FDD out-of-sync tests in non-DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [0] | [0] |

Figure A.6.5.1.1.1-1: SNR variation for out-of-sync testing

A.6.5.1.1.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.2			Radio Link Monitoring In-sync Test for FR1 FDD PCell configured with SSB-based RLM RS in non-DRX mode

A.6.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 FDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.2.1-1, A.6.5.1.2.1-2, A.6.5.1.2.1-3, and A.6.5.1.2.1-4 below. There is one cell (Cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.6.5.1.2.1-1: General test parameters for FR1 FDD in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active PSCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX |  |  | OFF | OFF | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 0.5 | 0.5 | 0.5 | 0.5 |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T3 |  | s | [1.46] | [1.46] | [1.36] | [1.36] |
| T4 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T5 |  | s | [1] | [1] | [1.1] | [1.1] |
| D1 |  | s | [0.42] | [0.42] | [0.72] | [0.72] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

A.6.5.1.2.1-2: Cell specific test parameters for FR1 FDD (cell # 1) for in-sync radio link monitoring tests #1-A, and #1-B in non-DRX mode

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

A.6.5.1.2.1-3: Cell specific test parameters for FDD (cell # 1) for in-sync radio link monitoring tests #2-A and #2-B in non-DRX mode

| Parameter | Unit | Test 2-A |  |  |  |  | Test 2-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

A.6.5.1.2.1-4: Measurement gap configuration for FDD in-sync tests in non-DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [TBD] | [TBD] |

Figure A.6.5.1.2.1-1: SNR variation for in-sync testing

A.6.5.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.3			Radio Link Monitoring Out-of-sync Test for FR1 TDD PCell configured with SSB-based RLM RS in non-DRX mode

A.6.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.3.1-1, A.6.5.1.3.1-2, and A. 6.5.1.3.1-3 below. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.6.5.1.3.1-1: General test parameters for FR1 TDD out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active PSCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX |  |  | OFF | OFF | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 1 | 1 | 1 | 1 |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T3 |  | s | [0.6] | [0.6] | [0.6] | [0.6] |
| D1 |  | s | [0.24] | [0.24] | [0.44] | [0.44] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.6.5.1.3.1-2: Cell specific test parameters for FR1 TDD (cell # 1) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in non-DRX mode

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.1.3.1-3: Measurement gap configuration for TDD out-of-sync tests in non-DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [0] | [0] |

Figure A.6.5.1.3.1-1: SNR variation for out-of-sync testing

A.6.5.1.3.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.4			Radio Link Monitoring In-sync Test for FR1 TDD PCell configured with SSB-based RLM RS in non-DRX mode

A.6.5.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.4.1-1, A.6.5.1.4.1-2, A.6.5.1.4.1-3, and A.6.5.1.4.1-4 below. There is one cell (Cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.6.5.1.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.6.5.1.4.1-1: General test parameters for FR1 TDD in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active PSCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX |  |  | OFF | OFF | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 0.5 | 0.5 | 0.5 | 0.5 |
| T2 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T3 |  | s | [1.46] | [1.46] | [1.36] | [1.36] |
| T4 |  | s | 0.4 | 0.4 | 0.4 | 0.4 |
| T5 |  | s | [1] | [1] | [1.1] | [1.1] |
| D1 |  | s | [0.42] | [0.42] | [0.72] | [0.72] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.6.5.1.4.1-2: Cell specific test parameters for FR1 TDD (cell # 1) for in-sync radio link monitoring tests #1-A, and #1-B in non-DRX mode

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.1.4.1-3: Cell specific test parameters for TDD (cell # 1) for in-sync radio link monitoring tests #2-A and #2-B in non-DRX mode

| Parameter | Unit | Test 2-A |  |  |  |  | Test 2-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.1.4.1-4: Measurement gap configuration for TDD in-sync tests in non-DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [TBD] | [TBD] |

Figure A.6.5.1.4.1-1: SNR variation for in-sync testing

A.6.5.1.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.5			Radio Link Monitoring Out-of-sync Test for FR1 FDD PCell configured with SSB-based RLM RS in DRX mode

A.6.5.1.5.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 FDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.5.1-1, A.6.5.1.5.1-2, A.6.5.1.5.1-3, A.6.5.1.5.1-4 and A.6.5.1.5.1-5. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.5.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.6.5.1.5.1-1: General test parameters for FR1 FDD out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active PSCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] | [R. TBD FDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX cycle |  | ms | 1280 | 1280 | 40 | 40 |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 32 | 32 | 4 | 4 |
| T2 |  | s | 12.8 | 12.8 | 1.6 | 1.6 |
| T3 |  | s | 13 | 13 | 1.8 | 1.8 |
| D1 |  | s | [12.9] | [12.9] | [1.3] | [1.3] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.6.5.1.5.1-2: Cell specific test parameters for FR1 FDD (cell # 1) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in DRX mode.

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |  | Test 2-A |  |  |  |  |  | Test 2-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |  | T1 |  | T2 |  | T3 |  | T1 |  | T2 |  | T3 |  | T1 |  | T2 |  | T3 |  |
| PDCCH_beta | dB | 4 |  |  |  | 4 |  |  |  |  |  | 4 |  |  |  |  |  | 4 |  |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  | 4 |  |  |  |  |  | 4 |  |  |  |  |  | 4 |  |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  | 0 |  |  |  |  |  | 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD |  | TBD |  | TBD |  | TBD |  | TBD |  | TBD |  | TBD |  | TBD |  | TBD |  | TBD |  | TBD |
|  | dBm/15 kHz | [-98] |  |  |  | [-98] |  |  |  |  |  | [-98] |  |  |  |  |  | [-98] |  |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.1.5.1-3: Measurement gap configuration for FDD out-of-sync tests in DRX mode.

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [0] | [0] |

Table A.6.5.1.5.1-4: DRX-Configuration for FDD out-of-sync tests.

| Field | Test 1-A | Test 1-B | Test 2-A | Test 1-B |
| --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] | [sl1] | [sl1] |
| drx-LongCycle | [ms1280] | [ms1280] | [ms40] | [ms40] |
| drx-StartOffset | [ms0] | [ms0] | [ms0] | [ms0] |
| shortDRX | disable | disable | disable | disable |

Table A.6.5.1.5.1-5: DRX-Configuration for FDD out-of-sync tests.

| Field | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |
| TimeAlignmentTimer | [Infinity] | infinity | infinity | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] | [sl5] | [sl10] |

Figure A.6.5.1.5.1-1: SNR variation for out-of-sync testing.

A.6.5.1.5.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.6			Radio Link Monitoring In-sync Test for FR1 FDD PCell configured with SSB-based RLM RS in DRX mode

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 FDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables Table A.6.5.1.6.1-1, Table A.6.5.1.6.1-2, Table A.6.5.1.6.1-3 and Table A.6.5.1.6.1-4. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure Table A.6.5.1.6.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

A.6.5.1.6.1	Test Purpose and Environment

Table A.6.5.1.6.1-1: General test parameters for FR1 FDD in-sync testing in DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B |
| Active PSCell |  |  | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD FDD] | [R. TBD FDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX cycle |  | ms | 40 | 40 |
| Gap pattern ID |  |  | [N.A.] | [N.A.] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] |
| T1 |  | s | 4 | 4 |
| T2 |  | s | 1.6 | 1.6 |
| T3 |  | s | [1.36] | [1.36] |
| T4 |  | s | 0.4 | 0.4 |
| T5 |  | s | 4 | 4 |
| D1 |  | s | [1] | [1] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.6.5.1.6.1-2: Cell specific test parameters for FR1 FDD (cell # 1) for in-sync radio link monitoring tests #1-A, and #1-B in DRX mode

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 5: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 6: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 7: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 8: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.1.6.1-3: DRX-Configuration for FDD in-sync tests

| Field | Test 1-A | Test 1-B |
| --- | --- | --- |
|  | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |
| drx-LongCycle | [ms40] | [ms40] |
| drx-StartOffset | [ms0] | [ms0] |
| shortDRX | disable | disable |

Table A.6.5.1.6.1-4: TimeAlignmentTimer -Configuration for FDD in-sync testing

| Field | Test 1-A | Test 1-B |
| --- | --- | --- |
|  | Value | Value |
| TimeAlignmentTimer | infinity | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] |

Figure A.6.5.1.6.1-1: SNR variation for in-sync testing.

A.6.5.1.6.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.7			Radio Link Monitoring Out-of-sync Test for FR1 TDD PCell configured with SSB-based RLM RS in DRX mode

A.6.5.1.7.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.6.5.1.7.1-1, A.6.5.1.7.1-2, A.6.5.1.7.1-3, A.6.5.1.7.1-4 and A.6.5.1.7.1-5. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.7.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2-A and test 2-B.

Table A.6.5.1.7.1-1: General test parameters for FR1 TDD out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| Active PSCell |  |  | Cell 1 | Cell 1 | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD | TBD | TBD |
| CP length |  |  | Normal | Normal | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 | 6 | 6 |
| DRX cycle |  | ms | 1280 | 1280 | 40 | 40 |
| Gap pattern ID |  |  | [N.A.] | [N.A.] | *[gp0] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 | 1000 | 1000 |
| N310 |  |  | 1 | 1 | 1 | 1 |
| N311 |  |  | 1 | 1 | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] | [5] | [10] |
| T1 |  | s | 32 | 32 | 4 | 4 |
| T2 |  | s | 12.8 | 12.8 | 1.6 | 1.6 |
| T3 |  | s | 13 | 13 | 1.8 | 1.8 |
| D1 |  | s | [12.9] | [12.9] | [1.3] | [1.3] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |  |

Table A.6.5.1.7.1-2: Cell specific test parameters for FR1 TDD (cell # 1) for out-of-sync radio link monitoring tests #1-A, #1-B, # 2-A and #2-B in DRX mode

| Parameter | Unit | Test 1-A |  |  | Test 1-B |  |  | Test 2-A |  |  | Test 2-B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  | [-98] |  |  | [-98] |  |  | [-98] |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  | [TDL-C 300ns 100Hz] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.1.7.1-3: Measurement gap configuration for TDD out-of-sync tests in DRX mode

| Field | Test 2-A | Test 2-B |
| --- | --- | --- |
|  | Value | Value |
| gapOffset | [0] | [0] |

Table A.6.5.1.7.1-4: DRX-Configuration for TDD out-of-sync tests

| Field | Test 1-A | Test 1-B | Test 2-A | Test 1-B |
| --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] | [sl1] | [sl1] |
| drx-LongCycle | [ms1280] | [ms1280] | [ms40] | [ms40] |
| drx-StartOffset | [ms0] | [ms0] | [ms0] | [ms0] |
| shortDRX | disable | disable | disable | disable |

Table A.6.5.1.7.1-5: TimeAlignmentTimer -Configuration for TDD out-of-sync testing

| Field | Test 1-A | Test 1-B | Test 2-A | Test 2-B |
| --- | --- | --- | --- | --- |
|  | Value | Value | Value | Value |
| TimeAlignmentTimer | [Infinity] | infinity | infinity | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] | [sl5] | [sl10] |

Figure A.6.5.1.7.1-1: SNR variation for out-of-sync testing

A.6.5.1.7.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

A.6.5.1.8			Radio Link Monitoring In-sync Test for FR1 TDD PCell configured with SSB-based RLM RS in DRX mode

A.6.5.1.8.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR1 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables Table A.6.5.1.8.1-1, Table A.6.5.1.8.1-2, Table A.6.5.1.8.1-3 and Table A.6.5.1.8.1-4. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure Table A.6.5.1.8.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.6.5.1.8.1-1: General test parameters for FR1 TDD in-sync testing in DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1-A | Test 1-B |
| Active PSCell |  |  | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| Channel Bandwidth (BWchannel) |  | MHz | [10] | [40] |
| SSB Subcarrier Spacing |  | KHz | [15] | [30] |
| [Data] Subcarrier Spacing |  | KHz | [15] | [30] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX cycle |  | ms | 40 | 40 |
| Gap pattern ID |  |  | [N.A.] | [N.A.] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity |  | slot | [5] | [10] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [5] | [10] |
| T1 |  | s | 4 | 4 |
| T2 |  | s | 1.6 | 1.6 |
| T3 |  | s | [1.36] | [1.36] |
| T4 |  | s | 0.4 | 0.4 |
| T5 |  | s | 4 | 4 |
| D1 |  | s | [1] | [1] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl2] areconfigured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.6.5.1.8.1-2: Cell specific test parameters for FR1 TDD (cell # 1) for in-sync radio link monitoring tests #1-A, and #1-B in DRX mode

| Parameter | Unit | Test 1-A |  |  |  |  | Test 1-B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | [-98] |  |  |  |  | [-98] |  |  |  |  |
| Propagation condition |  | [TDL-C 300ns 100Hz] |  |  |  |  | [TDL-C 300ns 100Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.6.5.1.8.1-3: DRX-Configuration for TDD in-sync tests

| Field | Test 1-A | Test 1-B |
| --- | --- | --- |
|  | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |
| drx-LongCycle | [ms40] | [ms40] |
| drx-StartOffset | [ms0] | [ms0] |
| shortDRX | disable | disable |

Table A.6.5.1.8.1-4: TimeAlignmentTimer -Configuration for TDD in-sync testing

| Field | Test 1-A | Test 1-B |
| --- | --- | --- |
|  | Value | Value |
| TimeAlignmentTimer | infinity | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl5] | [sl10] |

Figure A.6.5.1.8.1-1: SNR variation for in-sync testing

A.6.5.1.8.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

<START OF CHANGE 4>

### A.7.5.1	Radio Link Monitoring

In the following section, any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

-	UE output power higher than Transmit OFF power [-50] dBm (as defined in TS 38.101 [TBD]) means uplink signal

-	UE output power equal to or less than Transmit OFF power [-50] dBm (as defined in TS 38.101 [TBD]) means no uplink signal.

### A.7.5.1.1			Radio Link Monitoring Out-of-sync Test for FR2 TDD PCell configured with SSB-based RLM RS in non-DRX mode

#### A.7.5.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR2 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.7.5.1.1.1-1, A.7.5.1.1.1-2, and A.7.5.1.1.1-3 below. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.5.1.1.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Table A.7.5.1.1.1-1: General test parameters for FR2 TDD out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| Active PSCell |  |  | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] | [100] |
| SSB Subcarrier Spacing |  | KHz | [120] | [120] |
| [Data] Subcarrier Spacing |  | KHz | [120] | [120] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX |  |  | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] | [40] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [40] | [40] |
| T1 |  | s | 1 | 1 |
| T2 |  | s | 0.4 | 0.4 |
| T3 |  | s | [0.6] | [0.6] |
| D1 |  | s | [0.24] | [0.44] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |

Table A.7.5.1.1.1-2: Cell specific test parameters for FR2 TDD (cell # 1) for out-of-sync radio link monitoring tests #1 and #2 in non-DRX mode

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | TBD |  |  | TBD |  |  |
| Propagation condition |  | [TDL-A 30ns 75Hz] |  |  | [TDL-A 30ns 75Hz] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |  |

Table A.7.5.1.1.1-3: Measurement gap configuration for TDD out-of-sync tests in non-DRX mode

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [0] |

Figure A.7.5.1.1.1-1: SNR variation for out-of-sync testing

#### A.7.5.1.1.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.5.1.2			Radio Link Monitoring In-sync Test for FR2 TDD PCell configured with SSB-based RLM RS in non-DRX mode

#### A.7.5.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR2 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.7.5.1.2.1-1, A.7.5.1.2.1-2, and A.7.5.1.2.1-3 below. There is one cell (Cell 1), which is the active cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure A.7.5.1.2.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Table A.7.5.1.2.1-1: General test parameters for FR2 TDD in-sync testing in non-DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| Active PSCell |  |  | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] | [100] |
| SSB Subcarrier Spacing |  | KHz | [120] | [120] |
| [Data] Subcarrier Spacing |  | KHz | [120] | [120] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 4 | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 | 0 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX |  |  | OFF | OFF |
| Gap pattern ID |  |  | [N.A.] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 2000 | 2000 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] | [40] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [40] | [40] |
| T1 |  | s | 0.5 | 0.5 |
| T2 |  | s | 0.4 | 0.4 |
| T3 |  | s | [1.46] | [1.36] |
| T4 |  | s | 0.4 | 0.4 |
| T5 |  | s | [1] | [1.1] |
| D1 |  | s | [0.42] | [0.72] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |  |

Table A.7.5.1.2.1-2: Cell specific test parameters for FR2 TDD (cell # 1) for in-sync radio link monitoring tests #1 and test #2 in non-DRX mode

| Parameter | Unit | Test 1 |  |  |  |  | Test 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | TBD |  |  |  |  | TBD |  |  |  |  |
| Propagation condition |  | [TDL-A 30ns 75Hz] |  |  |  |  | [TDL-A 30ns 75Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.1.2.1-3: Measurement gap configuration for TDD in-sync tests in non-DRX mode.

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [TBD] |

Figure A.7.5.1.2.1-1: SNR variation for in-sync testing

#### A.7.5.1.2.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.5.1.3			Radio Link Monitoring Out-of-sync Test for FR2 TDD PCell configured with SSB-based RLM RS in DRX mode

#### A.7.5.1.3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR2 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables A.7.5.1.3.1-1, A.7.5.1.3.1-2, A.7.5.1.3.1-3, A.7.5.1.3.1-4 and A.7.5.1.3.1-5. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.7.5.1.3.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 2.

Table A.7.5.1.3.1-1: General test parameters for FR2 TDD out-of-sync testing in DRX mode

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| Active PSCell |  |  | Cell 1 | Cell 1 |
| RF Channel Number |  |  | 1 | 1 |
| PDCCH parameters |  |  | [R. TBD TDD] | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] | [20] |
| OCNG parameters |  |  | TBD | TBD |
| CP length |  |  | Normal | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] | [100] |
| SSB Subcarrier Spacing |  | KHz | [120] | [120] |
| [Data] Subcarrier Spacing |  | KHz | [120] | [120] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] | [2x2 Low] |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 | 1-0 |
|  | Number of Control OFDM symbols |  | 2 | 2 |
|  | Aggregation level | CCE | 8 | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 | 4 |
|  | DMRS precoder granularity |  | REG bundle size | REG bundle size |
|  | REG bundle size |  | 6 | 6 |
| DRX cycle |  |  | 1280 | 40 |
| Gap pattern ID |  |  | [N.A.] | *[gp0] |
| Layer 3 filtering |  |  | Enabled | Enabled |
| T310 timer |  | ms | 0 | 0 |
| T311 timer |  | ms | 1000 | 1000 |
| N310 |  |  | 1 | 1 |
| N311 |  |  | 1 | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD | TBD |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] | [40] |
| Periodic CSI reporting |  |  | PUCCH | PUCCH |
| CSI reporting periodicity |  | slot | [40] | [40] |
| T1 |  | s | 32 | 4 |
| T2 |  | s | 12.8 | 1.6 |
| T3 |  | s | 13 | 1.8 |
| D1 |  | s | [12.9] | [1.3] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.7.5.1.3.1-2: Cell specific test parameters for FR2 TDD (cell # 1) for out-of-sync radio link monitoring tests #1, and #2 in DRX mode

| Parameter | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| PDCCH_beta | dB | 4 |  |  | 4 |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  | 4 |  |  |
| PBCH_beta | dB | 0 |  |  | 0 |  |  |
| PSS_beta | dB |  |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |  |
| OCNG_beta | dB |  |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | TBD |  |  | TBD |  |  |
| Propagation condition |  | [TDL-A 30ns 75Hz] |  |  | [TDL-A 30ns 75Hz] |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 6: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 7: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 8: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 9: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure TBD. Note 10: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is TBD. |  |  |  |  |  |  |  |

Table A.7.5.1.3.1-3: Measurement gap configuration for TDD out-of-sync tests in DRX mode

| Field | Test 2 |
| --- | --- |
|  | Value |
| gapOffset | [0] |

Table A.7.5.1.3.1-4: DRX-Configuration for TDD out-of-sync tests

| Field | Test 1 | Test 2 |
| --- | --- | --- |
|  | Value | Value |
| drx-onDurationTimer | [ms6] | [ms6] |
| drx-InactivityTimer | [ms1] | [ms1] |
| drx-RetransmissionTimerDL | [sl1] | [sl1] |
| drx-RetransmissionTimerUL | [sl1] | [sl1] |
| drx-LongCycle | [ms1280] | [ms40] |
| drx-StartOffset | [ms0] | [ms0] |
| shortDRX | disable | Disable |

Table A.7.5.1.3.1-5: TimeAlignmentTimer -Configuration for TDD out-of-sync testing

| Field | Test1 | Test2 |
| --- | --- | --- |
|  | Value | Value |
| TimeAlignmentTimer | [Infinity] | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl40] | [sl40] |

Figure A.7.5.1.3.1-1: SNR variation for out-of-sync testing

#### A.7.5.1.3.2	Test Requirements

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

### A.7.5.1.4			Radio Link Monitoring In-sync Test for FR2 TDD PCell configured with SSB-based RLM RS in DRX mode

#### A.7.5.1.4.1	Test Purpose and Environment

The purpose of this test is to verify that the UE properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell when DRX is used. This test will partly verify the FR2 TDD radio link monitoring requirements in clause 8.1.

The test parameters are given in Tables Table A.7.5.1.4.1-1, Table A.7.5.1.4.1, Table A.7.5.1.4.1-3 and Table A.7.5.1.4.1-4. There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure Table A.7.5.1.4.1-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [5] ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

Table A.7.5.1.4.1-1: General test parameters for FR2 TDD in-sync testing in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PSCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| PDCCH parameters |  |  | [R. TBD TDD] |
| SSB Periodicity |  | ms | [20] |
| OCNG parameters |  |  | TBD |
| CP length |  |  | Normal |
| NR Channel Bandwidth (BWchannel) |  | MHz | [100] |
| SSB Subcarrier Spacing |  | KHz | [120] |
| [Data] Subcarrier Spacing |  | KHz | [120] |
| Correlation Matrix and Antenna Configuration |  |  | [2x2 Low] |
| In sync transmission parameters (Note 1) | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| Out of sync transmission parameters (Note 1) | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX cycle |  | ms | 40 |
| Gap pattern ID |  |  | [N.A.] |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 2000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| Number of NZP CSI-RS resource within the NZP CSI-RS resource set 1 for beam management |  |  | TBD |
| NZP CSI-RS periodicity in NZP CSI-RS resource set 1 |  | slot | [40] |
| Periodic CSI reporting |  |  | PUCCH |
| CSI reporting periodicity |  | slot | [40] |
| T1 |  | s | 4 |
| T2 |  | s | 1.6 |
| T3 |  | s | [1.36] |
| T4 |  | s | 0.4 |
| T5 |  | s | 4 |
| D1 |  | s | [1] |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: preambleTransMax value of [200], periodicity of PRACH occasion of [160 ms] and ra-ResponseWindow of [sl40] are configured for beam failure recovery. Note 3: E-UTRAN is in non-DRX mode under test. |  |  |  |  |

Table A.7.5.1.4.1-2: Cell specific test parameters for FR2 TDD (cell # 1) for in-sync radio link monitoring tests #1 in DRX mode

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| PDCCH_beta | dB | 4 |  |  |  |  |
| PDCCH_DMRS_beta | dB | 4 |  |  |  |  |
| PBCH_beta | dB | 0 |  |  |  |  |
| PSS_beta | dB |  |  |  |  |  |
| SSS_beta | dB |  |  |  |  |  |
| PDSCH_beta | dB |  |  |  |  |  |
| OCNG_bete | dB |  |  |  |  |  |
| SNR | dB | TBD | TBD | TBD | TBD | TBD |
|  | dBm/15 kHz | TBD |  |  |  |  |
| Propagation condition |  | [TDL-A 30ns 75Hz] |  |  |  |  |
| Note 1: tdd-UL-DL-configuration is TBD. Note 2: OCNG shall be used such that the resources in cell # 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: NZP CS-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure TBD. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section TBD. |  |  |  |  |  |  |

Table A.7.5.1.4.1-3: DRX-Configuration for TDD in-sync tests

| Field | Test 1 |
| --- | --- |
|  | Value |
| drx-onDurationTimer | [ms6] |
| drx-InactivityTimer | [ms1] |
| drx-RetransmissionTimerDL | [sl1] |
| drx-RetransmissionTimerUL | [sl1] |
| drx-LongCycle | [ms40] |
| drx-StartOffset | [ms0] |
| shortDRX | disable |

Table A.7.5.1.4.1-4: TimeAlignmentTimer -Configuration for TDD in-sync testing

| Field | Test 1 |
| --- | --- |
|  | Value |
| TimeAlignmentTimer | infinity |
| periodicityAndOffset in SchedulingRequestResourceConfig | [sl40] |

Figure A.7.5.1.4.1-1: SNR variation for in-sync testing.

#### A.7.5.1.4.2	Test Requirements

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90%.

<END OF CHANGE 4>