3GPP TSG-RAN WG4 Meeting #88bis               	         R4-1812519

# Chengdu, CN, 8th – 12th Oct, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on beam failure detection requirements (Section 8.5.2.2 and Section 8.5.3.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Clarification on beam failure detection requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification on beam failure detection requirements: - Applicability of N=1 condition for evalation period in FR2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for beam failure detection are not clearly defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2.2, 8.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of Change #1 >>

- 8.5.2.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB [ms] period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB [ms] period.

The value of TEvaluate_BFD_SSB is defined in Table 8.5.2.2-1 for FR1.

The value of TEvaluate_BFD_SSB is defined in Table 8.5.2.2-2 for FR2 with

-	N=1, only if the SSB configured for BFD is not configured for L1-RSRP, and

if UE is not provided higher layer parameter  failureDetectionResources and UE is provided by higher layer parameter TCI-state for PDCCH SSB that has QCL-TypeD, or

if the SSB configured for BFD is QCL-Type D with DM-RS for PDCCH and the QCL association is known to UE, or

if the SSB configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting and the QCL association is known to UE, and a CSI report with L1-RSRP measurement for the SSB configured for BFD has been made within [TBD]ms;

-	N=FFS, otherwise.

<< End of Change #1>>

<< Start of Change #2 >>

#### 8.5.3.2	Minimum requirement

Editor’s note: The requirements below have been derived without considering gap sharing when CSI-RS resource for BFD are partially overlapping with measurement gaps.

UE shall be able to evaluate whether the downlink radio link quality on the configured CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS [ms] period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS [ms] period.

The value of TEvaluate_BFD_CSI-RS is defined in Table 8.5.3.2-1 for FR1.

The value of TEvaluate_BFD_CSI-RS is defined in Table 8.5.3.2-2 for FR2 with

-	N=1, only if the CSI-RS for BFD is not configured for L1-RSRP, or the CSI-RS for BFD is configured for L1-RSRP with repetition-OFF and the TCI is given and QCL-D to SSB or CSI-RS with repetition-ON. And,

if UE is not provided higher layer parameter  failureDetectionResources and UE is provided by higher layer parameter TCI-state for PDCCH CSI-RS that has QCL-TypeD, or

if the CSI-RS configured for BFD is QCL-Type D with DM-RS for PDCCH and the QCL association is known to UE, or

if the CSI-RS resource configured for BFD is QCL-Type D and TDMed to CSI-RS resources configured for L1-RSRP reporting or SSBs configured for L1-RSRP reporting, all CSI-RS resources configured for BFD are mutually TDMed, and the QCL association is known to UE and a CSI report with L1-RSRP measurement for the CSI-RS configured for BFD has been made within [TBD]ms;

-	N=FFS, otherwise.

<< End of Change #2>>