3GPP TSG RAN WG4 Meeting #88bis		R4-1812708

Chengdu, CN, 8th – 12th Oct, 2018

Source: 	NTT DOCOMO, INC.

Title: 	Intra-frequency measurement requirements in DRX mode

Agenda item:	7.11.3.2.2

Document for:	Discussion

- Introduction

NR intra-frequency measurement requirement is still under discussion, especially the measurement requirement in non-DRX mode is discussed so far. Regarding the measurement requirements in DRX mode, although some contributions were proposed in the last RAN4 meeting, there was no agreement. In this contribution, we provide our views on intra-frequency measurement requirements in DRX mode.

- Discussion

Regarding measurement requirements in DRX mode in case of LTE-DC, following sentences are captured in [1], i.e., intra-frequency measurements in DRX mode apply and depend on the DRX cycle configured on each cell group. In NR DC case including EN-DC, NE-DC and NR-NR DC, the same principle as LTE-DC should be applied, otherwise unexpected measurement delay would be occurred. In case of EN-DC, for example, intra-frequency requirements in DRX mode for LTE PCell should apply the MCG DRX cycle and intra-frequency requirements in DRX mode for NR PSCell should apply the SCG DRX cycle.

### 8.8.2 Intra-frequency measurements requirements on PCell PCell intra-frequency measurements shall meet all applicable requirements in clause 8.1.2.2. If MCG DRX is in use, then the PCell intra-frequency requirements for when DRX is in use in clause 8.1.2.2 shall apply and shall depend on the MCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply. The applicable measurement accuracy requirements are in clause 9.1. ### 8.8.3 Intra-frequency measurements requirements on PSCell PSCell starts with activated state upon configuration and cannot be deactivated. PSCell intra-frequency measurements shall meet all applicable requirements in clause 8.1.2.2. If SCG DRX is in use, then the PSCell intra-frequency requirements for when DRX is in use in clause 8.1.2.2 shall apply and shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply. The applicable measurement accuracy requirements are in clause 9.1. ### 8.8.4 Inter-frequency and inter-RAT measurement requirements Inter-frequency measurements shall meet all applicable requirements in clause 8.1.2.3. If MCG DRX is in use, then the inter-frequency requirements for when DRX is in use in clause 8.1.2.3 shall apply and shall depend on the MCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply. The applicable measurement accuracy requirements are in clause 9.1. Inter-RAT measurements shall meet all applicable requirements in clause 8.1.2.4. If MCG DRX is in use, then the inter-RAT requirements for when DRX is in use in clause 8.1.2.4 shall apply and shall depend on the MCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply. The applicable measurement accuracy requirements are in clause 9.2, 9.3, 9.4 and 9.5.

Proposal 1:

In NR DC case including EN-DC, NE-DC and NR-NR DC, the same principle as LTE-DC should be applied. In case of EN-DC, for example, intra-frequency requirements in DRX mode for LTE PCell should apply the MCG DRX cycle and intra-frequency requirements in DRX mode for NR PSCell should apply the SCG DRX cycle.

In addition, measurement period in DRX mode for RLM and beam failure detection should also be discussed. We think that same principle as our proposal 1 can be applied to such cases.

Proposal 2:

Measurement period in DRX mode for RLM and beam failure detection should apply the same principle as proposal 1.

Regarding inter-frequency measurement requirements in DRX mode, we mentioned our views in the other paper [2].

- Conclusion

In this contribution, we provided our view on general principle for NR RRM test cases. Our observations and proposals are as follows:

Proposal 1:

In NR DC case including EN-DC, NE-DC and NR-NR DC, the same principle as LTE-DC should be applied. In case of EN-DC, for example, intra-frequency requirements in DRX mode for LTE PCell should apply the MCG DRX cycle and intra-frequency requirements in DRX mode for NR PSCell should apply the SCG DRX cycle.

Proposal 2:

Measurement period in DRX mode for RLM and beam failure detection should apply the same principle as proposal 1.

- Reference

[1] TS36.133

[2] R4-1812710, “Inter-frequency measurement requirements in DRX mode”, NTT DOCOMO, INC.