3GPP TSG-RAN WG4 Meeting #88bis	R4-1812849

Chengdu, China, 8 – 12 October 2018

Agenda Item:	7.12.5.3

Source:	Ericsson

Title:	Test points for NR RLM test

Document for:	Discussion

# 1	Introduction

We have shown the PDCCH simulation results [1]. We discuss how to derive the SNR test points based on the results.

# 2	Test points for RLM tests

Figure 1 illustrates the RLM test points used for LTE. As it is observed, RLM test is performed with 5 test points: SNR1 (initial), SNR2 (above Qout), SNR3 (below Qout), SNR4 (below Qin), and SNR5 (above Qin). Each SNR test points are derived with the following formula:

SNR2 = Qout + margin1

SNR3 = Qout - margin1

SNR4 = Qin - margin2

SNR5 = Qin + margin2

SNR1 = SNR5

Figure 1	Illustration of SNR test points for RLM tests.

For LTE, magin1 is set to 2dB for static channel and 3dB for fading channel. Also, margin2 is set to 2dB for static channel and 2.5dB for fading channel. If we follow the same assumption, the SNR values are derived as shown in Table 3.

Table 1	Test points for RLM tests derived from the same assumption as LTE.

| Test number | SCS | Propagation channel | Antenna configuration | SNR1 (dB) | SNR2 (dB) | SNR3 (dB) | SNR4 (dB) | SNR5 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15kHz | AWGN | 2x2 | -3.9 | -9.5 | -13.5 | -7.9 | -3.9 |
| 2 |  |  | 2x4 | -5.3 | -11 | -15 | -9.3 | -5.3 |
| 3 |  | TDL-C 300ns, 100Hz | 2x2 | 2.6 | -4.3 | -10.3 | -2.4 | 2.6 |
| 4 |  |  | 2x4 | -0.4 | -7 | -13 | -5.4 | -0.4 |
| 5 | 30kHz | AWGN | 2x2 | -3.9 | -9.5 | -13.5 | -7.9 | -3.9 |
| 6 |  |  | 2x4 | -5.4 | -11 | -15 | -9.4 | -5.4 |
| 7 |  | TDL-C 300ns, 100Hz | 2x2 | 2.8 | -4.3 | -10.3 | -2.2 | 2.8 |
| 8 |  |  | 2x4 | -0.3 | -7.2 | -13.2 | -5.3 | -0.3 |
| 9 | 120kHz | AWGN | 2x2 | -3.4 | -8.5 | -14.5 | -8.4 | -3.4 |
| 10 |  | TDL-A 30ns, 75Hz | 2x2 | 1 | -7.1 | -13.1 | -4 | 1 |

We propose to discuss how to derive SNR test points from Qin/Qout. One option is to follow LTE as we discussed so far. However since NR uses different SCS and carrier frequency from LTE. Also SSB with SCS=240kHz is different from PDCCH SCS=120kHz. Therefore RAN4 should discuss further the formula and margin to derive SNR test point. For reference, we provide the generalized formula for SNR test points as follows.

SNR2 = Qout + margin1_1

SNR3 = Qout - margin1_2

SNR4 = Qin - margin2_1

SNR5 = Qin + margin2_2

SNR1 = SNR5

Table 2	Margin for each scenario.

| Reference symbols | SCS for ref symbol | SCS for PDCCH | Propagation channel | Margin1_1 (dB) | Margin1_2 (dB) | Margin2_1 (dB) | Margin2_2 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CSI-RS | 15kHz | 15kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-C 300ns, 100Hz) |  |  |  |  |
|  | 30kHz | 30kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-C 300ns, 100Hz) |  |  |  |  |
|  | 120kHz | 120kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-A 30ns, 75Hz) |  |  |  |  |
| SS/PBCH | 15kHz | 15kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-C 300ns, 100Hz) |  |  |  |  |
|  | 30kHz | 30kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-C 300ns, 100Hz) |  |  |  |  |
|  | 120kHz | 120kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-A 30ns, 75Hz) |  |  |  |  |
|  | 240kHz | 120kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-A 30ns, 75Hz) |  |  |  |  |

# 4	Conclusion

Proposal 1: RAN4 should discuss the formula to derive SNR test points from Qin/Qout whether we can reuse the same formula as LTE.

SNR2 = Qout + margin1_1

SNR3 = Qout - margin1_2

SNR4 = Qin - margin2_1

SNR5 = Qin + margin2_2

SNR1 = SNR5

Proposal 2: RAN4 should discuss the margin(s) to derive SNR test points for RLM tests.

| Reference symbols | SCS for ref symbol | SCS for PDCCH | Propagation channel | Margin1_1 (dB) | Margin1_2 (dB) | Margin2_1 (dB) | Margin2_2 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CSI-RS | 15kHz | 15kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-C 300ns, 100Hz) |  |  |  |  |
|  | 30kHz | 30kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-C 300ns, 100Hz) |  |  |  |  |
|  | 120kHz | 120kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-A 30ns, 75Hz) |  |  |  |  |
| SS/PBCH | 15kHz | 15kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-C 300ns, 100Hz) |  |  |  |  |
|  | 30kHz | 30kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-C 300ns, 100Hz) |  |  |  |  |
|  | 120kHz | 120kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-A 30ns, 75Hz) |  |  |  |  |
|  | 240kHz | 120kHz | Static (AWGN) |  |  |  |  |
|  |  |  | Fading (TDL-A 30ns, 75Hz) |  |  |  |  |

# References

- R4-1812848, “PDCCH simulation results for NR RLM tests”, Ericsson.