3GPP TSG-RAN WG4 Meeting #88bis                 	R4-1813216

Chengdu, China, 8 – 12 October 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	NR PUSCH simulation results

Agenda item:	7.13.2.2

Document for:	Discussion

Introduction

In the RAN4#88 meeting, a way forward was agreed for NR PUSCH demodulation in [1]. In this contribution we provide simulation results with the simulation assumptions agreed in the WF.

Simulation results

Transform precoding disabled

For transform precoding disabled case, the following simulation assumptions were agreed in [1]:

Table 1: Simulation assumptions for simulation alignment – Transform precoding disabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1, 2 | 1 |
| Number of Rx | 2, 4, 8 | 1, 2 |
| Number of layers | 1, 2 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | - |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1, 1+1 | 1 |
| symbols length | 14 | [7] |
| start symbol index | 0 | [0 or 2 or 3] |
| Time domain resource allocation type | type A | type B |
| Frequency domain resource | full RB allocation of the applicable BW | full RB allocation of the applicable BW |
| MCS index | 2, 16, 20 | 2, 16, 20 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | TDL-A 30ns 10Hz | AWGN |
| SCS and BW | 15kHz: 10 30kHz: 40 | 60kHz: 100 120kHz: 100 |
| PTRS | Not configured | Configured with KPTRS =2, LPTRS =1 |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Simulation results with these parameters are shown in Table 2.

Table 2: Simulation results for Transform precoding disabled case

| PUSCH 1Tx2Rx - CP-OFDM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Frequency range | SCS, BW | MCS | DMRS pattern | SINR [dB] |
| FR1 | 15kHz, 10MHz | MCS2 | DMRS 1 | -2.1 |
|  |  |  | DMRS 1+1 | -1.9 |
|  |  | MCS16 | DMRS 1 | 9.2 |
|  |  |  | DMRS 1+1 | 9.5 |
|  |  | MCS20 | DMRS 1 | 12.5 |
|  |  |  | DMRS 1+1 | 12.8 |
|  | 30kHz, 40MHz | MCS2 | DMRS 1 | -3 |
|  |  |  | DMRS 1+1 | -2.8 |
|  |  | MCS16 | DMRS 1 | 8.6 |
|  |  |  | DMRS 1+1 | 9.1 |
|  |  | MCS20 | DMRS 1 | 12.1 |
|  |  |  | DMRS 1+1 | 12.4 |
| FR2 | 60kHz, 100MHz | MCS2 | DMRS 1 | -4.1 |
|  |  | MCS16 | DMRS 1 | 7.2 |
|  |  | MCS20 | DMRS 1 | 10.5 |
|  | 120kHz, 100MHz | MCS2 | DMRS 1 | -3.9 |
|  |  | MCS16 | DMRS 1 | 7.4 |
|  |  | MCS20 | DMRS 1 | 10.7 |

Transform precoding enabled

For transform precoding enabled case, the following simulation assumptions were agreed in [1]:

Table 3: Simulation assumptions for simulation alignment – Transform precoding enabled

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | enabled | enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| DMRS type | type 1 | Type 1 |
| Number of DMRS symbols | [1] | [1] |
| symbols length | 14 | [7] |
| start symbol index | 0 | [0 or 2 or 3] |
| Time domain resource allocation type | type A | type B |
| Frequency domain resource | PRB Number: [50] | PRB Number: [30] |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation channel | TDL-A 30ns 10Hz | AWGN |
| SCS and BW | 30kHz: 40 | 120kHz: 100 |
| PTRS | Not configured | [NRB0=0, NRB1=8, NRB2=NRB3=32, and NRB4=108] |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group | Disabled | Disabled |
| Frequency hopping | Disabled | Disabled |
| Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @70% of maximum throughput | SNR @70% of maximum throughput |

Simulation results with these parameters are shown in Table 4.

Table 4: Simulation results for Transform precoding enabled case

| PUSCH 1Tx2Rx – DFT-s-OFDM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Frequency range | SCS, BW | MCS | DMRS | SINR [dB] |
| FR1 | 30kHz, 40MHz | MCS2 | DMRS 1 | -2.7 |
| FR2 | 120kHz, 100MHz | MCS2 | DMRS 1 | -5 |

Conclusion

In this contribution we have provided simulation results for NR PUSCH.

- References

[1] R4-1811694, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, 3GPP TSG-RAN WG4 RAN#88, Gothenburg, SE, 20th – 24th Aug, 2018.