3GPP TSG-RAN WG4 Meeting #88Bis	R4-1813256

Chengdu, China, 08-12 October 2018

Title:	Updating simulation assumption for sTTI UE demodulation

Agenda Item:	6.7.7.1.1

Source:	Huawei, Hisilicon

Document for:	Discussion

1	General

1.1	Introduction

The simulation assumptions for PDSCH and PDCCH are aligned that resulted in R4-1805493 Update of simulation assumption for sTTI UE demodulation and CSI requirements. However, there are some changes during the last several meetings going. Therefore, this contribution is proposed for making a summary of changes on simulation assumptions so that there will be only one proposal that needs to be referred to when doing further related simulations.

2	UE demodulation requirements

2.1	Simulation assumption for Slot-PDSCH/Subslot-PDSCH

2.1.1	Minimum processing time

- Subslot-based transmission:

  - UE can choose set1 with m+4 or set 2 with m+6 according to UE capability.

- Slot-based transmission:

  - Minimum processing time for: m+4

2.1.2	Test parameters

Table 2.1.2-1 Test parameters for SPDSCH simulation

|  | Test 1 | Test 2 |
| --- | --- | --- |
| Bandwidth | 10MHz | 10MHz |
| Number of PDCCH symbols (CFI) | 2 | 2 |
| Cyclic prefix | Normal | Normal |
| Subframe configuration | Non-MBSFN | Non-MBSFN |
| DMRS sharing (applicable only for subslot TTI) | N/A | Enabled (Note 4) |
| Propagation channel | EVA30 | EPA5 |
| Antenna configuration | 4x2 Low | 2x2 Low |
| Transmission modes | TM3 dual layer | TM9 dual layer |
| Antenna ports | 0-3 | 7 and 8 for dual layer |
| Precoder update granularity | N/A | 1 PRG every 1ms |
| Beamforming precoder | N/A | Random precoder (TS36.101 B.4.1 for single layer, B.4.2 for dual layer) (Note 5) |
| TTI length (Note 1) | Slot-based / Subslot-based | Slot-based / Subslot-based |
| MCS | 16QAM 1/2 for slot-based 0.45 for subslot-based | QPSK 1/3 |
| Duplex mode (Note 2) | FDD / TDD | FDD / TDD |
| Evaluation criteria | 70% of Max Tput | 70% of Max Tput |
| CSI-RS signals | N/A | Antenna ports 15, 16, |
| CSI-RS periodicity and subframe offset | N/A | 5/ 2 |
| CSI-RS configuration | N/A | 0 |
| ZP-CSI-RS configuration | N/A | Not configured |
| v_shift (mod(cell_ID, 6)) | 0 | 0 |
| Associated SPDCCH transmission | CRS-based, Localized | CRS-based, Localized |
| Aggregation level | [2 SCCE] | [4 SCCE] |
| Used/Unused SPDCCH resource indication (Note 6) | ’10’ | ’10’ |
| DCI format | 7-1B | 7-1F |
| Configured SPDCCH-PRB-sets | {Set 1, Set 2} | {Set 3, Set 4} |
| SPDCCH-PRB-set used for DCI transmission | Set 1 for slot-based Set 2 for subslot-based | Set 3 for slot-based Set 4 for subslot-based |

2.1.3	SPDCCH-PRB-set configuration

Table 2.1.3-1 Configuration for SPDCCH

|  | Set 1 | Set 2 | Set 3 | Set 4 |
| --- | --- | --- | --- | --- |
| Reference symbol | CRS | CRS | CRS | CRS |
| Transmission type | Localized | Localized | Localized | Localized |
| Number of OFDM symbols | 1 | 1 | 2 | 2 |
| Number of PRBs | 16 | 16 | 8 | 8 |
| Allocated PRBs | {0, 1, …, 15} | {34, 35, …, 49} | {0, 1, …, 7} | {8, 9, …, 15} |
| Rate matching mode | Mode 1 | Not configured | Not configured | Mode 1 |
| SPDCCH L1 Reuse Indication | Not configured | {2, 0} | {1, 1} | Not configured |
| Total number of SCCEs | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) | 4 (16 SREGs) |

2.2	FRC tables (For information)

2.2.1	CRS-based PDSCH

2.2.1.1	Slot-based transmission

Table 2.2.1.1-1 FRC table for CRS-based, Slot-based PDSCH

|  | Unit | Test 1-2 (Ref. R.11) |
| --- | --- | --- |
| Description |  | 4 CRS ports |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | ½ |
| Number of HARQ processes |  | TBD |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per slot |  |  |
| For Sub-Frame 1,2,3,4,6,7,8,9 | Bits |  |
| Slot index 0 | Bits | 11448 |
| Slot index 1 | Bits | 14112 |
| For Sub-Frame 0 & 5 |  | N/A |
| Binary channel bits per slot |  |  |
| For Sub-Frame 1,2,3,4,6,7,8,9 | Bits |  |
| Slot index 0 | Bits | 22400 |
| Slot index 1 | Bits | 28288 |
| For Sub-Frame 0 & 5 |  | N/A |
| Note: Given per component carrier per codeword. |  |  |

2.2.1.2	Subslot-based transmission

Table 2.2.1.2-1 FRC table for CRS-based, Subslot-based PDSCH

|  | Unit | Test 1-2 (Ref. R.11) |
| --- | --- | --- |
| Description |  | 4 CRS ports |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/2 |
| Number of HARQ processes |  | 8 |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per subslot |  |  |
| For Sub-Frame 1,2,3,4,6,7,8,9 |  |  |
| Subslot index 1 | Bits | 5160 |
| Subslot index 2 | Bits | 3880 |
| Subslot index 3 | Bits | 2664 |
| Subslot index 4 | Bits | 3880 |
| Subslot index 5 | Bits | 5160 |
| For Sub-Frame 0 & 5 |  | N/A |
| Binary channel bits per subslot |  |  |
| For Sub-Frame 1,2,3,4,6,7,8,9 |  |  |
| Subslot index 1 | Bits | 12032 |
| Subslot index 2 | Bits | 8832 |
| Subslot index 3 | Bits | 5888 |
| Subslot index 4 | Bits | 8832 |
| Subslot index 5 | Bits | 12288 |
| For Sub-Frame 0 & 5 |  | N/A |
| Note: Given per component carrier per codeword. |  |  |

2.2.2	DMRS-based PDSCH

2.2.2.1	Slot-based transmission

Table 2.2.2.1-1 FRC table for DMRS-based, Slot-based PDSCH

|  | Unit | Test 2 (Ref. R.66) |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | QPSK |
| Target coding rate |  | 1/3 |
| Number of HARQ processes |  | [8] |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per slot |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Slot index 0 | Bits | 3624 |
| Slot index 1 | Bits | 4392 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Slot index 0 | Bits | 2856 |
| Slot index 1 | Bits | 4392 |
| For Sub-Frame 0 & 5 |  | N/A |
| Binary channel bits per slot |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Slot index 0 | Bits | 10000 |
| Slot index 1 | Bits | 13360 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Slot index 0 | Bits | 9600 |
| Slot index 1 | Bits | 13360 |
| For Sub-Frame 0 & 5 |  | N/A |
| Note: CSI-RS configuration = 0 for NZP-CSI-RS Note: Given per component carrier per codeword. |  |  |

Number of bits changes whether CSI-RS is transmitted within the same slot.

2.2.2.2	Subslot-based transmission

Table 2.2.2.2-1 FRC table for DMRS-based, Subslot-based PDSCH

|  | Unit | Test 2 (Ref. R.44) |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| Allocated resource blocks |  | 50 |
| Modulation |  | 16QAM |
| Target coding rate |  | 1/3 |
| Number of HARQ processes |  | FFS |
| Maximum number of HARQ transmissions |  | 4 |
| Information bit payload per subslots |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Subslot index 1 | Bits | 1736 |
| Subslot index 2 | bits | 1192 |
| Subslot index 3 | Bits | 776 |
| Subslot index 4 | Bits | 1192 |
| Subslot index 5 | Bits | 1736 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Subslot index 1 | Bits | 1736 |
| Subslot index 2 | bits | 1192 |
| Subslot index 3 | Bits | 776 |
| Subslot index 4 | Bits | 1192 |
| Subslot index 5 | Bits | 1736 |
| For Sub-Frame 0 & 5 |  | N/A |
| Binary channel bits per subslots |  |  |
| For Sub-Frames 1,3,4,6,8,9 |  |  |
| Subslot index 1 | Bits | 4960 |
| Subslot index 2 | bits | 4032 |
| Subslot index 3 | Bits | 2688 |
| Subslot index 4 | Bits | 4032 |
| Subslot index 5 | Bits | 5088 |
| For Sub-Frames 2,7 (2 NZP-CSI-RS ports) |  |  |
| Subslot index 1 | Bits | 4960 |
| Subslot index 2 | bits | 3696 |
| Subslot index 3 | Bits | 2688 |
| Subslot index 4 | Bits | 4032 |
| Subslot index 5 | Bits | 5088 |
| For Sub-Frame 0 & 5 |  | N/A |
| Note: CSI-RS configuration = 0 for NZP-CSI-RS Note: Given per component carrier per codeword. |  |  |

3. Conclusion

Proposal 1: We propose to use Table 2.1.2-1 as the updated simulation assumption for PDSCH.

Proposal 2: We propose to use Table 2.2.1.1-1 and 2.2.1.2-1 as the updated FRC table for CRS-based PDSCH.

Proposal 3: We propose to use Table 2.2.2.1-1 and 2.2.2.2-1 as the updated FRC table for DMRS-based PDSCH.

References

- R4-1805493, “Update of simulation assumption for sTTI UE demodulation and CSI requirements”, Ericsson.

- R4-1809375, “Way forward on reference channels for sTTI demodulation and CSI requirements”, Qualcomm, Ericsson, Huawei.