3GPP TSG-RAN WG4 #88bis	R4-1813376

October 8th ‒ 12th, 2018

Chengdu, CN

Source: 	Qualcomm Incorporated

Title: 	AMPR for NS_15 for CAT M2 Sub-PRB

Agenda item:	6.5.2.1

Document for:	Approval

# Introduction

In [1], AMPR for NS_12 has been proposed. In this paper, we present our results and proposals for the same.

The simulations were performed for 2 tone BPSK while for 3 and 6 tones QPSK modulation was used.

# Discussion

For this NS value and many other NS cases, we think that the center frequency needs to be specified as well as only certain frequency ranges need this AMPR. Below is a figure of AMPR required for 5 MHz BW for NS15.

Similarly, AMPR sims were performed for 10 and 15 MHz BWs as well. Below is the proposal for AMPR for NS_15 for 5, 10 and 15 MHz BWs.

Proposal:

| BW [MHz] | 5 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Fc | 846.5 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (0, <54) | (0, <54) | (0, <54) | (0, 200-234) | (0, >234) | (0, >234) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤10.5 | ≤8.5 | ≤5.5 | ≤3.5 | ≤10 | ≤9 |
| BW [MHz] | 10 MHz |  |  |  |  |  |
| Fc | 844 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (4, <48) | (4, <48) | (4, <48) | (4, 200-234) | (4, >234) | (4, >234) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤10.5 | ≤8.5 | ≤5.5 | ≤3.5 | ≤10 | ≤9 |
| BW [MHz] | 10 MHz |  |  |  |  |  |
| Fc | 844 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (3, <18) | (3, <18) | (3, <18) | (3, 240-270) | (3, >270) | (3, >270) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤10.5 | ≤8.5 | ≤5.5 | ≤3.5 | ≤10 | ≤9 |
| BW [MHz] | 10 MHz |  |  |  |  |  |
| Fc | 844 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (2, >246) | (1, >276) |  |  |  |  |
| Lcsc | ≥2 | ≥2 |  |  |  |  |
| AMPR [dB] | ≤3.5 | ≤3.5 |  |  |  |  |
| BW [MHz] | 15 MHz |  |  |  |  |  |
| Fc | 841.5 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (8, <48) | (8, <48) | (8, <48) | (8, 200-234) | (8, >234) | (8, >234) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤10.5 | ≤8.5 | ≤5.5 | ≤3.5 | ≤10 | ≤9 |
| BW [MHz] | 15 MHz |  |  |  |  |  |
| Fc | 841.5 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (7, <18) | (7, <18) | (7, <18) | (7, 240-270) | (7, >270) | (7, >270) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤10.5 | ≤8.5 | ≤5.5 | ≤3.5 | ≤10 | ≤9 |
| BW [MHz] | 15 MHz |  |  |  |  |  |
| Fc | 841.5 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (6, >246) | (5, >276) |  |  |  |  |
| Lcsc | ≥2 | ≥2 |  |  |  |  |
| AMPR [dB] | ≤3.5 | ≤3.5 |  |  |  |  |

# Conclusion

In this paper, AMPR for NS_15 has been proposed for BWs 5, 10 and 15 MHz for CAT M2.

# References

[1] R4-1810488, A-MPR of CAT-M2 device for subPRB allocation, 3GPP TSG-RAN4 Meeting #87, Gothenburg, Sweden, 20 August – 24 August 2018