TSG-RAN Working Group 4 (Radio) meeting #88bis	R4-1813547

Chengdu, China, 8th – 12th October 2018

Source:	Ericsson

Title:	Conditions to apply antenna directivity when calculate TRP

Agenda item:	6.1.3.4

Document for:	Approval

- Introduction

For in-band and adjacent frequency bands, e.g. Out-of-band (OOB) emissions and TRP values for ACLR it is suggested that a measured peak EIRP value divided by a declared/calculated directivity value can be used to assess the TRP. For example, in case of antenna arrays, a popular way to estimate directivity is given by 10lgN, where N is the number of antenna elements.

To be able to utilize this methodology we need to understand what the uncertainty of the assumed directivity value is.

- Discussion

Measurement in a single point is clearly not sufficient to judge whether a declared or calculated directivity value agrees with reality.

In adjacent channels the radiation can be highly correlated, resulting in a directivity close to the in-band value. However, it can also be totally un-correlated by coincidence or by design, case when the directivity will be smaller than in the case of full correlation, which leads to a risk of under-estimating the TRP by a large margin.

## 2.1 Antenna pattern metrics

The antenna pattern can be simulated if enough information is available. The accuracy of the result can be tested by measurements of certain parameters in certain angular directions. Since the EIRP peak value will be measured there is no need to check absolute values. Only relative parameters are needed.

| Parameter | Definition |
| --- | --- |
| Horizontal Half Power Beamwidth (H-HPBW) Beam Width points | Difference in theta angle between -3 dB points |
| Vertical Half Power Beamwidth (V-HPBW) Beam Width points | Difference in phi angle between -3 dB points |
| Horizontal First null beam width (H-FNBW) nulls | Difference between phi angles of the first nulls |
| Vertical First null beam width (H-FNBW) nulls | Difference between theta angles of the first nulls |
| Side lobe level of closest side lobes | At first sidelobes (4 lobes) the level relative to the main beam peak |
| Scan loss at predefined directions corresponding to moderate predefined beam steering directions | Peak value ratio: steered vs not steered beam |

Note: modelling of extreme scanning angles requires accurate modelling of mutual coupling. Scan loss at lower scanning angles reveals the gain drop due to the element pattern.

Figure 1 Half power beam width (black dashed lines), first nulls (red dashed lines), first sidelobe (purple dashed lines) could be used as control points of the simulation model.

Figure 2 Scan loss (indicated by purple dashed lines) could be used as control points. The green and orange patterns depict scanned beams to -/+ 30 degrees, respectively.

Remark: By using numerical optimization (convex optimization) the antenna weights can be adjusted to any values at N control points, where N is the number of antenna elements. To avoid a large number of control points, there must be additional information provided by the model.

- Conclusion

The usage of the formula TRP = EIRP/D where EIRP is measured and D is calculated, needs to be complemented by MU-analysis. A low number of control points alone, is not enough to guarantee a reasonable agreement between measurement and model.

Proposal: Information about the conditions necessary for correct usage of the formula TRP = EIRP/D, where EIRP is measured and D is calculated, shall be introduced in the TR and TS.