# 3GPP TSG- RAN WG4 Meeting #88bis 	R4-1813879

# Chengdu, China, 8-12 Aug 2018

Source: 	Huawei

Title: 	WF on power boosting

Agenda Item:	7.9.2.3

Document for:	Approval

# Introduction

From the AH meeting held Monday evening mainly [25],

- Huawei to lead a WF on power boosting.

# Discussion

## Contributions

R4-1812270, “Discussion on NR Test model”, CATT

R4-1812579, “Discussion on TDD configurations for NR test model”, ZTE

R4-1812581, “TDD configuration for NR BS test model for FR2”, NTT DoCoMo

R4-1812595, “Further discussion on the remaining issues for NR test model”, ZTE

R4-1812601, “TP to TS 38.141-1: FR1 test model(4.9.2)”, ZTE

R4-1812603, “TP to TS 38.141-2: FR2 test model(Section 4.9.3)”, ZTE

R4-1812743, “Power boosting/deboosting configuration for NR Test Models”, Ericsson

R4-1812744, “TP to TS 38.141-1: Section 4.9.2.3 Data content of PHY channels”, Ericsson

R4-1812745, “TP to TS 38.141-2: Section 4.9.2.3 Data content of PHY channels”, Ericsson

R4-1812746, “TP to TS 38.141-1: Section 4.9.3.2.2 NR test model 1”, Ericsson

R4-1812747, “TP to TS 38.141-2: Section 4.9.3.2.2 NR test model 2”, Ericsson

R4-1812748, “Synchronization considerations for Test Model Design”, Ericsson

R4-1813171, “TP to TS38.141-2: Test model (4.9) “, NEC

R4-1813277, “TP for FR1 base conformation test models”, Huawei, HiSilicon

R4-1813278, “TP for FR2 base conformation test models”, Huawei, HiSilicon

R4-1813279, “TDD configurations for FR2”, Huawei, HiSilicon

R4-1813280, “SS block in test models”, Huawei, HiSilicon

R4-1813281, “Boosting patterns in test models”, Huawei, HiSilicon

R4-1813282, “TDD configurations for test models used in MSR tests”

R4-1813283, “Reference Symbol for Test Model”, Huawei, HiSilicon

R4-1813536, “TP to TS 38.141-1: Clause 4.9.3 Data content of Physical channels and Signals for NR FR1 Test models”, Nokia, Nokia Shanghai Bell

R4-1813537, “TP to TS 38.141-2: Clause 4.8.3 Data content of Physical channels and Signals for NR FR2 Test models”, Nokia, Nokia Shanghai Bell

R4-1813539, “TP to TS 38.141-1: Clause 4.9.2 Test models”, Nokia, Nokia Shanghai Bell

R4-1813540, “TP to TS 38.141-2: Clause 4.9.3 Test models”, Nokia, Nokia Shanghai Bell

R4-1813875, “BS conformance testing ad-hoc meeting minutes”, Huawei

| Contribution | Source | Proposal |
| --- | --- | --- |
| R4-1812270 | CATT | use the same number of boosted and de-boosted RBs for NR test model since the RE power dynamic range requirement are the same for LTE and NR. NR test model; [dB]; modulation scheme of boosted PDSCH; modulation scheme of de-boosted PDSCH. NR-TM1.2; 0.4; 3; QPSK; QPSK. NR-TM3.2; 0.6; -3; QPSK; 16QAM. NR-TM3.3; 0.5; -6; 16QAM; QPSK. |
| R4-1812595 | ZTE | use the same power boosting/de-boosting levels for FR1 NR test model as that for E-UTRA test model +3dB power boosting for UEM/ACLR measurement in NR-TM1.2; -3dB power de-boosting for 64QAM EVM measurement PRBs in NR-TM3.3; -6dB power de-boosting for QPSK EVM measurement PRBs in NR-TM3.3. every 2nd PRB are assumed to be power boosted for NR-TM1.2, NR-TM3.2 and NR-TM3.3 |
| R4-1812743 | Ericsson | Introduce power boosting of +3 dB for every 3rd PRB starting from the second PRB in an OFDM symbol and adopt the deboosting level according to Table 3. Typo in formula for deboosting |
| R4-1813281 | Huawei | TM2: P(s)=(s×⌈N_RB/(10×2^μ)⌉+f"mod" 2)"mod" N_RB,s=0,⋯,10×2^μ-1 TM1.2, P(k,s)=(s "mod " N+⌊k/M⌋N+k "mod " M)"mod" N_RB |
| R4-1813536 | Nokia | Provided formula for TM2 based on spacing (NRB/10*2µ) |

### Boosting values

Background

Boosting values and percentage / fraction summarized from contributions

|  | NR-FR1 TM1.2 | NR-FR1 TM3.2 | NR-FR1 TM3.3 |
| --- | --- | --- | --- |
| CATT | 0.4; 3dB | 0.6; -3 dB | 0.5, -6 dB |
| ZTE | 0.5; 3dB | 0.5; -3dB | 0.5; -6dB |
| Ericsson | 0.33, 3dB | None | None |
| Huawei | 0.4; 3dB | 0.6; -3 dB | 0.5, -6 dB |
| Nokia | ~50%, 3 dB | ~50%, -3 dB | ~50%, -6dB |

Background

This was from LTE

|  | Min | Modulation | Max | Modulation |
| --- | --- | --- | --- | --- |
| TM 1.2 | -4.7 dB | QPSK (measured) | +3 dB | QPSK (measured) |
| TM 3.2 | -3 dB | 16QAM (measured) | +2.4 dB | QPSK (not measured) |
| TM 3.3 | -6 dB | QPSK (measured) | +2.4 dB | 16QAM (not measured) |

QPSK range: -6 to +3 dB; 16QAM range: -3 to 3 dB: from Ericsson paper stating 36.104 requirements

Summary

- 40% + 3dB PDSCH boost (TM1.2), 60% + -3dB boost 16QAM (TM3.2), 50% + -6dB QPSK (TM3.3) ; [CATT, HW]

  - proposal matches LTE

- 50% + 3dB PDSCH boost (TM1.2), 50% + -3dB boost 16QAM (TM3.2), 50% + -6dB QPSK (TM3.3) ; [ZTE]

  - The boosting pattern for TM1.2 should be re-examined – it may not be feasible

- ~33% + 3dB PDSCH boost (TM1.2), No tests for TM3.2 and TM3.3 ; [Ericsson]

- Nokia: NRBvPP where P is “Nominal RBG size P configuration 1 as in table 5.1.2.2.1-1 in TS 38.214” The values range from 36% to 50%

  - v=2.5 for TM1.2 and 2.5 for TM3.2

Way forward

- TM1.2

  - 40% + 3dB PDSCH:

    - note QPSK deboost range is -3.65 to -4.73

  - 33% + 3dB PDSCH:

    - note QPSK deboost range is -2.03 to -2.92

  - other

- TM3.2:

  - 60% + -3dB boost 16QAM:

    - note QPSK boost range = 2.04 to 2.43

  - Working assumption: 50% + -3dB boost 16QAM:

    - note QPSK boost range = 1.51 – 1.76 dB

  - other

- TM3.3

  - 50% + -6dB QPSK

  - other

Agreement:

The following boosted PRB percentages and levels are from E-UTRA and as guidance used as starting percentages of boosted PRBs

- TM1.2

  - 40% / 3dB boost PDSCH:

- TM3.2:

  - 60% / -3dB boost 16QAM:

- TM3.3

  - 50% / -6dB boost QPSK

Proposed Agreement

- Use RBG mapping for boosted RB

|  | TM1.2 | TM3.2 | TM3.3 |
| --- | --- | --- | --- |
| Boost / deboost (dB) | 3 dB | -3 dB | -6 dB |
| % boost | 40% | 60% | 50% |
| Min/max deboost / boost (dB) | -2.4 / -4.7 dB | 1.8 / 2.4 dB | 1.5 / 2.4 |
| Min/max percent boost (dB) | 30 / 40% | 50.6 / 60% | 36 / 50% |

Proposed Procedure

Determining number of RBG for boosting/deboosting based upon dynamic range constraint and boosting %

RBG size: P: use Table 5.1.2.2.1-1 from 38.214, configuration 1 column to determine P from NRB (sets of P consecutive RBs will be boosted)

Number available RBG: NRBP

Number boosted RBG: xNRBP

Number deboosted RBs: NRB-PxNRBP

Deboost value: 10×log10NRB-PxNRBPNRB-10y/10PxNRBP

### Pattern for TM1.2, TM3.2, TM3.3

Summary

- RB location as a function of slot number

  - Keep same location for all slots [Ericsson, ZTE]

  - Vary location each slot [Nokia, Huawei]

- RB locations given by function p(.)

  - pk=xk+s, s∈0,⋯,x , k=0,…,NRB-sx-1 (Ericsson, ZTE)

  - pMk+i=Nk+i, i=0,…,M-1 , k=0,…,NRB/M-1, MN=f , f is the fraction boosted (Huawei)

    - If M=1, equation simplifies to one above where N=x,

  - pn,s=pn-1,s mod 10+1modNRBG_group-2, s=0,…,10×2u-1 , n=0…,5 initialization p0,s=0; s=0,…,9   (Nokia)

Proposed agreement

Same location at each slot, following table indicates locations of boosted RBG

RBG location

0

NRBP-1

2n, n=1,…,xNRBP-2

### Pattern for TM2

Summary

- 1 RB location changes as a function of slot number

  - Slope is m= NRB10*2μ (Huawei, Nokia)

    - ps=s×m+fmod2mod NRB, s=0,⋯,10×2μ-1, f is frame number (Huawei)

    - ps=s×mmod NRB-1,   s=2,⋯,10×2μ-1,  p0=0 p1=NRB-1 (Nokia)

Way Forward

- Agree on slope

- Agree on formula or combine

- capture the following for locations

| Slot | RB location |
| --- | --- |
| 3n | 0 |
| 3n+1 | NRB2 |
| 3n+2 | NRB-1 |

- Power level for control region for TM2 : goal to reduce PAPR spikes and to assist synchronization because only 1 RB is tested in TM2

Put in square brackets in the TP: reduce power of control region by 6 dB

Companies to investigate PAPR of TM2 with reduced power