3GPP TSG- RAN WG4 Meeting #88bis 	R4-1813882

Chengdu, China, 8-12 Aug 2018

Source: 	Huawei

Title: 	TP for test model for FR1

Agenda Item:	7.9.2.3

Document for:	Approval

# Introduction

From the AH meeting held Monday evening mainly,

- Huawei to lead a WF on power boosting.

# Discussion

# References

R4-1812270, “Discussion on NR Test model”, CATT

R4-1812579, “Discussion on TDD configurations for NR test model”, ZTE

R4-1812581, “TDD configuration for NR BS test model for FR2”, NTT DoCoMo

R4-1812595, “Further discussion on the remaining issues for NR test model”, ZTE

R4-1812601, “TP to TS 38.141-1: FR1 test model(4.9.2)”, ZTE

R4-1812603, “TP to TS 38.141-2: FR2 test model(Section 4.9.3)”, ZTE

R4-1812743, “Power boosting/deboosting configuration for NR Test Models”, Ericsson

R4-1812744, “TP to TS 38.141-1: Section 4.9.2.3 Data content of PHY channels”, Ericsson

R4-1812745, “TP to TS 38.141-2: Section 4.9.2.3 Data content of PHY channels”, Ericsson

R4-1812746, “TP to TS 38.141-1: Section 4.9.3.2.2 NR test model 1”, Ericsson

R4-1812747, “TP to TS 38.141-2: Section 4.9.3.2.2 NR test model 2”, Ericsson

R4-1812748, “Synchronization considerations for Test Model Design”, Ericsson

R4-1813171, “TP to TS38.141-2: Test model (4.9) “, NEC

R4-1813277, “TP for FR1 base conformation test models”, Huawei, HiSilicon

R4-1813278, “TP for FR2 base conformation test models”, Huawei, HiSilicon

R4-1813279, “TDD configurations for FR2”, Huawei, HiSilicon

R4-1813280, “SS block in test models”, Huawei, HiSilicon

R4-1813281, “Boosting patterns in test models”, Huawei, HiSilicon

R4-1813282, “TDD configurations for test models used in MSR tests”

R4-1813283, “Reference Symbol for Test Model”, Huawei, HiSilicon

R4-1813536, “TP to TS 38.141-1: Clause 4.9.3 Data content of Physical channels and Signals for NR FR1 Test models”, Nokia, Nokia Shanghai Bell

R4-1813537, “TP to TS 38.141-2: Clause 4.8.3 Data content of Physical channels and Signals for NR FR2 Test models”, Nokia, Nokia Shanghai Bell

R4-1813539, “TP to TS 38.141-1: Clause 4.9.2 Test models”, Nokia, Nokia Shanghai Bell

R4-1813540, “TP to TS 38.141-2: Clause 4.9.3 Test models”, Nokia, Nokia Shanghai Bell

R4-1813875, “BS conformance testing ad-hoc meeting minutes”, Huawei

# Appendix TP

********************* BEGIN ******************

### 4.9.2	Test models

#### 4.9.2.1	General

The following sections describe the NR FR1 test models needed for BS type 1-C and BS type 1-H.

#### 4.9.2.2 	NR FR1 test models

The set-up of physical channels for transmitter tests shall be according to one of the NR FR1 test models (NR-FR1-TM) below. A reference to the applicable test model is made within each test.

The following general parameters are used by all NR test models:

-	Duration is 1 radio frame (10 ms) for FDD and 2 radio frames for TDD (20 ms)

-	The slots are numbered 0 to 102µ – 1 where µ is the numerology corresponding to the subcarrier space

-	NRB is the maximum transmission bandwidth configuration seen in table 5.3.2-2 in TS 38.104 [2].

-	Normal CP

-	Virtual resource blocks of localized type

For NR FR1-TDD, test models are derived based on the uplink/downlink configuration as showing in the table 4.9.2.2-1 using information element TDD-UL-DL-ConfigCommon.

[If the number of downlink symbols in a special slot is less than 7 symbols, no measurements will be performed on that special slot].

Table 4.9.2.2-1: Configurations of TDD for BS type 1-C and BS type 1-H test models

| Field name | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| --- | --- | --- | --- |
| referenceSubcarrierSpacing | 15 | 30 | 60 |
| Periodicity (ms) for dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
| nrofDownlinkSlots | 3 | 7 | 14 |
| nrofDownlinkSymbols | 10 | 6 | 12 |
| nrofUplinkSlots | 1 | 2 | 4 |
| nrofUplinkSymbols | 2 | 4 | 8 |

Common physical channel parameters for all NR FR1 test models are specified in the following tables: table 4.9.2.2-2 for PDCCH, table 4.9.2.2-3 for PDSCH. Specific physical channel parameters for NR FR1 test models are described in sections 4.9.2.2.1 to 4.9.2.2.8.

Table 4.9.2.2-2: Common physical channel parameters for PDCCH

| Parameter |  |
| --- | --- |
| # of symbols used for control channel | 2 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level(s) | 1 |
| # of RBs not allocated by PDCCH in the each symbol | NRB – 6 |
| DM-RS configuration and density | Comb structure with same frequency density of ¼ (i.e., every 4th subcarrier) on all REGs |
| ratio of PDCCH EPRE to DM-RS EPRS | 0 dB |
| Boosting level of control region | [0 dB] |

Table 4.9.2.2-3: Common physical channel parameters for PDSCH

| Parameter |  |
| --- | --- |
| DM-RS configuration and density | Type 1, Comb2 (every other subcarrier) in symbols 3 and 11 |
| ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

#### 4.9.2.2.1	NR FR1 test model 1.1 (NR-FR1-TM1.1)

This model shall be used for tests on:

-	BS output power

-	Unwanted emissions

-	Occupied bandwidth

-	ACLR

-	Operating band unwanted emissions

-	Transmitter spurious emissions

-	Transmitter intermodulation

Table 4.9.2.2.1-1: Specific physical channel parameters of NR-FR1-TM1.1

| Parameter |  |
| --- | --- |
| # of QPSK PDSCH PRBs | NRB |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |

#### 4.9.2.2.2	NR FR1 test model 1.2 (NR-FR1-TM1.2)

This model shall be used for tests on:

-	Unwanted emissions

-	ACLR

-	Operating band unwanted emissions

Table 4.9.2.2.2-1: Physical channel parameters of NR-FR1-TM1.2

| Parameter |  |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted | x=40% |
| # of QPSK PDSCH RBGs which are boosted | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
|  | NRBx, i.e., every xth PRB] |
| Level of boosting (dB) | 3 |
| Location of PDSCH RBGs which are boosted | 0, NRBP-1, 2n, n=1,…,xNRBP-2 |
| # of QPSK PDSCH PRBs which are deboosted | NRB-PxNRBP NRB-NRBx |
| Level of deboosting (dB) | 10log10NRB-PxNRBPNRB-103/10PxNRBP |
|  | 10*log10NRB-NRBxNRB-y*NRBx |

#### 4.9.2.2.3	NR FR1 test model 2 (NR-FR1-TM2)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 64QAM PRB allocation (at min power)

-	Frequency error (at min power)

Table 4.9.2.2.3-1: Specific physical channel parameters of NR-FR1-TM2

| Parameter |  |
| --- | --- |
| # of 64QAM PDSCH PRBs | 1 |
| Level of boosting (dB) | 0 |
| Location of 64QAM PRB | Slot; RB; n. 3n; 0; n=0,…,10×2μ3-1. 3n+1; NRB2; n=0,…,10×2μ-13-1. 3n+2; NRB-1-1; n=0,…,10×2μ-23-1. |
| # of PDSCH PRBs which are not allocated | NRB-1 |
| # of PDSCH PRBs which are not allocated in symbols allocated for channel | NRB-6 |

#### 4.9.2.2.4	NR FR1 test model 2a (NR-FR1-TM2a)

This model shall be used for tests on:

-	Total power dynamic range (lower OFDM symbol power limit at min power),

-	EVM of single 256QAM PRB allocation (at min power)

-	Frequency error (at min power)

Physical channel parameters and numbers of the allocated PRB are defined in table 4.9.2.2.3-1 with all 64QAM PDSCH PRBs replaced by 256QAM PDSCH PRBs.

#### 4.9.2.2.5	NR FR1-test model 3.1 (NR-FR1-TM3.1)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 64QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 64QAM modulation (at max power)

Physical channel parameters are defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 64QAM PDSCH PRBs.

#### 4.9.2.2.6	NR FR1 test model 3.1a (NR-FR1-TM3.1a)

This model shall be used for tests on:

-	Output power dynamics

-	Total power dynamic range (upper OFDM symbol power limit at max power with all 256QAM PRBs allocated)

-	Transmitted signal quality

-	Frequency error

-	EVM for 256QAM modulation (at max power)

Physical channel parameters are defined in table 4.9.2.2.1-1 with all QPSK PDSCH PRBs replaced by 256QAM PDSCH PRBs.

#### 4.9.2.2.7	NR FR1 test model 3.2 (NR-FR1-TM3.2)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for 16QAM modulation

Table 4.9.2.2.7-1: Specific physical channel parameters of NR-FR1-TM3.2

| Parameter |  |
| --- | --- |
| Percent of 16QAM PDSCH PRBs boosted | x=60% |
| # of 16QAM PDSCH PRBs within a slot for which EVM is measured | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | -3 |
| Location of 16QAM PRBs which are boosted | 0, NRBP-1, 2n, n=1,…,xNRBP-2 |
| # of QPSK PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB-PxNRBP |
| Level of boosting (dB) | 10log10NRB-PxNRBPNRB-10-3/10PxNRBP |

#### 4.9.2.2.8	NR FR1 test model 3.3 (NR-FR1-TM3.3)

This model shall be used for tests on:

-	Transmitted signal quality

-	Frequency error

-	EVM for QPSK modulation

Table 4.9.2.2.8-1: Specific physical channel parameters of NR-FR1-TM3.3

| Parameter |  |
| --- | --- |
| Percent of QPSK PDSCH PRBs boosted | x=50% |
| # of QPSK PDSCH RBGs within a slot for which EVM is measured | xNRBP, where P is determined from Table 5.1.2.2.1-1 from 38.214[x], configuration 1 column using NRB |
| Level of boosting (dB) | -6 |
| Location of QPSK RBGs which are boosted | 0, NRBP-1, 2n, n=1,…,xNRBP-2 |
| # of 16QAM PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB-PxNRBP |
| Level of boosting (dB) | 10log10NRB-PxNRBPNRB-10-6/10PxNRBP |

******************** END *********************