3GPP TSG-RAN WG4 Meeting #88-Bis	R4-1814016

# Chengdu, China, 08 - 12 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 5228 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on introduction of 6CCs and 7CCs FDD-TDD CA demodulation performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15_xDL1UL-Perf |  |  |  |  |  |  | Date: |  | 2018-09-25 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | CA demodulation performance requirements for 6DL/7DL FDD-TDD CA need be introudecd in TS 36.101. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CA demodulation and CSI requirements are updated to include 6CCs and 7CCs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No demodulation and CSI requirements are available for supporting the 6CCs and 7CCs FDD-TDD combinations in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3, 9.6.1.3, 9.6.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36-521-3. CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 8.2.3	TDD FDD CA (Fixed Reference Channel)

The parameters specified in Table 8.2.3-1 are valid for all the TDD FDD CA tests unless otherwise stated.

Table 8.2.3-1: Common Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Uplink downlink configuration (Note 1) for TDD CC only |  |  | 1 |
| Special subframe configuration (Note 2) for TDD CC only |  |  | 4 |
| Inter-TTI Distance |  |  | 1 |
| Maximum number of HARQ processes per component carrier | FDD PCell | Processes | 8 for FDD and TDD CCs |
|  | TDD PCell | Processes | 11 for FDD CC; 7 for TDD CC |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} for QPSK and 16QAM {0,0,1,2} for 64QAM |
| Number of OFDM symbols for PDCCH per component carrier |  | OFDM symbols | 4 for 1.4 MHz bandwidth, 3 for 3 MHz and 5 MHz bandwidths, 2 for 10 MHz, 15 MHz and 20 MHz bandwidths |
| Cyclic Prefix |  |  | Normal |
| Cell_ID |  |  | 0 |
| Cross carrier scheduling |  |  | Not configured |
| ACK/NACK feedback mode |  |  | PUCCH format 3 |
| Downlink HARQ-ACK timing | FDD PCell |  | As specified in Clause 7.3.3 in TS36.213 [6] |
|  | TDD PCell |  | As specified in Clause 7.3.4 in TS36.213 [6] |
| Note 1: as specified in Table 4.2-2 in TS 36.211 [4]. Note 2: as specified in Table 4.2-1 in TS 36.211 [4]. |  |  |  |

The applicability of ther requirements are specified in Clause 8.1.2.3. The single carrier performance with different bandwidths for multiple CA configurations specified in Clause 8.2.3 cannot be applied for UE single carrier test.

#### 8.2.3.1	Single-antenna port performance

The single-antenna performance in a given multi-path fading environments is determined by the SNR for which a certain relative information bit throughput of the reference measurement channels in Annex A.3.3 is achieved. The purpose of these tests is to verify the single-antenna performance with different channel models and MCS.

##### 8.2.3.1.1	Minimum Requirement for FDD PCell

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.1.1-4 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.1.1-5 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.1.1-6 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.1.1-7 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.1.1-8 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with FDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.1.1-9 based on single carrier requirement specified in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3, with the addition of the parameters in Table 8.2.3.1.1-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.1.1-1: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | QPSK |
| PDSCH transmission mode |  |  | 1 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.1.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.4 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.3 |
| 3 MHz | R.42-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.1 |
| 5MHz | R.42-2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.0 |
| 10MHz | R.2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |
| 15MHz | R.42-3 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.6 |
| 20MHz | R.42 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |

Table 8.2.3.1.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width |  | Reference Channel |  | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration |  | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) |  | SNR (dB) |  |
| 1.4 MHz |  | R.4 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -0.6 |  |
| 3 MHz |  | R.42-1 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -0.8 |  |
| 5MHz |  | R.42-2 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -1.2 |  |
| 10MHz |  | R.2 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -1.6 |  |
| 15MHz |  | R.42-3 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -1.4 |  |
| 20MHz |  | R.42 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -1.4 |  |

Table 8.2.3.1.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any FDD CC. |  |  |  |  |  |

Table 8.2.3.1.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any FDD CC. |  |  |  |  |  |

Table 8.2.3.1.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | ≥8 |  |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |  |

Table 8.2.3.1.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.1-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.1.1-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

##### 8.2.3.1.2	Minimum Requirement for TDD PCell

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.1.2-4 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 3DL CCs, the requirements are specified in Table 8.2.3.1.2-5 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 4DL CCs, the requirements are specified in Table 8.2.3.1.2-6 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 5DL CCs, the requirements are specified in Table 8.2.3.1.2-7 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 6DL CCs, the requirements are specified in Table 8.2.3.1.2-8 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

For TDD FDD CA with TDD PCell with 7DL CCs, the requirements are specified in Table 8.2.3.1.2-9 based on single carrier requirement specified in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3, with the addition of the parameters in Table 8.2.3.1.2-1 and the downlink physical channel setup according to Annex C.3.2.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.1.2-1: Test Parameters for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) |
| Modulation |  |  | QPSK |
| PDSCH transmission mode |  |  | 1 |
| Note 1: . Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. Note 3: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.1.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.4 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.3 |
| 3 MHz | R.42-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.1 |
| 5MHz | R.42-2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.0 |
| 10MHz | R.2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |
| 15MHz | R.42-3 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.6 |
| 20MHz | R.42 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.7 |

Table 8.2.3.1.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width |  | Reference Channel |  | OCNG Pattern |  | Propagation Condition |  | Correlation Matrix and Antenna Configuration |  | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) |  | SNR (dB) |  |
| 1.4 MHz |  | R.4 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -0.6 |  |
| 3 MHz |  | R.42-1 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -0.8 |  |
| 5MHz |  | R.42-2 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -1.2 |  |
| 10MHz |  | R.2 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -1.6 |  |
| 15MHz |  | R.42-3 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -1.4 |  |
| 20MHz |  | R.42 TDD |  | OP.1 TDD |  | EVA5 |  | 1x2 Low |  | 70 |  | -1.4 |  |

Table 8.2.3.1.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any TDD CC. |  |  |  |  |  |

Table 8.2.3.1.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any TDD CC. |  |  |  |  |  |

Table 8.2.3.1.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | ≥8 |  |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |  |

Table 8.2.3.1.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. Note 2: 30usec timing difference between PCell and any SCell is applied in inter-band CA case, where PCell can be assigned on any CC. |  |  |  |  |  |

Table 8.2.3.1.2-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.1.2-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

#### 8.2.3.2	Open-loop spatial multiplexing performance 2Tx Antenna port

##### 8.2.3.2.1	Minimum Requirement for FDD PCell

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.2.1-4 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.2.1-5 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.2.1-6 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.2.1-7 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.2.1-8 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with FDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.2.1-9 based on single carrier requirement specified in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3, with the addition of the parameters in Table 8.2.3.2.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.2.1-1: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| Note 1: . Note 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.2.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.11-5 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 13.6 |
| 3 MHz | R.11-6 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 5MHz | R.11-2 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 10MHz | R.11 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 15MHz | R.11-7 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 20MHz | R.30 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |

Table 8.2.3.2.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.11-5 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.2 |
| 3 MHz | R.11-6 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 5MHz | R.11-7 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.6 |
| 10MHz | R.11-8 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 15MHz | R.11-9 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 20MHz | R.30-1 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.0 |

Table 8.2.3.2.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.2.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.1.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.1.2-2 and Table 8.2.3.1.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.1-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |

Table 8.2.3.2.1-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.2.1-2 and Table 8.2.3.2.1-3 per CC | 8, ≥11 |

##### 8.2.3.2.1A	Soft buffer management test for FDD PCell

For TDD-FDD CA, the requirements are specified in Table 8.2.3.2.1A-2, with the addition of the parameters in Table 8.2.3.2.1A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the UE performance with proper instantaneous buffer implementation for FDD as PCell.

Table 8.2.3.2.1A-1: Test Parameters for CA

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD Carrier | TDD Carrier |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| PDSCH transmission mode |  |  | 3 | 3 |
| Note 1: . Note 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |  |

Table 8.2.3.2.1A-2: Minimum performance (FRC) for CA

| Test num. | Band-width |  | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate gory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1 | PCell | 20MHz | R.30 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 13.2 | 3 |
|  | SCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) |  |  | 70 | 13.2 |  |
| 2 | PCell | 20MHz | R.35-1 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.3 | 4 |
|  | SCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) |  |  | 70 | 16.3 |  |
| 3 | PCell | 10MHz | R.35-3 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.0 | 3 |
|  | SCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) |  |  | 70 | 13.2 |  |
| 4 | PCell | 10MHz | R.35-3 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.0 | 4 |
|  | SCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) |  |  | 70 | 16.3 |  |
| 5 | PCell | 15MHz | R.35-2 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.0 | 3 |
|  | SCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) |  |  | 70 | 13.2 |  |
| 6 | PCell | 15MHz | R.35-2 FDD | OP.1 FDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.0 | 4 |
|  | SCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) |  |  | 70 | 16.3 |  |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability and test rules of requirements for different CA configurations and bandwidth combination sets are defined in 8.1.2.3B. |  |  |  |  |  |  |  |  |  |

##### 8.2.3.2.2	Minimum Requirement for TDD PCell

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.2.2-4 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.2.2-5 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.2.2-6 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.2.2-7 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.2.2-8 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

For TDD FDD CA with TDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.2.2-9 based on single carrier requirement specified in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3, with the addition of the parameters in Table 8.2.3.2.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the performance of large delay CDD with 2 transmitter antennas.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.2.2-1: Test Parameters for Large Delay CDD (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| Note 1: . Note 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.2.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.11-5 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 13.6 |
| 3 MHz | R.11-6 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 5MHz | R.11-2 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.3 |
| 10MHz | R.11 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 15MHz | R.11-7 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 20MHz | R.30 FDD | OP.1 FDD | EVA70 | 2x2 Low | 70 | 12.9 |

Table 8.2.3.2.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.11-5 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.2 |
| 3 MHz | R.11-6 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 5MHz | R.11-7 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.6 |
| 10MHz | R.11-8 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.8 |
| 15MHz | R.11-9 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 12.9 |
| 20MHz | R.30-1 TDD | OP.1 TDD | EVA70 | 2x2 Low | 70 | 13.0 |

Table 8.2.3.2.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.2.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.2.2-2 and Table 8.2.3.2.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.2.2-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.2.2-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

##### 8.2.3.2.2A	Soft buffer management test for TDD PCell

For TDD-FDD CA, the requirements are specified in Table 8.2.3.2.2A-2, with the addition of the parameters in Table 8.2.3.2.2A-1 and the downlink physical channel setup according to Annex C.3.2. The purpose is to verify the UE performance with proper instantaneous buffer implementation for TDD as PCell.

Table 8.2.3.2.2A-1: Test Parameters for CA

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | FDD Carrier | TDD Carrier |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 (Note 1) |
|  |  | dB | 0 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| PDSCH transmission mode |  |  | 3 | 3 |
| Note 1: . Note 2: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |  |

Table 8.2.3.2.2A-2: Minimum performance (FRC) for CA

| Test num. | Band-width |  | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate gory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1 | PCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1)) | EVA70 | 2x2 Low | 70 | 13.2 | 3 |
|  | SCell | 20MHz | R.30 FDD | OP.1 FDD (Note 1 |  |  | 70 | 13.2 |  |
| 2 | PCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.2 | 4 |
|  | SCell | 20MHz | R.35-1 FDD | OP.1 FDD (Note 1) |  |  | 70 | 16.2 |  |
| 3 | PCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 13.2 | 3 |
|  | SCell | 10MHz | R.35-3 FDD | OP.1 FDD (Note 1) |  |  | 70 | 16.0 |  |
| 4 | PCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.2 | 4 |
|  | SCell | 10MHz | R.35-3 FDD | OP.1 FDD (Note 1) |  |  | 70 | 15.8 |  |
| 5 | PCell | 20MHz | R.30-2 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 13.2 | 3 |
|  | SCell | 15MHz | R.35-2 FDD | OP.1 FDD (Note 1) |  |  | 70 | 15.8 |  |
| 6 | PCell | 20MHz | R.35-1 TDD | OP.1 TDD (Note 1) | EVA70 | 2x2 Low | 70 | 16.2 | 4 |
|  | SCell | 15MHz | R.35-2 FDD | OP.1 FDD (Note 1) |  |  | 70 | 15.8 |  |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability and test rules of requirements for different CA configurations and bandwidth combination sets are defined in 8.1.2.3B. |  |  |  |  |  |  |  |  |  |

#### 8.2.3.3		Closed-loop spatial multiplexing performance 4Tx Antenna Port

##### 8.2.3.3.1	Minimum Requirement for FDD PCell

For TDD FDD CA with FDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.3.1-4 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.3.1-5 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.3.1-6 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.3.1-7 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.3.1-8 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with FDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.3.1-9 based on single carrier requirement specified in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3, with the addition of the parameters in Table 8.2.3.3.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.3.1-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Wideband precoding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | 4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.3.1-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.4 |
| 3 MHz | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10MHz | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.3.3.1-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3 MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.3.3.1-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.3.1-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.3.1-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 4x20 | 20 | 3x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 2 | 4x20 | 2×20 | 2×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 3 | 3x20+15 | 20+15 | 2×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 4 | 2×15+2x20 | 2×15 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 5 | 3x20+15 | 2×20+15 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 6 | 2×15+2x20 | 2x15+20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 7 | 3x20+10 | 2x20+10 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 8 | 4x20 | 3x20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 9 | 10+3x20 | 10 | 3x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 10 | 2x10+2x20 | 2x10 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 11 | 2x10+20+15 | 2x10 | 20+15 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 12 | 10+3x20 | 10+20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 13 | 10+15+2x20 | 10+15 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| 14 | 10+15+2x20 | 10+15+20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.3.1-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.3.1-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |

Table 8.2.3.3.1-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.3.1-2 and Table 8.2.3.3.1-3 per CC | 8, ≥11 |

##### 8.2.3.3.2	Minimum Requirement for TDD PCell

For TDD FDD CA with TDD PCell and 2DL CCs, the requirements are specified in Table 8.2.3.3.2-4 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 3DL CCs, the requirements are specified in Table 8.2.3.3.2-5 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 4DL CCs, the requirements are specified in Table 8.2.3.3.2-6 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 5DL CCs, the requirements are specified in Table 8.2.3.3.2-7 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 6DL CCs, the requirements are specified in Table 8.2.3.3.2-8 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

For TDD FDD CA with TDD PCell and 7DL CCs, the requirements are specified in Table 8.2.3.3.2-9 based on single carrier requirement specified in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3, with the addition of the parameters in Table 8.2.3.3.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding.

The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.2.3.3.2-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | 3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | Widelband pre-coding for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, 8 for 15MHz and 20MHz CCs |
| PMI delay (Note 2) | FDD CC | ms | 8 |
|  | TDD CC | ms | 10 or 11 |
| Reporting interval | FDD CC | ms | 1 |
|  | TDD CC | ms | 1 or 4 (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| CSI request field (Note 3) |  |  | ‘10’ |
| PDSCH transmission mode |  |  | TM4 |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: Multiple CC-s under test are configured as the 1st set of serving cells by higher layers. Note 4: ACK/NACK bits are transmitted using PUSCH with PUCCH format 3. Note 5: The same PDSCH transmission mode is applied to each component carrier. |  |  |  |

Table 8.2.3.3.2-2: Single carrier performance with different bandwidths for multiple CA configurations for FDD SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.14-4 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.4 |
| 3 MHz | R.14-5 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10MHz | R.14 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP.1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.3.3.2-3: Single carrier performance with different bandwidths for multiple CA configurations for TDD PCell and SCell (FRC)

| Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1.4 MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3 MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.3.3.2-4: Minimum performance for multiple CA configurations with 2DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 2x20 | 20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 2 | 20+10 | 10 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 3 | 20+15 | 15 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B |  |  |  |  |  |

Table 8.2.3.3.2-5: Minimum performance for multiple CA configurations with 3DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 3x20 | 20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 2 | 20+20+15 | 15 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 3 | 20+20+10 | 10 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 4 | 3x20 | 2x20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 5 | 20+20+15 | 20+15 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 6 | 20+20+10 | 20+10 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 7 | 20+10+10 | 2x10 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 8 | 10+15+20 | 10 | 15+20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| 9 | 10+15+20 | 10+15 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | ≥5 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.3.2-6: Minimum performance for multiple CA configurations with 4DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  |  | Minimum performance requirement |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC |  | TDD CC |  |  |  |
| 1 | 4x20 |  | 20 | 3x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 2 | 4x20 |  | 2×20 | 2×20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 3 | 3x20+15 |  | 20+15 | 2×20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 4 | 2×15+2x20 |  | 2×15 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 5 | 3x20+15 |  | 2×20+15 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 6 | 2×15+2x20 |  | 2x15+20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 7 | 3x20+10 |  | 2x20+10 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 8 | 4x20 |  | 3x20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 9 | 10+3x20 |  | 10 | 3x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 10 | 2x10+2x20 |  | 2x10 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 11 | 2x10+20+15 |  | 2x10 | 20+15 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 12 | 10+3x20 |  | 10+20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 13 | 10+15+2x20 |  | 10+15 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| 14 | 10+15+2x20 |  | 10+15+20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC |  | ≥8 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |  |  |

Table 8.2.3.3.2-7: Minimum performance for multiple CA configurations with 5DL CCs (FRC)

| Test number | Aggregated Bandwidth (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 15+4×20 | 15+2×20 | 2×20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 2 | 2×15+3×20 | 2×15+20 | 2×20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 3 | 4x20+20 | 4x20 | 20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 4 | 3x20+2x20 | 3x20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 5 | 2x20+3x20 | 2x20 | 3X20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 6 | 20+4x20 | 20 | 4x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 7 | 10+4x20 | 10 | 4x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 8 | 10+20+3x20 | 10+20 | 3x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 9 | 2x10+3x20 | 2x10 | 3x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 10 | 10+2×20+2×20 | 10+2x20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| 11 | 10+15+20+2x20 | 10+15+20 | 2x20 | As defined in Table 8.2.3.3.2-2 and Table 8.2.3.3.2-3 per CC | 8, ≥11 |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 8.1.2.3B. |  |  |  |  |  |

Table 8.2.3.3.2-8: Minimum performance for multiple CA configurations with 6DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+5×20 | 1×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+4×20 | 2×20 | 4×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

Table 8.2.3.3.2-9: Minimum performance for multiple CA configurations with 7DL CCs (FRC)

| Test number | CA Bandwidth combination (MHz) |  |  | Minimum performance requirement | UE Category |
| --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |
| 1 | 1×20+6×20 | 1×20 | 6×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |
| 2 | 2×20+5×20 | 2×20 | 5×20 | As defined in Table 8.2.3.1.1-2 and Table 8.2.3.1.1-3 per CC | 8, ≥11 |

#### 8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity

For dual connectivity the requirements are specified in Table 8.2.3.4-4, based on single carrier requirement specified in Table 8.2.3.4-2 and Table 8.2.3.4-3, with the addition of the parameters in Table 8.2.3.4-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests is to verify the closed loop rank-two performance with wideband and frequency selective precoding by using dual connectivity transmission.

Table 8.2.3.4-1: Test Parameters for Multi-Layer Spatial Multiplexing (FRC) for TDD-FDD dual connectivity

| Parameter |  | Unit | Values |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -6 |
|  |  | dB | -6 (Note 1) |
|  |  | dB | -3 |
| at antenna port |  | dBm/15kHz | -98 |
| Precoding granularity |  | PRB | 6 for 1.4MHz, 4 for 3MHz and 5MHz CCs, 6 for 10MHz CCs, and 8 for 15MHz CCs and 20MHz CCs |
| PMI delay (Note 2) |  | ms | 8 for FDD CC 10 or 11 for TDD CC |
| Reporting interval |  | ms | 1 for FDD CC 1 or 4 for TDD CC (Note 3) |
| Reporting mode |  |  | PUSCH 1-2 |
| CodeBookSubsetRestriction bitmap |  |  | 0000000000000000000000000000000011111111111111110000000000000000 |
| PDSCH transmission mode |  |  | 4 |
| ACK/NACK transmission |  |  | Separate ACK/NACK feedbacks with PUCCH format 1b on the MCG and SCG |
| CSI feedback |  |  | Separate PUSCH feedbacks on the MCG and SCG |
| Time offset between MCG CC and SCG CC |  | μs | 0 for UE under test supporting synchronous dual connectivity; 334 for UE under test supporting both asynchronous and synchrounous dual connectivity (Note 5) |
| Note 1: . Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: For Uplink - downlink configuration 1 the reporting interval will alternate between 1ms and 4ms. Note 4: The same PDSCH transmission mode is applied to each component carrier. Note 5: As defined in TS36.300 [11]. Note 6: If the UE supports both SCG bearer and Split bearer, the SCG bearer is configured. |  |  |  |

Table 8.2.3.4-2: FDD single carrier performance for multiple dual connectivity configurations

| Bandwidth | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.14-4 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.36 |
| 3MHz | R.14-5 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 5MHz | R.14-6 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 9.5 |
| 10 MHz | R.14 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 15MHz | R.14-7 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.1 |
| 20MHz | R.14-3 FDD | OP. 1 FDD | EVA5 | 4x2 Low | 70 | 10.3 |

Table 8.2.3.4-3: TDD single carrier performance for multiple dual connectivity configurations

| Bandwidth | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1.4MHz | R.43-1 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 11.0 |
| 3MHz | R.43-2 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 9.8 |
| 5MHz | R.43-3 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.0 |
| 10 MHz | R.43-4 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.5 |
| 15MHz | R.43-5 TDD | OP.1 TDD | EVA5 | 4x2 Low | 70 | 10.6 |
| 20MHz | R.43 TDD | OP. 1 TDD | EVA5 | 4x2 Low | 70 | 10.7 |

Table 8.2.3.4-4: Minimum performance Multi-Layer Spatial Multiplexing (FRC) for dual connectivity

| Test num. | Bandwidth combination | Requirement | UE category |
| --- | --- | --- | --- |
| 1 | 2x20 MHz | As specified in Table 8.2.3.4-2 and Table 8.2.3.4-3 per CC | ≥5 |
| Note 1: The OCNG pattern applies for each CC. Note 2: The applicability of requirements for different dual connectvity configurations and bandwidth combination sets is defined in 8.1.2.3A. |  |  |  |

< End of changes >

# < Unchanged sections omitted >

#### 9.6.1.3	TDD-FDD CA with FDD PCell

The following requirements apply to UE Category ≥5. For TDD-FDD CA with FDD PCell with 2 DL CC, for the parameters specified in Table 9.6.1.3-1 and Table 9.6.1.3-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell reported shall be such that

wideband CQIPCell – wideband CQISCell ≥ 2

for more than 90% of the time.

Table 9.6.1.3-1: Parameters for PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 2 DL CA)

| Parameter |  | Unit | PCell |  | SCell |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | N/A | 2 |  |
| Special subframe configuration |  |  | N/A | 4 |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 10 |  | 4 |
|  |  | dB[mW/15kHz] | -88 |  | -94 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 10 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 9 | 14 (shift of 5 ms relative to Pcell) |  |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.3-2: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 2 DL CA)

| Test number | Bandwidth combination |
| --- | --- |
| 1 | 20MHz for FDD cell and 20MHz for TDD cell |
| 2 | 10MHz for FDD cell and 20MHz for TDD cell |
| 3 | 15MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. |  |

The following requirements for 3DL CA apply to UE Category ≥5. For TDD-FDD CA with FDD PCell with 3 DL CC, for the parameters specified in Table 9.6.1.3-3 and Table 9.6.1.3-4, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

for more than 90% of the time.

The following requirements for 4DL CA apply to UE Cateogry ≥8. For TDD-FDD CA with FDD PCell with 4 DL CC, for the parameters specified in Table 9.6.1.3-3 and Table 9.6.1.3-5, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, and SCell1 and SCell3 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

for more than 90% of the time.

Table 9.6.1.3-3: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 3 and 4 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | N/A | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | N/A | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 20 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 19 | 24 (shift of 5 ms relative to Pcell) | 29 for SCell 2 (shift of 10 ms relative to Pcell) , 34 for SCell 3 (shift of 15ms relative to PCell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.3-4: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 3 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 20MHz for FDD cell and 2x20MHz for TDD cell |
| 2 | 15MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 10MHz for FDD cell and 2x20MHz for TDD cell |
| 4 | 2x20MHz for FDD cell and 20MHz for TDD cell |
| 5 | 20+15MHz for FDD cell and 20MHz for TDD cell |
| 6 | 20+10MHz for FDD cell and 20MHz for TDD cell |
| 7 | 2x10MHz for FDD cell and 20MHz for TDD cell |
| 8 | 10MHz for FDD cell and 15+20MHz for TDD cell |
| 9 | 10+15MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

Table 9.6.1.3-5: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 4 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 20MHz for FDD cell and 3x20MHz for TDD cell |
| 2 | 2x20MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 20+15MHz for FDD cell and 2x20MHz for TDD cell |
| 4 | 2x15MHz for FDD cell and 2x20MHz for TDD cell |
| 5 | 2x20+15MHz for FDD cell and 20MHz for TDD cell |
| 6 | 2x15+20MHz for FDD cell and 20MHz for TDD cell |
| 7 | 2x20+10MHz for FDD cell and 20MHz for TDD cell |
| 8 | 3x20MHz for FDD cell and 20MHz for TDD cell |
| 9 | 10MHz for FDD cell and 3x20MHz for TDD cell |
| 10 | 2x10MHz for FDD cell and 2x20MHz for TDD cell |
| 11 | 2x10MHz for FDD cell and 20+15MHz for TDD cell |
| 12 | 10+20MHz for FDD cell and 2x20MHz for TDD cell |
| 13 | 10+15MHz for FDD cell and 2x20MHz for TDD cell |
| 14 | 10+15+20MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 5DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with FDD PCell with 5 DL CC, for the parameters specified in Table 9.6.1.3-6 and Table 9.6.1.3-7, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, and SCell 1 and SCell 4 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

for more than 90% of the time.

Table 9.6.1.3-6: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 5 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | N/A | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | N/A | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 39 | 44 (shift of 5 ms relative to Pcell) | 49 for SCell 2 (shift of 10 ms relative to Pcell), 54 for SCell 3 (shift of 15 ms relative to Pcell), 59 for SCell 4 (shift of 20 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.3-7: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 5 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 15MHz+2×20MHz for FDD cell and 2x20MHz for TDD cell |
| 2 | 2×15MHz+20MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 4x20(FDD)+20(TDD)MHz |
| 4 | 3x20(FDD)+2x20(TDD)MHz |
| 5 | 2x20(FDD)+3x20(TDD)NA |
| 6 | 20(FDD)+4x20(TDD)MHz |
| 7 | 10(FDD)+4x20(TDD)MHz |
| 8 | 2x10(FDD)+3x20(TDD)MH |
| 9 | 10+20(FDD)+3x20(TDD)MHz |
| 10 | 10+2×20(FDD)+2×20(TDD)MHz |
| 11 | 10+15+20(FDD)+2x20(TDD)MHz |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 6DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with FDD PCell with 6 DL CC, for the parameters specified in Table 9.6.1.3-8 and Table 9.6.1.3-9, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, SCell 1 and SCell 4, and SCell 1 and SCell 5 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

for more than 90% of the time.

Table 9.6.1.3-8: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 6 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4, SCell5 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | N/A | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | N/A | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 39 | 44 (shift of 5 ms relative to Pcell) | 49 for SCell 2 (shift of 10 ms relative to Pcell), 54 for SCell 3 (shift of 15 ms relative to Pcell), 59 for SCell 4 (shift of 20 ms relative to Pcell), 64 for SCell 5 (shift of 25 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.3-9: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 6 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 1×20(FDD)+5×20(TDD) |
| 2 | 2×20(FDD)+4×20(TDD) |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 7DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with FDD PCell with 7 DL CC, for the parameters specified in Table 9.6.1.3-10 and Table 9.6.1.3-11, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, SCell 1 and SCell 4, SCell 1 and SCell 5, SCell 1 and SCell 6 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

wideband CQISCell1 – wideband CQISCell6 ≥ 2

for more than 90% of the time.

Table 9.6.1.3-10: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with FDD PCell, 7 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4, SCell5, SCell6 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | N/A | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | N/A | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 39 | 44 (shift of 5 ms relative to Pcell) | 49 for SCell 2 (shift of 10 ms relative to Pcell), 54 for SCell 3 (shift of 15 ms relative to Pcell), 59 for SCell 4 (shift of 20 ms relative to Pcell), 64 for SCell 5 (shift of 25 ms relative to Pcell), 69 for SCell 6 (shift of 30 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.3-11: PUCCH 1-0 static test (TDD-FDD CA with FDD PCell, 7 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 1×20(FDD)+6×20(TDD) |
| 2 | 2×20(FDD)+5×20(TDD) |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

#### 9.6.1.4	TDD-FDD CA with TDD PCell

The following requirements apply to UE Category ≥5. For TDD-FDD CA with TDD PCell with 2 DL CC, for the parameters specified in Table 9.6.1.4-1 and Table 9.6.1.4-2, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell reported shall be such that

wideband CQIPCell – wideband CQISCell ≥ 2

for more than 90% of the time.

Table 9.6.1.4-1: Parameters for PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 2 DL CA)

| Parameter |  | Unit | PCell |  | SCell |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 | N/A |  |
| Special subframe configuration |  |  | 4 | N/A |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 10 |  | 4 |
|  |  | dB[mW/15kHz] | -88 |  | -94 |
|  |  | dB[mW/15kHz] | -98 |  | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 10 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 8 | 13 (shift of 5 ms relative to Pcell) |  |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.4-2: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 2 DL CA)

| Test number | Bandwidth combination |
| --- | --- |
| 1 | 20MHz for TDD cell and 20MHz for FDD cell |
| 2 | 20MHz for TDD cell and 10MHz for FDD cell |
| 3 | 20MHz for TDD cell and 15MHz for FDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. |  |

The following requirements for 3DL CA apply to UE Category ≥5. For TDD-FDD CA with TDD PCell with 3 DL CC, for the parameters specified in Table 9.6.1.4-3 and Table 9.6.1.4-4, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

for more than 90% of the time.

The following requirements for 4DL CA apply to UE Cateogry ≥8. For TDD-FDD CA with TDD PCell with 4 DL CC, for the parameters specified in Table 9.6.1.4-3 and Table 9.6.1.4-5, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, and SCell1 and SCell3 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

for more than 90% of the time.

Table 9.6.1.4-3: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 3 and 4 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 | 2 for TDD Cell N/A for FDD Cell |  |
| Special subframe configuration |  |  | 4 | 4 for TDD Cell N/A for FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 20 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 18 | 23 (shift of 5 ms relative to Pcell) | 28 for SCell 2 (shift of 10 ms relative to Pcell) , 33 for SCell 3 (shift of 15ms relative to PCell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.4-4: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 3 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 2x20MHz for TDD cell and 20MHz for FDD cell |
| 2 | 2x20MHz for TDD cell and 15MHz for FDD cell |
| 3 | 2x20MHz for TDD cell and 10MHz for FDD cell |
| 4 | 2x20MHz for FDD cell and 20MHz for TDD cell |
| 5 | 20+15MHz for FDD cell and 20MHz for TDD cell |
| 6 | 20+10MHz for FDD cell and 20MHz for TDD cell |
| 7 | 2x10MHz for FDD cell and 20MHz for TDD cell |
| 8 | 10MHz for FDD cell and 15+20MHz for TDD cell |
| 9 | 10+15MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

Table 9.6.1.4-5: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 4 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 3x20MHz for TDD cell and 20MHz for FDD cell |
| 2 | 2x20MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 20+15MHz for FDD cell and 2x20MHz for TDD cell |
| 4 | 2x15MHz for FDD cell and 2x20MHz for TDD cell |
| 5 | 2x20+15MHz for FDD cell and 20MHz for TDD cell |
| 6 | 2x15+20MHz for FDD cell and 20MHz for TDD cell |
| 7 | 2x20+10MHz for FDD cell and 20MHz for TDD cell |
| 8 | 3x20MHz for FDD cell and 20MHz for TDD cell |
| 9 | 10MHz for FDD cell and 3x20MHz for TDD cell |
| 10 | 2x10MHz for FDD cell and 2x20MHz for TDD cell |
| 11 | 2x10MHz for FDD cell and 20+15MHz for TDD cell |
| 12 | 10+20MHz for FDD cell and 2x20MHz for TDD cell |
| 13 | 10+15MHz for FDD cell and 2x20MHz for TDD cell |
| 14 | 10+15+20MHz for FDD cell and 20MHz for TDD cell |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 5DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with TDD PCell with 5 DL CC, for the parameters specified in Table 9.6.1.4-6 and Table 9.6.1.4-7, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3 and SCell 1 and SCell 4 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

for more than 90% of the time.

Table 9.6.1.4-6: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 5 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 | 2 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Special subframe configuration |  |  | 4 | 4 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 for SCell 2 (shift of 10 ms relative to Pcell), 53 for SCell 3 (shift of 15 ms relative to Pcell), 58 for SCell 4 (shift of 20 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.4-7: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 5 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 15MHz+2×20MHz for FDD cell and 2x20MHz for TDD cell |
| 2 | 2×15MHz+20MHz for FDD cell and 2x20MHz for TDD cell |
| 3 | 4x20(FDD)+20(TDD)MHz |
| 4 | 3x20(FDD)+2x20(TDD)MHz |
| 5 | 2x20(FDD)+3x20(TDD)NA |
| 6 | 20(FDD)+4x20(TDD)MHz |
| 7 | 10(FDD)+4x20(TDD)MHz |
| 8 | 2x10(FDD)+3x20(TDD)MH |
| 9 | 10+20(FDD)+3x20(TDD)MHz |
| 10 | 10+2×20(FDD)+2×20(TDD)MHz |
| 11 | 10+15+20(FDD)+2x20(TDD)MHz |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 6DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with TDD PCell with 6 DL CC, for the parameters specified in Table 9.6.1.4-8 and Table 9.6.1.4-9, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, SCell1 and SCell4, and SCell1 and SCell5 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

for more than 90% of the time.

Table 9.6.1.4-8: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 6 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4, SCell5 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 | 2 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Special subframe configuration |  |  | 4 | 4 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 for SCell 2 (shift of 10 ms relative to Pcell), 53 for SCell 3 (shift of 15 ms relative to Pcell), 58 for SCell 4 (shift of 20 ms relative to Pcell), 63 for SCell 5 (shift of 25 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.4-9: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 6 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 1x20(FDD)+ 5x20(TDD)MHz |
| 2 | 2x20(FDD)+4x20(TDD)MHz |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

The following requirements for 7DL CA apply to UE Category 8 and ≥11. For TDD-FDD CA with TDD PCell with 7 DL CC, for the parameters specified in Table 9.6.1.4-10 and Table 9.6.1.4-11, and using the downlink physical channels specified in tables C.3.2-1 and C.3.2-2 on each cell, the difference between the wideband CQI indices of PCell and SCell1 reported, and the difference between the wideband CQI indices of SCell1 and SCell2, SCell1 and SCell3, SCell1 and SCell4, SCell1 and SCell5, and SCell1 and SCell6 reported shall be such that

wideband CQIPCell – wideband CQISCell1 ≥ 2

wideband CQISCell1 – wideband CQISCell2 ≥ 2

wideband CQISCell1 – wideband CQISCell3 ≥ 2

wideband CQISCell1 – wideband CQISCell4 ≥ 2

wideband CQISCell1 – wideband CQISCell5 ≥ 2

wideband CQISCell1 – wideband CQISCell6 ≥ 2

for more than 90% of the time.

Table 9.6.1.4-10: PUCCH 1-0 static test on multiple cells (TDD-FDD CA with TDD PCell, 7 DL CA)

| Parameter |  | Unit | PCell | SCell1 | SCell2, SCell3, SCell4, SCell5 |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission mode |  |  | 1 |  |  |
| Uplink downlink configuration |  |  | 2 | 2 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Special subframe configuration |  |  | 4 | 4 if Scell1 is TDD Cell N/A if Scell1 is FDD Cell |  |
| Downlink power allocation |  | dB | 0 |  |  |
|  |  | dB | 0 |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |
| SNR |  | dB | 12 | 6 | 0 |
|  |  | dB[mW/15kHz] | -86 | -92 | -98 |
|  |  | dB[mW/15kHz] | -98 | -98 | -98 |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |
| PUCCH Report Type |  |  | 4 |  |  |
| Reporting periodicity |  | ms | Npd = 40 |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 38 | 43 (shift of 5 ms relative to Pcell) | 48 for SCell 2 (shift of 10 ms relative to Pcell), 53 for SCell 3 (shift of 15 ms relative to Pcell), 58 for SCell 4 (shift of 20 ms relative to Pcell), 63 for SCell 5 (shift of 25 ms relative to Pcell), 68 for SCell 5 (shift of 30 ms relative to Pcell) |
| Note 1: 3 symbols are allocated to PDCCH. No PDSCH for user data is scheduled for the UE with one sided dynamic OCNG Pattern OP.1 FDD and OP.1 TDD as described in Annex A.5.1.1 and A.5.2.1. Note 2: Void Note 3: Void |  |  |  |  |  |

Table 9.6.1.4-11: PUCCH 1-0 static test (TDD-FDD CA with TDD PCell, 7 DL CA)

| Test number | Bandwidth combination (MHz) |
| --- | --- |
| 1 | 1x20(FDD)+ 6x20(TDD)MHz |
| 2 | 2x20(FDD)+5x20(TDD)MHz |
| Note 1: The applicability of requirements for different CA configurations and bandwidth combination sets is defined in 9.1.1.2A. The test coverage for different number of component carriers is defined in 9.1.1.3. Note 2: If more than one cell can be configured as PCell, choose one of the cells with the smallest bandwidth as PCell. Mapping of PCell and Scells to the CCs shall be constant for all the iterations during the test. Each execution of the test shall use the same mapping. |  |

< End of changes >