3GPP TSG-RAN WG4 Meeting #89	R4-1814427

Spokane, USA, 8th - 12th Nov, 2018

Agenda item:	7.9.3.2

Source: 	ZTE Corporation .

Title: 	TP to TS 38.141-1:  transmitted signal quality (Section 6.5)

Document for:	Approval

Introduction

This contribution is a text proposal to draft TS 38.141-1 [1] to complete transmitted signal quality conformance testing.

As agreed in the WF, the NR EVM test case should be optimized compared with legacy LTE EVM test case.

Agreements for reduction of BS NR conformance test cases,

Modulation scheme for EVM requirement:

For supported modulation orders without power back off, the EVM limit should be tested only at the highest modulation order.

For the supported modulation order(s) with power back off declared by BS vendor, the EVM limit should be tested only at the highest modulation order(s) with same reduced output power.

The interpretation on the above agreement could be as following:

FR1:

if 256QAM is transmitted with max output power, then only 256QAM EVM is tested;

if 256QAM is declared with power backoff, then 64QAM with maximum output power and 256QAM with power backoff is tested.

- Reference

- Draft TS 38.141-1 v1.1.0_afterChengdu, Huawei.

- R4-1808318,WF on test cases for NR conformance testing,Huawei, Ericsson, Nokia, ZTE, approved.

Text Proposal

--------------Start of text proposal-------------

6.5	Transmitted signal quality

6.5.1	General

Unless otherwise stated, the requirements in clause 6.5 apply during the transmitter ON period.

6.5.2	Frequency error

6.5.2.1	Definition and applicability

Frequency error is the measure of the difference between the actual NR BS transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

It is not possible to verify by testing that the data clock is derived from the same frequency source as used for RF generation. This may be confirmed by the manufacturer’s declaration.

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

6.5.2.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2], subclause 6.5.1.2.

6.5.2.3	Test purpose

The test purpose is to verify that frequency error is within the limit specified by the minimum requirement.

6.5.2.4	Method of test

Requirement is tested together with modulation quality test, as described in subclause 6.5.3.

6.5.2.5	Test Requirements

The modulated carrier frequency of each NR carrier configured by the BS shall be accurate to within the accuracy range given in table 6.5.2.5-1 observed over 1 ms.

Table 6.5.2.5-1: Frequency error test requirement

| BS class | Accuracy |
| --- | --- |
| Wide Area BS | ±(0.05 ppm + 12 Hz) |
| Medium Range BS | ±(0.1 ppm + 12 Hz) |
| Local Area BS | ±(0.1 ppm + 12 Hz) |

6.5.3	Modulation quality

6.5.3.1	Definition and applicability

Modulation quality is defined by the difference between the measured carrier signal and a reference signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector.

For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band.

For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band.

6.5.3.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2], subclause 6.5.2.2.

6.5.3.3	Test purpose

The test purpose is to verify that modulation quality is within the limit specified by the minimum requirement.

6.5.3.4	Method of test

6.5.3.4.1	Initial conditions

Test environment:

-	normal; see annex B.2.

RF channels to be tested for single carrier:

-	B, M and T; see subclause 4.9.1.

RF bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW, MRFBW and TRFBW in single-band operation, see subclause 4.9.1; BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

6.5.3.4.2	Procedure

The minimum requirement is applied to all antenna connectors or TAB connectors, they may be tested one at a time or multiple antenna connectors or TAB connectors may be tested in parallel as shown in annex D.1.1 for BS type 1-C or in annex D.3.1 for BS type 1-H. Whichever method is used the procedure is repeated until all antenna connectors or TAB connectors necessary to demonstrate conformance have been tested.

1)	For an antenna connector or TAB connector declared to be capable of single carrier operation only (D.20), set the antenna connector or the TAB connector to transmit a signal according to NR-FR1-TM 3.1 if 256QAM is not supported by BS or according to NR-FR1-TM 3.1a if 256QAM is supported by BS without power back off or according to NR-FR1-TM3.1 and NR-FR1-TM3.1a if 256QAM is supported by BS with power back off, at manufacturer's declared rated output power.

For an antenna connector or TAB connector declared to be capable of multi-carrier and/or CA operation, set the antenna connector or the TAB connector to transmit according to NR-FR1-TM 3.1 if 256QAM is not supported by BS or according to NR-FR1-TM 3.1a if 256QAM is supported by BS without power back off or according to NR-FR1-TM3.1 and NR-FR1-TM3.1a if 256QAM is supported by BS with power back off, on all carriers configured using the applicable test configuration and corresponding power setting specified in subclause 4.7.

For  NR-FR1-TM 3.1a, power back-off shall be applied if it is declared.

2)	Measure the EVM and frequency error as defined in annex F.

3)	Repeat steps 1 and 2 for NR-FR1-TM 2 if 256QAM is not supported by BS or for NR-FR1-TM 2a if 256QAM is supported by BS. For NR-FR1-TM 2 and NR-FR1-TM 2a the OFDM symbol power shall be at the lower limit of the dynamic range according to the test procedure in subclause 6.3.3.4 and test requirements in subclause 6.3.3.5.

In addition, for multi-band connector(s), the following steps shall apply:

4)	For multi-band connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.3.5	Test requirements

The EVM of each NR carrier for different modulation schemes on PDSCH shall be less than the limits in table 6.5.3.5-1.

Table 6.5.3.5-1 EVM requirements for BS type 1-C and BS type 1-H

| Modulation scheme for PDSCH | Required EVM (%) |
| --- | --- |
| QPSK | 18.5 % |
| 16QAM | 13.5 % |
| 64QAM | 9 % |
| 256QAM | 4.5 % |

EVM shall be evaluated for each NR carrier over all allocated resource blocks and downlink subframes and with RS density configuration of DM-RS of comb 2 (every other subcarrier) in symbol 3 and 11. Different modulation schemes listed in table 6.5.3.5-1 shall be considered for rank 1.

For NR, for all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink subframes within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

Table 6.5.3.5-2, 6.5.3.5-3, 6.5.3.5-4 below specify the EVM window length (W) for normal CP for BS type 1-C and BS type 1-H.

Table 6.5.3.5-2 EVM window length for normal CP for NR, FR1, 15 kHz SCS

| Channel Bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (%) (Note) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| 10 | 1024 | 72 | 28 | 40 |
| 15 | 1536 | 108 | 44 | 40 |
| 20 | 2048 | 144 | 58 | 40 |
| 25 | 2048 | 144 | 72 | 50 |
| 30 | 3072 | 216 | 108 | 50 |
| 40 | 4096 | 288 | 144 | 50 |
| 50 | 4096 | 288 | 144 | 50 |
| Note: These percentages are informative and apply to a slot’s symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

Table 6.5.3.5-3 EVM window length for normal CP for NR, FR1, 30 kHz SCS

| Channel Bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-13 (%) (Note) |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | 8 | 40 |
| 10 | 512 | 36 | 14 | 40 |
| 15 | 768 | 54 | 22 | 40 |
| 20 | 1024 | 72 | 28 | 40 |
| 25 | 1024 | 72 | 36 | 50 |
| 30 | 1536 | 108 | 54 | 50 |
| 40 | 2048 | 144 | 72 | 50 |
| 50 | 2048 | 144 | 72 | 50 |
| 60 | 3072 | 216 | 130 | 60 |
| 70 | 3072 | 216 | 130 | 60 |
| 80 | 4096 | 288 | 172 | 60 |
| 90 | 4096 | 288 | 172 | 60 |
| 100 | 4096 | 288 | 172 | 60 |
| Note: These percentages are informative and apply to a slot’s symbols 1 through 13. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

Table 6.5.3.5-4 EVM window length for normal CP for NR, FR1, 60 kHz SCS

| Channel Bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-13 (%) (Note) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 18 | 8 | 40 |
| 15 | 384 | 27 | 11 | 40 |
| 20 | 512 | 36 | 14 | 40 |
| 25 | 512 | 36 | 18 | 50 |
| 30 | 768 | 54 | 26 | 50 |
| 40 | 1024 | 72 | 36 | 50 |
| 50 | 1024 | 72 | 36 | 50 |
| 60 | 1536 | 108 | 64 | 60 |
| 70 | 1536 | 108 | 64 | 60 |
| 80 | 2048 | 144 | 86 | 60 |
| 90 | 2048 | 144 | 86 | 60 |
| 100 | 2048 | 144 | 86 | 60 |
| Note : These percentages are informative and apply to a slot’s symbols 1 through 13. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |

6.5.4		Time alignment error

6.5.4.1	Definition and applicability

This requirement applies to frame timing in TX diversity, MIMO transmission, carrier aggregation and their combinations.

Frames of the NR signals present at the BS transmitter antenna connectors or TAB connectors are not perfectly aligned in time and may experience certain timing differences in relation to each other.

For BS type 1-C, the TAE is defined as the largest timing difference between any two signals belonging to different antenna connectors for a specific set of signals/transmitter configuration/transmission mode.

For BS type 1-H, the TAE is defined as the largest timing difference between any two signals belonging to TAB connectors belonging to different transmitter groups at the transceiver array boundary, where transmitter groups are associated with the TAB connectors in the transceiver unit array corresponding to TX diversity, MIMO transmission, carrier aggregation for a specific set of signals/transmitter configuration/transmission mode.

6.5.4.2	Minimum requirement

The minimum requirements for BS type 1-C and BS type 1-H are in TS 38.104 [2], subclause 6.5.3.2.

6.5.4.3	Test purpose

To verify that the time alignment error is within the limit specified by the minimum requirement.

6.5.4.4	Method of test

6.5.4.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: 	M; see subclause 4.9.1.

RF bandwidth positions to be tested for multi-carrier and/or CA:

-	MRFBW in single-band operation, see subclause 4.9.1.

-	 BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation, see subclause 4.9.1.

6.5.4.4.2	Procedure

For BS type 1-C antenna connectors to be tested are for a specific set of signals/transmitter configuration/transmission mode.

For BS type 1-H TAB connectors to be tested are identified from the declared sets of TAB connector beam forming groups in the TAE groups declaration (D.40).

Compliance is to be demonstrated between all pairs of single-band connectors and/or multi-band connectors, however it is not required to exhaustively measure TAE between every combination of pairs of representative connectors. Compliance can be demonstrated by comparison of a reduced set of representative measurement results.

1)	Conducted measurement setup:

- For BS type 1-C:  Connect two antenna connectors to the measurement equipment according to annex D.1.3. Terminate any unused antenna connector(s).

- For BS type 1-H: Connect two representative TAB connectors one from separate TAE group (D.40) to the measurement equipment according to annex D.3.x. Terminate any unused TAB connector(s).

2)	Set the connectors under test to transmit NR-FR1-TM 1.1 or any DL signal using TX diversity, MIMO transmission or carrier aggregation.

NOTE:	For TX diversity and MIMO transmission, different ports may be configured in NR-FR1-TM 1.1.

3)	For a connectors declared to be capable of single carrier operation only (D.20), set the representative connectors under test to transmit at rated carrier output power (Prated,c,AC, or Prated,c,TABC, D.25).

If the connector under test supports intra band contiguous or non-contiguous CA, set the representative connectors to transmit using the applicable test configuration and corresponding power setting specified in subclause 4.7.

If the BS supports inter band CA, set the representative connectors to transmit, for each band, a single carrier or all carriers, using the applicable test configuration and corresponding power setting specified in subclause 4.7.

4)	Measure the time alignment error between the reference symbols on the carrier(s) from the representative connectors under test.

5)	Repeat step 1 - 4 for any other configuration of connectors, which could be required to demonstrate compliance.

In addition, for multi-band connectors, the following steps shall apply:

6)	For a multi-band connectors and single band tests, repeat the steps above per involved operating band where single band test configurations and test models shall apply with no carrier activated in the other operating band.

6.5.4.5	Test requirement

For MIMO or TX diversity transmissions, at each carrier frequency, TAE shall not exceed 90 ns.

For intra-band contiguous CA, with or without MIMO or TX diversity, TAE shall not exceed 285 ns.

For intra-band non-contiguous CA, with or without MIMO or TX diversity, TAE shall not exceed 3.025 µs.

For inter-band CA, with or without MIMO or TX diversity, TAE shall not exceed 3.025 µs.

--------------End of text proposal-------------