3GPP TSG-RAN WG4 Meeting #89	R4-1814462

# Spokane, WA, USA, 12th – 16th Nov 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Darft CR for EN-DC MTTD and MRTD requirement (section 7.5 and 7.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Correction and editorial revision are needed for NR EN-DC MTTD and MRTD requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For inter-band EN-DC asynchronous operation, the requirement of MTTD and MRTD is applicable for all related scenarios, so no explicit mentioning of all scenrios is needed in both Section 7.5.2 and 7.6.2. For inter-band EN-DC synchronous operation, UE is mandatory to support and no capability signaing is indicated. Therefore, the expression “provided that the UE indicates that it is capable of synchronous EN-DC [16]” is deleted from Section 7.5.2 and 7.6.2. Furthermore, the scenario of E-UTRA FDD-NR FDD is added for synchronous inter-band EN-DC requirement. For intra-band EN-DC asynchronous operation, there is only E-UTRA FDD-NR FDD scenario depending on UE capability, and there is no E-UTRA TDD-NR TDD intra-band EN-DC asynchronous scenario defined in Rel-15. Therefore, the corresponding parts are corrected in Section 7.5.3 and 7.6.3. For intra-band EN-DC synchronous operation, UE is mandatory to support it in E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD scenarios. Therefore, the expression “provided the UE indicates that it is only capable of synchronous EN-DC [16]” is deleted from Section 7.6.3, and these two scenarios are explicitly mentioned in Section 7.5.3 for more clear description. The notes in Table 7.5.2-1 and Table 7.6.2-1 is revised to better reflect why the note is related to intra-band EN-DC, while the table is located in the section for inter-band EN-DC. The notes in Table 7.5.2-2 and Table 7.6.2-2 is not needed, since the counterpart for intra-band EN-DC does not refer to these two tables. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some description for NSA EN-DC UE measurement capability is not clear or not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5, 7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

## 7.5	Maximum Transmission Timing Difference

### 7.5.1	Introduction

A UE shall be capable of handling a relative transmission timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell to be aggregated for EN-DC operation.

A UE shall be capable of handling a relative transmission timing difference among slot timing boundaries of different carriers to be aggregated in NR carrier aggregation.

### 7.5.2	Minimum Requirements for inter-band EN-DC

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1.

Table 7.5.2-1 Maximum uplink transmission timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note1 | 62.5 |
| NOTE 1: For E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC, for which the requirement is defined in Section 7.5.3 and this Table 7.5.2-1 is also applicable, the scenario with 120kHz PSCell does not exist. |  |  |

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-2. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA FDD-NR FDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC.

Table 7.5.2-2 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 | 35.21 |
| 15 | 60 | 35.21 |
| 15 | 120 | 35.21 |

### 7.5.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling a maximum uplink transmission timing difference between E-UTRA PCell and PSCell as shown in Table 7.5.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation [16].

No requirement on maximum uplink transmission timing difference is applicable for synchronous E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC.

### 7.5.4	Minimum Requirements for NR Carrier Aggregation

The UE shall be capable of handling at least a relative transmission timing difference between slot timing of different TAGsas shown in Table 7.5.4-1, provided that the UE is:

-	configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA mode. or

-	configured with more than one sTAG for inter-band NR carrier aggregation in NSA mode.

Table 7.5.4-1: Maximum transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range | Maximum transmission timing difference (µs) |
| --- | --- |
| FR1 | 35.21 |
| FR2 | 8.5 |
| Between FR1 and FR2 | [TBD] |

## 7.6	Maximum Receive Timing Difference

### 7.6.1	Introduction

A UE shall be capable of handling a relative receive timing difference between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell to be aggregated for EN-DC operation.

A UE shall be capable of handling a relative receive timing difference among slot timing boundaries of different carriers to be aggregated in NR carrier aggregation.

### 7.6.2	Minimum Requirements for inter-band EN-DC

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-1.

Table 7.6.2-1: Maximum receive timing difference requirement for asynchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) (Note 1) | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 500 |
| 15 | 30 | 250 |
| 15 | 60 | 125 |
| 15 | 120Note2 | 62.5 |
| NOTE 1 : DL Sub-carrier spacing is min{SCSSS, SCSDATA}. NOTE 2 : For E-UTRA FDD-NR FDD and E-UTRA TDD-NR TDD intra-band EN-DC, for which the requirement is defined in Section 7.6.3 and this Table 7.6.2-1 is also applicable, the scenario with 120kHz does not exit. |  |  |

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell at the UE receiver as shown in Table 7.6.2-2. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD, E-UTRA FDD-NR FDD, E-UTRA TDD-NR FDD and E-UTRA FDD-NR TDD inter-band EN-DC.

Table 7.6.2-2: Maximum receive timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) (Note1) | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 33 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |
| NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

Table 7.6.2-3	Void

### 7.6.3	Minimum Requirements for intra-band EN-DC

For intra-band EN-DC, only collocated deployment is applied.

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.2-1 for E-UTRA FDD-NR FDD intra-band EN-DC provided the UE indicates that it is capable of asynchronous EN-DC operation[16].

The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from E-UTRA PCell and slot timing of signal from PSCell as shown in Table 7.6.3-1. The requirements for synchronous EN-DC are applicable for E-UTRA TDD-NR TDD and E-UTRA FDD-NR FDD intra-band EN-DC.

Table 7.6.3-1 Maximum receive timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | DL Sub-carrier spacing in PSCell (kHz) Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 3 |
| 15 | 30 | 3 |
| 15 | 60 | 3 |
| NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

Table 7.6.3-2	Void

### 7.6.4	Minimum Requirements for NR Carrier Aggregation

For intra-band CA, only collocated deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-1 below.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 3 |
| FR2 | 3 |

For inter-band NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in Table 7.6.4-2 below.

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2 | 8 |
| Between FR1 and FR2 | [TBD] |

### >>>>>End of Change 1<<<<<