3GPP TSG-RAN WG4 Meeting #89                 	R4-1814478

Spokane, US, 12th – 16th Nov 2018

Agenda item:	7.13.2.3.2

Source:	Samsung

Title:	TP for TS 38.141-1 on NR PUCCH format 2 conducted performance requirements

Document for:	Approval

- Introduction

This contribution provides a text proposal on PUCCH format 2 for section 8.3.3 for TS38.141-1 [1] according to the approved specification drafting plan [2].

# Reference

[1] 3GPP TS38.141-1, NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing, v1.1.0, Nov 2018.

[2] R4-1811729, Specification drafting plan for NR BS demodulation performance requirements, China Telecom, August 2018.

# Text Proposal

## ----- Start of TP -----

## 8.3	Performance requirements for PUCCH

### 8.3.3	Performance requirements for PUCCH format 2

#### 8.3.3.1	NACK to ACK detection

##### 8.3.3.1.1	Definition and applicability

The performance requirement of PUCCH format 2 for NACK to ACK detection is determined by the two parameters: probability of false detection of the ACK and the NACK to ACK detection probability. The performance is measured by the required SNR at probability of the NACK to ACK detection equal to 0.001 or less. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK at particular bit position when input is only noise. Each false bit detection is counted as one error.

The NACK to ACK detection probability is the probability of detecting an ACK bit when an NACK bit was sent on particular bit position. Each NACK bit erroneously detected as ACK bit is counted as one error. Erroneously detected NACK bits in the definition do not contain the NACK bits which are mapped from DTX, i.e. NACK bits received when DTX is sent should not be considered.

A test for a specific combination of SCS and channel bandwidth is only applicable if the BS declares to support it.

The applicability of tests in TS 38.104 [2] with different SCS and channel bandwidth combination is according to the following principle:

Editor’s note: Applicability rule is FFS.

The number of encoded ACK/NACK bits per sub-frame is defined as presented below, with random codeword selection

- 4AN bits.

##### 8.3.3.1.2	Minimum requirement

The minimum requirement is in TS 38.104 [1] subclause 8.3.4.

##### 8.3.3.1.3	Test purpose

The test shall verify the receiver’s ability not to falsely detect NACK bits as ACK bits under multipath fading propagation conditions for a given SNR.

##### 8.3.3.1.4	Method of test

###### 8.3.3.1.4.1	Initial Condition

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	for single carrier (SC): M; see subclause 4.9.1

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex [I.3.2].

###### 8.3.3.1.4.2	Procedure

1)	Adjust the AWGN generator, according to the combinations of SCS and channel bandwidth defined in table 8.3.3.1.4.2-1.

Table 8.3.3.1.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5 MHz |
|  | 10 | [-80.3] dBm / 9.36 MHz |
|  | 40 | [-74.1] dBm / 38.88 MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64 MHz |
|  | 20 | [-77.4] dBm / 18.36 MHz |
|  | 40 | [-74.2] dBm / 38.16 MHz |
|  | 100 | [-70.1] dBm / 98.28 MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [TBD], and the specific test parameters are configured as blow:

Table 8.3.3.1.4.2-2: Test parameters

| Parameter | Test |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| nrofPRBs | 4 |
| nrofSymbols | 1 |
| the number of UCI bits | 14 |
| startingSymbolIndex | 13 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex TBD

4)	Adjust the equipment so that the SNR specified in table 8.3.3.1.5-1 and table 8.3.3.1.5-2 is achieved at the BS input during the UCI transmissions.

5)	The signal generator sends random codeword from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of NACK bits detected as ACK.

##### 8.3.3.1.5	Test requirements

The fraction of falsely detected ACK bits shall be less than 1% and the fraction of NACK bits falsely detected as ACK shall be less than 0.1% for the SNR listed in table 8.3.3.1.5-1 and table 8.3.3.1.5-2.

Table 8.3.3.1.5-1: Required SNR for PUCCH format 2 demodulation tests with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex [TBD]) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |

Table 8.3.3.1.5-2: Required SNR for PUCCH format 2 demodulation tests with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex TBD) | Channel Bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |

#### 8.3.3.2	ACK missed detection

##### 8.3.3.2.1	Definition and applicability

The performance requirement of PUCCH format 2 for ACK missed detection is determined by the two parameters: probability of false detection of the ACK and the probability of detection of ACK on the wanted signal. The performance is measured by the required SNR at probability of detection equal to 0.99. The probability of false detection of the ACK shall be 0.01 or less.

The probability of false detection of the ACK is defined as a conditional probability of erroneous detection of the ACK when input is only noise

The probability of detection of ACK is defined as conditional probability of detection of the ACK when the signal is present.

A test for a specific combination of SCS and channel bandwidth is only applicable if the BS declares to support it.

The applicability of tests in TS 38.104 [2] with different SCS and channel bandwidth combination is according to the following principle:

Editor’s note: Applicability rule is FFS.

The number of encoded ACK/NACK bits per sub-frame is defined as presented below, with random codeword selection

- 4 bits.

##### 8.3.3.2.2	Minimum requirements

The minimum requirements are in TS 38.104 [1] subclause 8.3.4.

##### 8.3.3.2.3	Test purpose

The test shall verify the receiver’s ability to detect ACK bits under multipath fading propagation conditions for a given SNR.

##### 8.3.3.2.4	Method of test

###### 8.3.3.2.4.1	Initial Condition

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	for single carrier (SC): M; see subclause 4.9.1

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex [I.3.2].

###### 8.3.3.2.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth defined in table 8.3.3.2.4.2-1.

Table 8.3.3.2.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5 MHz |
|  | 10 | [-80.3] dBm / 9.36 MHz |
|  | 40 | [-74.1] dBm / 38.88 MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64 MHz |
|  | 20 | [-77.4] dBm / 18.36 MHz |
|  | 40 | [-74.2] dBm / 38.16 MHz |
|  | 100 | [-70.1] dBm / 98.28 MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [TBD], and the specific test parameters are configured as blow:

Table 8.3.3.2.4.2-2: Test parameters

| Parameter | Test |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| nrofPRBs | 4 |
| nrofSymbols | 1 |
| the number of UCI bits | 14 |
| startingSymbolIndex | 13 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in [TBD]

4)	Adjust the equipment so that the SNR specified in table 8.3.6.5-1 and table 8.3.6.5-1 is achieved at the BS input during the UCI transmissions.

5)	The signal generator sends random codeword from applicable codebook, in regular time periods. The following statistics are kept: the number of ACK bits detected in the idle periods and the number of NACK bits detected as ACK.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.3.2.4.2-1. The following statistics are kept: the number of ACKs detected in the idle periods and the number of missed ACKs.

Figure 8.3.3.2.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

##### 8.3.3.2.5	Test requirements

The fraction of falsely detected ACKs on the wanted signal shall be less than 1% and the fraction of correctly detected ACKs shall be larger than 99% for the SNR listed in in table 8.3.3.2.5-1 and table 8.3.3.2.5-2.

Table 8.3.3.2.5-1: Required SNR for PUCCH format 2 demodulation tests with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex [TBD]) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |

Table 8.3.3.2.5-2: Required SNR for PUCCH format 2 demodulation tests with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex TBD) | Channel Bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |

#### 8.3.3.3	UCI performance requirements

##### 8.3.3.3.1	Definition and applicability

The performance is measured by the required SNR at UCI block error probability not exceeding 1%.

The UCI block error probability is defined as the conditional probability of incorrectly decoding the UCI information when the UCI information is sent. All UCI information shall be decoded.

A test for a specific combination of SCS and channel bandwidth is only applicable if the BS declares to support it.

The applicability of tests in TS 38.104 [2] with different SCS and channel bandwidth combination is according to the following principle:

Editor’s note: Applicability rule is FFS.

The number of encoded UCI bits per sub-frame is defined as presented below, with random codeword selection

- 22 bits.

##### 8.3.3.3.2	Minimum Requirement

The minimum requirement is TS 38.104 [2] sub-clause 8.3.4.

##### 8.3.3.3.3	Test purpose

The test shall verify the receiver’s ability to detect UCI under multipath fading propagation conditions for a given SNR.

##### 8.3.3.3.4	Method of test

###### 8.3.3.3.4.1	Initial Condition

Test environment:	Normal, see subclause B.2.

RF channels to be tested:	for single carrier (SC): M; see subclause 4.9.1

1)	Connect the BS tester generating the wanted signal, multipath fading simulators and AWGN generators to all BS antenna connectors for diversity reception via a combining network as shown in Annex [I.3.2].

###### 8.3.3.3.4.2	Procedure

1)	Adjust the AWGN generator, according to the channel bandwidth defined in table 8.3.3.3.4.2-1.

Table 8.3.3.3.4.2-1: AWGN power level at the BS input

| Subcarrier spacing(SCS) (kHz) | Channel bandwidth [MHz] | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | [-83.5] dBm / 4.5MHz |
|  | 10 | [-80.3] dBm / 9.36MHz |
|  | 40 | [-74.1] dBm / 38.88MHz |
| 30 kHz | 10 | [-80.7] dBm / 8.64MHz |
|  | 20 | [-77.4] dBm / 18.36MHz |
|  | 40 | [-74.2] dBm / 38.16MHz |
|  | 100 | [-70.1] dBm / 98.28MHz |

2)	The characteristics of the wanted signal shall be configured according to TS 38.211 [TBD], and the specific test parameters are configured as blow:

Table 8.3.3.3.4.2-2: Test parameters

| Parameter | Test |
| --- | --- |
| Modulation | QPSK |
| startingPRB | 0 |
| intraSlotFrequencyHopping | enabled |
| secondHopPRB | The largest PRB index - nrofPRBs |
| nrofPRBs | 9 |
| nrofSymbols | 2 |
| the number of UCI bits | 22 |
| startingSymbolIndex | 12 |

3)	The multipath fading emulators shall be configured according to the corresponding channel model defined in Annex TBD.

4)	Adjust the equipment so that the SNR specified in table 8.3.3.3.5-1 or table 8.3.3.3.5-2 is achieved at the BS input during the UCI transmissions.

5)	The signal generator sends a test pattern with the pattern outlined in figure 8.3.3.3.4.2-1. The following statistics are kept: the number of incorrectly decoded UCI.

Figure 8.3.3.3.4.2-1: Test signal pattern for PUCCH format 2 demodulation tests

##### 8.3.3.3.5	Test requirements

The fraction of incorrectly decoded UCI is shall be less than 1% for the SNR listed in table 8.3.3.3.5-1 and table 8.3.3.3.5-2.

Table 8.3.3.3.5-1: Required SNR for PUCCH format 2 demodulation tests with 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex [TBD]) | Channel Bandwidth / SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 MHz | 10 MHz | 20 MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] |

Table 8.3.3.3.5-2: Required SNR for PUCCH format 2 demodulation tests with 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex TBD) | Channel Bandwidth/ SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10MHz | 20MHz | 40MHz | 100MHz |
| 1 | 2 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 4 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |
|  | 8 | Normal | TDLC300-100 Low | [TBD] | [TBD] | [TBD] | [TBD] |

## ----- End of TP -----