3GPP TSG-RAN WG4 Meeting RAN4#89                                              R4-1814493

Spokane ,US, 12 -16 Nov, 2018

Agenda Item:   7.13.1.4.4

Source:        Samsung

Title:          Simulation results for RI test cases

Document for:  Discussion

# Introduction

In last RAN4 meeting, list of test cases has been captured in WF [1]:

RI test - Test 1: FR1 FDD 10MHz&15kHz, 2*2,4*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 4*4 - Test 3: FR2 100MHz&120kHz, 2*2

In this contribution, initial simulation results for RI test cases provided.

# Discussion

## Simulation assumption

| Parameter | Unit | Test 1 (FR1 FDD) | Test 2 (FR1 FDD) | Test 3 FR2 |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | 7D1S2U | DDSU |
| Propagation channel |  | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | TDL_A 30ns, 75Hz |
| Correlation and antenna configuration |  | 2Rx: 2*2 ULA Low, ULA High 4Rx: 2*4 ULA Low, ULA High 4*4 ULA Low | 2Rx: 2*2 ULA Low, ULA High 4Rx: 2*4 ULA Low, ULA High 4*4 ULA Low | 2Rx: 2*2 [ULA Low, XP High] |
| CSI-RS resource setting (periodic) |  |  |  |  |
| resourceConfigType |  | Periodic | Periodic | Option 1:Periodic Option 2: Aperiodic |
| NZP CSI-RS ports |  | 2/4 | 2/4 | 2 |
| reportConfigType |  | periodic | periodic | Option 1:Periodic Option 2: Aperiodic |
| Reporting periodicity and slot offset |  | 5/1 | 10/1 | FFS |
| Codebook Restriction |  | FFS | FFS | FFS |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

## Simulation results

Simulation results for FR1 FDD

Table 1 RI test with 2*2

| SNR | 2*2 Low |  | 2*2 High |  |
| --- | --- | --- | --- | --- |
|  | Follow RI/Fixed RI=1 | Follow RI/Fixed RI=2 | Follow RI/Fixed RI=1 | Follow RI/Fixed RI=2 |
| -6 | 1.26 | 1.74 | 0.92 | 3.65 |
| -4 | 1.04 | 1.12 | 0.78 | 2.08 |
| -2 | 1.29 | 1.30 | 0.93 | 2.15 |
| 0 | 1.17 | 1.32 | 0.99 | 2.10 |
| 2 | 1 | 1.50 | 1 | 2.20 |
| 4 | 1.21 | 1.29 | 1.00 | 2.35 |
| 6 | 1.19 | 1.27 | 1.00 | 2.60 |
| 8 | 1.17 | 1.22 | 1.00 | 2.79 |
| 10 | 1.13 | 1.09 | 1.00 | 2.41 |
| 12 | 1.13 | 1.04 | 1.00 | 1.99 |
| 14 | 1.13 | 1.02 | 1.00 | 1.71 |
| 16 | 1.19 | 1.00 | 1.00 | 1.55 |
| 18 | 1.23 | 0.98 | 1.00 | 1.46 |
| 20 | 1.36 | 0.96 | 1.00 | 1.36 |
| 22 | 1.44 | 0.95 | 1.00 | 1.26 |
| 24 | 1.58 | 0.95 | 1.00 | 1.23 |
| 26 | 1.63 | 0.95 | 1.00 | 1.18 |
| 28 | 1.66 | 0.96 | 1.00 | 1.16 |
| 30 | 1.73 | 0.97 | 1.00 | 1.16 |

Table 2 RI test with 4*4

| SNR [dB] | 2*4 Low |  | 2*4 High |  | 4*4 High |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RI=1 | RI=2 | RI=1 | RI=2 | RI=1 | RI=2 | RI=3 | RI=4 |
| -6 | 1.10 | 1.21 | 0.85 | 3.15 |  |  |  |  |
| -4 | 1.12 | 1.25 | 1.00 | 3.09 |  |  |  |  |
| -2 | 1.09 | 1.24 | 1.00 | 2.87 | 0.70 | 1.02 | 1.11 | 1.14 |
| 0 | 1.10 | 0.99 | 1.00 | 2.92 | 1.00 | 1.15 | 1.30 | 1.30 |
| 2 | 1 | 1.11 | 1 | 2.93 | 0.80 | 0.98 | 1.32 | 1.09 |
| 4 | 1.17 | 1.05 | 1.00 | 2.73 | 0.91 | 1.09 | 1.02 | 1.46 |
| 6 | 1.26 | 0.99 | 1.00 | 2.50 | 1.03 | 1.04 | 1.16 | 1.01 |
| 8 | 1.28 | 0.98 | 1.00 | 2.50 | 1.07 | 1.01 | 1.13 | 1.06 |
| 10 | 1.35 | 1.03 | 1.00 | 2.24 | 1.12 | 1.02 | 0.99 | 1.16 |
| 12 | 1.41 | 1.02 | 1.00 | 2.00 | 1.13 | 1.01 | 0.98 | 0.94 |
| 14 | 1.52 | 1.02 | 1.00 | 1.94 | 1.12 | 0.95 | 0.96 | 0.86 |
| 16 | 1.68 | 1.01 | 1.00 | 1.99 | 1.25 | 1.02 | 1.13 | 1.00 |
| 18 | 1.72 | 0.99 | 1.00 | 1.92 | 1.00 | 0.99 | 1.07 | 1.18 |
| 20 | 1.77 | 0.97 | 1.00 | 1.62 | 1.38 | 1.01 | 1.05 | 1.58 |
| 22 | 1.78 | 0.98 | 1.00 | 1.38 | 1.39 | 1.08 | 1.06 | 1.54 |
| 24 | 1.83 | 0.98 | 1.00 | 1.19 | 1.14 | 0.82 | 0.83 | 1.16 |
| 26 | 1.75 | 0.97 | 1.00 | 1.07 |  |  |  |  |
| 28 | 1.70 | 0.93 | 1.00 | 1.02 |  |  |  |  |
| 30 | 1.66 | 0.93 | 1.00 | 1.01 |  |  |  |  |

Simulation results for FR1 TDD

Table 3 RI test with 2*2

| SNR | 2*2 Low |  | 2*2 High |  |
| --- | --- | --- | --- | --- |
|  | Follow RI/Fixed RI=1 | Follow RI/Fixed RI=2 | Follow RI/Fixed RI=1 | Follow RI/Fixed RI=2 |
| -6 | 1.20 | 1.86 | 0.89 | 3.74 |
| -4 | 1.07 | 1.23 | 0.75 | 2.23 |
| -2 | 1.19 | 1.42 | 0.90 | 2.20 |
| 0 | 1.23 | 1.46 | 0.99 | 2.27 |
| 2 | 1 | 1.51 | 1 | 2.29 |
| 4 | 1.18 | 1.37 | 1.00 | 2.35 |
| 6 | 1.17 | 1.37 | 1.00 | 2.47 |
| 8 | 1.15 | 1.25 | 1.00 | 2.62 |
| 10 | 1.11 | 1.10 | 1.00 | 2.49 |
| 12 | 1.09 | 1.03 | 1.00 | 2.24 |
| 14 | 1.10 | 1.00 | 1.00 | 1.77 |
| 16 | 1.16 | 0.95 | 1.00 | 1.56 |
| 18 | 1.26 | 0.96 | 1.00 | 1.39 |
| 20 | 1.40 | 0.93 | 1.00 | 1.32 |
| 22 | 1.46 | 0.93 | 1.00 | 1.21 |
| 24 | 1.62 | 0.92 | 1.00 | 1.15 |
| 26 | 1.66 | 0.94 | 1.00 | 1.12 |
| 28 | 1.80 | 0.95 | 1.00 | 1.12 |
| 30 | 1.82 | 0.96 | 1.00 | 1.13 |

Table 4 RI test with 4*4

| SNR [dB] | 2*4 Low |  | 2*4 High |  | 4*4 High |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RI=1 | RI=2 | RI=1 | RI=2 | RI=1 | RI=2 | RI=3 | RI=4 |
| -6 | 1.05 | 1.29 | 0.75 | 2.86 |  |  |  |  |
| -4 | 1.11 | 1.32 | 0.99 | 3.27 |  |  |  |  |
| -2 | 1.11 | 1.33 | 1.00 | 2.85 |  |  |  |  |
| 0 | 1.04 | 1.08 | 1.00 | 2.93 |  |  |  |  |
| 2 | 1 | 1.21 | 1 | 2.82 |  |  |  |  |
| 4 | 1.11 | 1.10 | 1.00 | 2.54 |  |  |  |  |
| 6 | 1.19 | 1.05 | 1.00 | 2.51 |  |  |  |  |
| 8 | 1.16 | 1.02 | 1.00 | 2.48 |  |  |  |  |
| 10 | 1.23 | 1.03 | 1.00 | 2.25 |  |  |  |  |
| 12 | 1.27 | 1.04 | 1.00 | 2.20 |  |  |  |  |
| 14 | 1.37 | 1.02 | 1.00 | 1.96 |  |  |  |  |
| 16 | 1.54 | 1.03 | 1.00 | 1.98 |  |  |  |  |
| 18 | 1.70 | 0.99 | 1.00 | 1.92 |  |  |  |  |
| 20 | 1.62 | 0.94 | 1.00 | 1.68 |  |  |  |  |
| 22 | 1.61 | 0.96 | 1.00 | 1.51 |  |  |  |  |
| 24 | 1.63 | 0.95 | 1.00 | 1.19 |  |  |  |  |
| 26 | 1.58 | 0.98 | 1.00 | 1.05 |  |  |  |  |
| 28 | 1.42 | 0.91 | 1.00 | 1.01 |  |  |  |  |
| 30 | 1.38 | 0.93 | 1.00 | 1.01 |  |  |  |  |

Simulation results for FR2 TDD

Table 4 RI test with 2*2

| SNR | 2*2 Low |  | 2*2 High |  |
| --- | --- | --- | --- | --- |
|  | Follow RI/Fixed RI=1 | Follow RI/Fixed RI=2 | Follow RI/Fixed RI=1 | Follow RI/Fixed RI=2 |
| -6 | 1.05 | 1.29 | 0.75 | 2.86 |
| -4 | 1.11 | 1.32 | 0.99 | 3.27 |
| -2 | 1.11 | 1.33 | 1.00 | 2.85 |
| 0 | 1.04 | 1.08 | 1.00 | 2.93 |
| 2 | 1 | 1.21 | 1 | 2.82 |
| 4 | 1.11 | 1.10 | 1.00 | 2.54 |
| 6 | 1.19 | 1.05 | 1.00 | 2.51 |
| 8 | 1.16 | 1.02 | 1.00 | 2.48 |
| 10 | 1.23 | 1.03 | 1.00 | 2.25 |
| 12 | 1.27 | 1.04 | 1.00 | 2.20 |
| 14 | 1.37 | 1.02 | 1.00 | 1.96 |
| 16 | 1.54 | 1.03 | 1.00 | 1.98 |
| 18 | 1.70 | 0.99 | 1.00 | 1.92 |
| 20 | 1.62 | 0.94 | 1.00 | 1.68 |
| 22 | 1.61 | 0.96 | 1.00 | 1.51 |
| 24 | 1.63 | 0.95 | 1.00 | 1.19 |
| 26 | 1.58 | 0.98 | 1.00 | 1.05 |
| 28 | 1.42 | 0.91 | 1.00 | 1.01 |
| 30 | 1.38 | 0.93 | 1.00 | 1.01 |

With above simulation results, below proposals were given for requirements:

FR1: 2Rx (FDD and TDD)

| Test Number | Test 1 (2X2) | Test 2 (2X2) | Test 3 (2X2) |
| --- | --- | --- | --- |
| MIMO correlation | ULA Low | ULA Low | ULA High for FR1 |
| Metric | Gamma 2-1.0 | Gamma 1-1.05 | Gamma 1 -0.9 |
| SNR | 0 dB | 20 dB | 20 dB |

FR1: 4Rx (FDD and TDD)

| Test Number | Test 1 (2X4) | Test 2 (2X4) | Test 3 (2X4) | Test 4 (4X4) |
| --- | --- | --- | --- | --- |
| MIMO correlation | ULA Low | ULA Low | ULA High | ULA Low |
| Metric | Gamma 2-1.0 | Gamma 1-1.05 | Gamma 1-0.9 | Gamma 2-TBD |
| SNR | -4dB | 16dB | 16dB | TBD |

FR2: 2Rx (TDD)

| Test Number | Test 1 (2X2) | Test 2 (2X2) | Test 3 (2X2) |
| --- | --- | --- | --- |
| MIMO correlation | ULA Low | ULA Low | XP High for FR1 |
| Metric | Gamma 2-1.0 | Gamma 1-1.05 | Gamma 1 -0.9 |
| SNR | -2 dB | 20 dB | 20 dB |

# Conclusion

In this contribution, we provided initial simulation results for RI test cases.

Below proposals were given for requirements:

FR1: 2Rx (FDD and TDD)

| Test Number | Test 1 (2X2) | Test 2 (2X2) | Test 3 (2X2) |
| --- | --- | --- | --- |
| MIMO correlation | ULA Low | ULA Low | ULA High for FR1 |
| Metric | Gamma 2-1.0 | Gamma 1-1.05 | Gamma 1 -0.9 |
| SNR | 0 dB | 20 dB | 20 dB |

FR1: 4Rx (FDD and TDD)

| Test Number | Test 1 (2X4) | Test 2 (2X4) | Test 3 (2X4) | Test 4 (4X4) |
| --- | --- | --- | --- | --- |
| MIMO correlation | ULA Low | ULA Low | ULA High | ULA Low |
| Metric | Gamma 2-1.0 | Gamma 1-1.05 | Gamma 1-0.9 | Gamma 2-TBD |
| SNR | -4dB | 16dB | 16dB | TBD |

FR2: 2Rx (TDD)

| Test Number | Test 1 (2X2) | Test 2 (2X2) | Test 3 (2X2) |
| --- | --- | --- | --- |
| MIMO correlation | ULA Low | ULA Low | XP High for FR1 |
| Metric | Gamma 2-1.0 | Gamma 1-1.05 | Gamma 1 -0.9 |
| SNR | -2 dB | 20 dB | 20 dB |

# Reference

[1] R4-1814235, “Way forward on NR UE CSI requirements”, Samsung

[2] RAN4 email reflector, “Simulation assumption of CSI test cases”, Samsung