3GPP TSG-RAN WG4 Meeting #89	R4-1814515

Spokane, USA, 12th - 16th Nov, 2018

Agenda item:	7.9.4.4.3.1

Source: 	ZTE Corporation .

Title: 	TP to TS 38.141-2: OTA transmitter transient period (Section 6.5.2)

Document for:	Approval

Introduction

This contribution is a text proposal to draft TS 38.141-2 [1] to complete transient period conformance testing based on the contribution [2][3].

- Reference

- Draft TS 38.141-2 v1.1.0_afterChengdu, Huawei.

- R4-1814516, timing alignment for intra-band CA, ZTE

- R4-1814450, Further discussion on the FR2 transient period, ZTE.

--------------Start of text proposal-------------

6.5.2	OTA transmitter transient period

6.5.2.1	Definition and applicability

The OTA transmitter transient period requirements apply only to TDD operation of BS.

The OTA transmitter transient period is the time period during which the transmitter unit is changing from the OFF period to the ON period or vice versa. The OTA transmitter transient period is illustrated in figure 6.5.2.1-1.

Transmitter output power

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transmitter transient

period

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 6.5.2.1-1: Illustration of the relations of transmitter ON period,
transmitter OFF period and transmitter transient period

This requirement applies at each co-location test antenna conducted output supporting reception in the operating band for BS type 1-O, and at each RIB supporting transmission in the operating band for BS type 2-O.

6.5.2.2	Minimum requirement

The minimum requirement for BS type 1-O is in TS 38.104 [2], subclause 9.5.3.2.

The minimum requirement for BS type 2-O is in TS 38.104 [2], subclause 9.5.3.3.

6.5.2.3	Test purpose

The purpose of this test is to verify the OTA transmitter transient periods are within the limits of the minimum requirements.

6.5.2.4	Method of test

6.5.2.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested: M; see subclause 4.9.1.

Base Station RF Bandwidth positions to be tested:

-	MRFBW in single band operation, see subclause 4.9.1;

-	BRFBW_T'RFBW and B'RFBW_TRFBW in multi-band operation; see subclause 4.9.1.

Directions to be tested: The requirement is specified as co-location requirement. For general description of co-location requirements, refer to subclause 4.12.

6.5.2.4.2	Procedure

6.5.2.4.2.1	General procedure

OTA test requires correct use of an appropriate test facility which has been calibrated and is capable of performing the specified measurements.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

6.5.2.4.2.2	BS type 1-O

3)	Place the co-location test antenna as specified in subclause 4.12.

4)	Set the BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test model NR-FR1-TM1.1 and set of physical channels in subclause 4.9 in TS 38.141-1..

5)	Measure the mean power spectrum density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

6)	For an BS supporting contiguous CA, measure the mean power spectral density at the output(s) of co-location test antenna as power sum over all supported polarizations over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is the smallest Sub Carrier Spacing in kHz used in the Aggregated BS Channel Bandwidth.

In addition, for a multi-band RIB, the following steps shall apply:

7)	For a multi-band RIB and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

6.5.2.4.2.3	BS type 2-O

[3)	Place the measurement antenna at the transmitter reference direction

4)	Configure the BS such that the beam peak direction(s) applied during the power measurement step 6 are consistent with the grid and measurement approach for the TRP test.

5)	Set the NR BS to transmit according to the applicable test configuration in subclause 4.7 using the corresponding test model NR-FR2-TM1.1 and set of physical channels in subclause 4.9.

In addition, for an NR BS declared to be capable of multi-carrier and/or CA operation use the applicable test signal configuration and corresponding power setting specified in subclause 4.7.

6)	Measure EIRP sample when transmitter is ON at maximum rated power, call this P_EIRP_ON by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. From maximum transmit power measurement also corresponding TRP, P_TRP_ON is known.

a)	If the test facility only supports single polarization, then measure EIRP with the test facility's test antenna/probe polarization matched to the NR BS.

b)	If the test facility supports dual polarization then measure total EIRP for two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	Measure EIRP sample when transmitter is OFF, call this P_EIRP_OFF by either a) or b) below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3μs to 35/N μs before start of next transmitter ON period - 3μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz. From maximum transmit power measurement also corresponding TRP, P_TRP_ON is known.

8)	For an NR BS supporting contiguous CA, measure EIRP by either a) or be below. EIRP is measured over 70/N μs filtered with a square filter of bandwidth equal to the Aggregated Channel Bandwidth BWChannel_CA centred on (Fedge_high+Fedge_low)/2. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period - 3 μs. N = SCS/15, where SCS is the smallest Sub Carrier Spacing in kHz used in the Aggregated BS Channel Bandwidth, call this P_EIRP_OFF.

9)	 Calculate the difference between TRP when transmitter is on and TRP OFF requirement -36dBm/MHz, call this  ∆TRP=P_TRP_ON+36dBm/MHz, where P_TRP_ON is declared TRP by manufacturers and expressed in the unit of dBm/MHz.

10)	 Calculate the required EIRP off power as  P_EIRP_ON-∆TRP where P_EIRP_ON is declared at the conformance testing direction by manufacturers and expressed in the unit of dBm/MHz.

For multi-band capable NR BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carriers activated in the other band.]

6.5.2.5	Test requirements

6.5.2.5.1	BS type 1-O

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to subclause 6.5.2.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 6.0 GHz.

For multi-band RIB, the requirement is only applicable during the transmitter OFF period in all supported operating bands.

6.5.2.5.2	BS type 2-O

The measured EIRP OFF power  P_EIRP_OFF according to subclause 6.5.2.4.2.3 shall be less than [P_EIRP_ON-∆TRP+XX] dBm/MHz for carrier frequency 24.25 GHz < f ≤ 29 GHz.

The  measured EIRP OFF power  EIRP_OFF according to subclause 6.5.2.4.2.3 shall be less than [P_EIRP_ON-∆TRP+XX] dBm/MHz for carrier frequency  37 GHz < f ≤  40 GHz.

For multi-band RIBs, the requirement is only applicable during the transmitter OFF period in all supported operating bands.]