3GPP TSG-RAN WG4 Meeting #89                                                                         R4-1814527

# Spokane, US, 12th – 16th November, 2018

Agenda Item:	7.12.1

Source:	Intel Corporation

Title:	Discussion about inter-frequency accuracy requirement

Document for:	Discussion

# Introduction

In last meeting, there is a WF[1] about inter-frequency accuracy requirement and asked companies to provide simulation results.

On inter-frequency measurement accuracy and side condition: - Option 1: If the side condition is -6dB, the LTE inter-frequency measurement accuracy is reused for NR FR1 inter-frequency measurement. - Option 2: If the side condition is -4dB, the NR FR1 inter-frequency measurement accuracy need to be updated according to the simulation results. - The simulation assumption is R4-1806551 - Other options are not precluded

# SS RSRP for inter-frequency with SNR = -4dB in FR1

From our simulation, the absolute and relative RSRP delta error is reduced by less than 0.4dB where the gain is not obvious. We suggest to keep the requirement of SNR=-4 dB as the same with that of SNR=-6dB.

Fig.1 RSRP delta CDF for SNR= -3dB/-4dB/-6dB with 5 samples

Observation 1: the SS-RSRP accuracy gain (smaller than 0.4dB) is not obvious when side condition increases from -4dB to -6dB.

# SS RSRQ in FR1

Since e frequency relative  RQ reqRSRP and RSSI can be measured with comparable accuracy as LTE, the RSRQ gain is not obvious either.

Observation 2: the SS-RSRQ accuracy gain is not obvious when side condition increases from -4dB to -6dB.

# SS-SINR in FR1

Our simulation results are as follows:

Fig.2 SINR delta CDF for SNR= -3dB/-4dB/-6dB with 5 samples

From our simulation, the SINR accuracy requirement can improved by 0.4dB when SNR=-4dB compared with SNR = -6dB.

Observation 3: The SS-SINR accuracy gain is 0.4dB when side condition increases from -4dB to -6dB.

# Conclusion

In this contribution, we provide observation for SS based measurement requirements for RSRP/RSRQ/SINR.

Observation 1: the SS-RSRP accuracy gain (smaller than 0.4dB) is not obvious when side condition increases from -4dB to -6dB.

Observation 2: the SS-RSRQ accuracy gain is not obvious when side condition increases from -4dB to -6dB.

Observation 3: the SS-SINR accuracy gain is 0.4dB when side condition increases from -4dB to -6dB.

# References

[1] R4-1814065, “Way forward on inter-frequency measurement accuracy”, CMCC