3GPP TSG-RAN WG4 Meeting #88bis	R4-1814599

# Spokane, US, 12th – 16th, November 2018

Agenda item:	7.6.11.4

Source: 	Intel Corporation

Title: 	Out-of-Band blocking for intra-band contiguous CA requirement for all BW Classes

Document for:	Discussion

# Introduction

The out of band blocking requirement has been defined only for BW Class C in the TS 38.101-1. The definition of this requirement is missing for the rest of the BW classes. In this contribution we provide our view for the requirement for all BW Classes.

# Discussion

## Background

- Definition of Out-of-band Blocking for NR band for BW Class C in TS38.101-1 [1] :

Out-of-band blocking parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 110 MHz | 120 MHz | 130 MHz | 140 MHz | 150 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 160 MHz | 180 MHz | 200 MHz |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 9 | 9 | 9 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below …. |  |  |  |  |  |  |

Out of-band blocking for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -45 | -30 | -15 |
|  | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(200,3CBW) or FDL_high + MAX(200,3CBW) ≤ f ≤ 12750 |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | 1 ≤ f ≤ FDL_low – MAX(150,3CBW) or FDL_high + MAX(150,3CBW) ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

## Definition of Out-of-Band blocking for intra-band contiguous CA for all BW classes

In NR spec [1] only BW Class C is considered for intra-band contiguous CA requirements. In the contribution [2] we have given our view regarding the jammer BW for all BW Classes. In summary, we proposed to consider the minimum CC BW from the edges for the jammer BW.

| BW Class | AggBW | Max BW (MHz) | Jammer BW (MHz) | ∆OBB sig | OBB_Range |
| --- | --- | --- | --- | --- | --- |
| B | 20M-50M | 50 | 5 | 9 | TBD |
| F | 60M-100M | 100 | 20 | 9 | TBD |
| C | 110M-200M | 200 | 50 | 9 | TBD |
| D | 210M-300M | 300 | 50 | 9 | TBD |
| E | 310M-400M | 400 | 50 | 9 | TBD |
| G | 110M-150M | 150 | 40 | 9 | TBD |
| H | 160M-200M | 200 | 40 | 9 | TBD |
| I | 210M-250M | 250 | 40 | 9 | TBD |
| J | 260M-300M | 300 | 40 | 9 | TBD |
| K | 310M-350M | 350 | 40 | 9 | TBD |
| L | 360M-400M | 400 | 40 | 9 | TBD |

Observation 1: The jammer BW is defined as the minimum CC bandwidth of the CCs located on the edges from the corresponding BW Class

In Table 1 we provide the OBB parameters, for the power in transmission BW we consider the 9 dB for all BW Classes as defined in the spec for the NR bands single carrier for BW above 20 MHz. In Table 2 we provide the out of band blocking for all BW Classes.

Table 1: Out-of-band blocking parameters for intra-band contiguous CA for all BW Classes

| RX parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F | G |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |  |
|  | dB | 9 | 9 | 9 | 9 | 9 | 9 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |  |
|  |  | H | I | J | K | L |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |  |
|  | dB | 9 | 9 | 9 |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below …. |  |  |  |  |  |  |  |

Table 2: Out of-band blocking for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Range1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n77, n78 (NOTE 3) | Pinterferer | dBm | -45 | -30 | -15 |
|  | Finterferer (CW) | MHz | N/A | N/A | TBD |
| n79 (NOTE 4) | Finterferer (CW) | MHz | N/A | N/A | TBD |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: CBW denotes the channel bandwidth of the wanted signal NOTE 3: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 2700 MHz and FInterferer < 4800 MHz. For CBW > 15 MHz, the requirement for Range 1 is not applicable and Range 2 applies from the frequency offset of 3CBW from the band edge. For CBW larger than 60 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm, for FInterferer > 3650 MHz and FInterferer < 5750 MHz. For CBW ≥ 40 MHz, the requirement for Range 2 is not applicable and Range 3 applies from the frequency offset of 3CBW from the band edge. |  |  |  |  |  |

Proposal 1:	The introduction of the out band blocking requirement for intra-band contiguous CA for all BW classes as provided in Table 1 and 2.

# Conclusion

In this contribution we have provided our view on the out-of-band blocking for intra-band contiguous CA for all BW classes. In summary, we have made the following proposal:

Proposal 1:	The introduction of the out band blocking requirement for intra-band contiguous CA for all BW classes as provided in Table 1 and 2.

# References

- 3GPP TS 38.101-1 V15.3.0 (2018-09)

- R4-1814598, “ACS and IBB intra-band contiguous requirements for all BW Classes”, Intel, RAN4#89