3GPP TSG-RAN WG4#89 Meeting	R4-1814670

Spokane, U.S.A., 12 – 16 November 2018

Agenda item:	6.12.2.2

Title: 	PDSCH simulation results and discussion for 8Rx UEs

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN Plenary Meeting #78, the Rel.15 WI “UE requirements for LTE DL 8Rx antenna ports” has been approved and its WID is included in [1]. In the scope of the WID [1], there is part related to RAN4 UE demodulation/CSI performance tests as shown below:

- For UE demodulation/CSI requirements

  - Define channel model for downlink 8Rx antennas

    - Specify the antenna configuration and MIMO channel correlation matrices for 8Rx antennas;

    - Specify the static channel model;

  - Define test cases for the rank lower than or equal to 4.

    - Adopt the test cases studied in SI to define requirements.

  - Define test cases for the rank higher than 4 in fading channel.

    - Base on combinations of rank and MCS that can achieve the maximum configured throughput.

  - Define SDR test for 8Rx in WI.

  - Define CSI test for 8Rx.

  - No PDCCH/PCFICH demodulation requirement is expected for 8Rx.

- Considering the test coverage of 8Rx, test applicability rule is needed to define

  - Define applicability rule of existing performance requirements for 8Rx capable UEs.

In RAN4#86bis Meeting, a WF [2] was agreed on the PDSCH demodulation tests for 8Rx UE as follows:

  - For test cases of ranks lower than or equal to 4, introduce:

TM2: Rank 1, 16QAM 1/2, EVA5, 2x8 medium correlation B

TM3: Rank 2, 16QAM 1/2, EVA70, 2x8 low

  - FFS: Whether to introduce rank 5~8 demodulation performance tests

In RAN4#88 Meeting, a WF [3] was agreed on the PDSCH demodulation tests for 8Rx UE as follows:

  - Introduce 8Rx demodulation test(s)

TM9

Rank higher than four, including at least one rank from {6, 8}

16QAM

EPA5

In this contribution, we first provide our updated simulation results for PDSCH demodulation performance and then share our views on the 8Rx capable UE demodulation requirement.

# Discussion

## Simulation results and requirement proposals for PDSCH test cases with ranks lower than or equal to 4

Table 1 lists the simulation parameters for TM2 and TM3 test cases.

Table 1 Simulation parameters for TM2 and TM3 test cases

| Test number | Transmission mode | Bandwidth and MCS | Reference channel | Propagation condition | Correlation matrix and antenna configuration |
| --- | --- | --- | --- | --- | --- |
| 1 | TM2 | 10 MHz 16QAM,1/2 | R.11 FDD | EVA5 | 2x8 Low Medium correlation B, ULA |
| 2 | TM3 | 10 MHz 16QAM,1/2 | R.11 FDD | EVA70 | 2x8 Low |

Figure 1 and 2 illustrate the simulation results for the above two test cases without considering any impairment margins.

Figure 1 Simulation results for TM2

Observation 1: For TM2 test case, the maximum configured throughput is reached at 1dB, and 70% of maximum configured throughput is reached at -2.4dB.

Proposal 1: For TM2 test case, the PDSCH demodulation requirement is set to -2.4dB without considering impairment margins.

Figure 2 Simulation results for TM3

Observation 2: For TM3 test case, the maximum configured throughput is reached at 4dB, and 70% of maximum configured throughput is reached at 1.6dB.

Proposal 2: For TM3 test case, the PDSCH demodulation requirement is set to 1.6dB without considering impairment margins.

## Simulation results and views on PDSCH demodulation performance requirements for ranks higher than 4

Table 2 lists the simulation parameters for TM9 test cases with 16QAM. More simulation assumption details can be found in [4].

Table 2 Simulation parameters for TM9 test cases

| TM9 | 10 MHz 16QAM, 1/2 | R.xx FDD | EPA5 | 8x8 Low |
| --- | --- | --- | --- | --- |

Figure 3 (Rank6) and Figure 4 (Rank8) illustrate the simulation results for the above TM9 test cases with 8×8 antenna configuration and 16QAM modulation, and without considering any impairment margins.

Figure 3 PDSCH TM9 simulation results with 8×8 antenna configuration, 16QAM and Rank6

Figure 4 PDSCH TM9 simulation results with 8×8 antenna configuration, 16QAM and Rank8

Observation 3: Both Ran6 and Rank8 with 16QAM & 1/2 code rate can fly, in terms of achieving the maximum configured throughput within practical SNR range (for 70% maximum configured throughput, at 12.2dB and 16.8dB, respectively).

However, the throughput achieved by 16QAM & 1/2 code rate combined with Rank8 can be easily replaced by using higher modulation order or larger code rate combined with Rank6 or even lower Ranks. Therefore, in our view, such low modulation order and code rate are not valid use case for Rank8 downlink transmission.

Proposal 3: Define TM9 with 16QAM and 6-layer test case for the 8×8 8Rx UE’s PDSCH demodulation requirement.

# Conclusion

In this contribution, we first provide our updated simulation results for PDSCH demodulation performance and then share our views on the 8Rx capable UE demodulation requirement.

Observation 1: For TM2 test case, the maximum configured throughput is reached at 1dB, and 70% of maximum configured throughput is reached at -2.4dB.

Observation 2: For TM3 test case, the maximum configured throughput is reached at 4dB, and 70% of maximum configured throughput is reached at 1.6dB.

Observation 3: Both Ran6 and Rank8 with 16QAM & 1/2 code rate can fly, in terms of achieving the maximum configured throughput within practical SNR range (for 70% maximum configured throughput, at 12.2dB and 16.8dB, respectively).

Proposal 1: For TM2 test case, the PDSCH demodulation requirement is set to -2.4dB without considering impairment margins.

Proposal 2: For TM3 test case, the PDSCH demodulation requirement is set to 1.6dB without considering impairment margins.

The throughput achieved by 16QAM & 1/2 code rate combined with Rank8 can be easily replaced by using higher modulation order or larger code rate combined with Rank6 or even lower Ranks. Therefore, in our view, such low modulation order and code rate are not valid use case for Rank8 downlink transmission.

Proposal 3: Define TM9 with 16QAM and 6-layer test case for the 8×8 8Rx UE’s PDSCH demodulation requirement.

# Reference

[1] R4-172842, “New WI proposal: UE requirements for LTE DL 8Rx antenna ports”, Huawei, HiSilicon, Dec. 2017

[2] R4-1805552, “Way forward on 8Rx demodulation requirements”, Huawei, April 2018

[3] R4-1811868, “Way forward on 8Rx demodulation and CSI tests”, Huawei, August 2018

[4] R4-1811722, “Simulation assumptions for 8Rx tests”, Huawei, August 2018